3GPP TSG-RAN WG4 Meeting #114bis	R4-2504499

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On LBCA RRM requirement impact

Agenda item:	7.7.3

Document for:	Discussion

Introduction

The discussion about RRM requirement impact due to introduction of LBCA feature started in the last RAN4 meeting. In this contribution we continue the discussion.

Discussion

In the last meeting, RAN4 sent an LS to RAN1 about the required availability of SSB on PCell and SCell. In this RAN1 meeting, RAN1 will start the pattern design, which directly defines whether and what type of impact there is on the RAN4 requirements.

Switch delay

In the last RAN4 meeting, it was agreed that RRM will not introduce a further switching delay on top of the switching time agreed by RF.

- Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM.

In RAN4#114 meeting, RF has agreed the following about the switching period:

Issue 2: Switching period values -- - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings

However, in the last meeting’s WF, the following issue was left open:

Issue 1-1-1: RRM work scope - Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

Based on the former agreement, we do not see a need to discuss this issue further. RRM shall simply follow the switching time agreed in RF.

No further discussion needed about interruption on SDL or PCell due to switching time on PCell or SDL SCell.

Interruption due to a third carrier

In RAN4 #114, RF agreed that there may be a third carrier combined with 2 low band carriers, which not to be used with a switching pattern.

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

Considering the RF agreement, it is not precluded that additional carriers can be combined with the LBCA-sw. In this scenario, where we have 3 carriers, 2 low band carriers which operate with carrier switching, and another carrier that does not need carrier switching. If this is allowed by RF and UE capabilities, we need do discuss from RRM perspective whether the 3rd carrier may experience interruptions or not.

RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

RAN4 to discuss the need for interruption requirements in the following scenario

  - UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Regarding the scenario above, allowing for interruptions could be very detrimental for performance of the 3rd band. In L3 measurements, RAN4 has specified interruption requirements for measurements without gaps, and those interruptions can be controlled by the network by configuring the SMTC periodicity, e.g. 40, 80, 160 ms. In the case of LBCA-sw the situation is different. If the UE architecture is such that it causes interruptions on the 3rd carrier every time it switches between FDD and SDL carrier, the interruption overhead would be very high for the 3rd carrier. We still don’t have the RAN1 design of the carrier switching pattern, but it can be assumed that it needs much more frequent switching than what is used for L3 measurements. In one example, if the LBCA-sw pattern has a periodicity of 10 ms, and the UE causes 2 interruptions during this period, the number of interruptions on the 3rd carrier would be very high. If that is needed, RAN4 needs to discuss what are the interruption requirements, and how the UE can signal to the network which band combinations generate interruptions on the non-switched carriers.

Since this WID has only few meetings to conclude the work, we believe that the best approach is to consider only the cases without interruptions on the 3rd carrier as part of this WID.

If a third carrier is configured without switching pattern, no interruption is allowed on the third carrier due to LBCA-sw operation on PCell and SDL SCell.

Measurement requirements for activated and deactivated SCell

In the last meeting the discussion about measurement requirements for activated and deactivated SDL SCell depending on whether the SCell is SSB-less or not led to the following agreements and FFS points:

- SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used.

It was agreed that measurement requirements for SSB-less SCell are not to be introduced. But it remained open whether to introduce requirements for SSB-based SCell operation when the switching pattern is used.

The need and details for such requirements heavily depends on the switching pattern design and the SSB availability on the activated or deactivated SCell based on the pattern. The SSB periodicity in the measurement delay requirements may depend on the availability of SSBs on the SCell. This depends on how the switching pattern is designed in comparison to the SSB/SMTC periodicity. Additionally, the amount of SSB available on each cell might be depending on radio conditions, so discussion on the need for the UE to report need for SSB availability is needed.

If the switching pattern designed by RAN1 impacts the SSB/SMTC availability on the SCell, define active SCell measurement delay requirements based on the periodicity of the available SSB/SMTC. Details to be agreed once the switching pattern design gets clearer in RAN1.

As part of the agreement for de-activated SDL SCell measurement, it is open how to define the requirements for deactivated SCell measurements. In previous releases, deactivated SCell measurements have been considered using gap-less measurements following measCycleSCell. The measurements without gaps are only possible in this situation because the UE has a spare RF chain that can be used for measurement of the deactivated SCell. However, for the case of LBCA-sw it is not clear how the UE can perform the measurements. In this scenario, whenever the UE is monitoring the signals from the SDL SCell it cannot receive DL data in the FDD PCell.

If the UE cannot receive DL signals for FDD and SDL carriers simultaneously, then measurements without gaps cannot be used for LBCA-sw.

Since the UE cannot monitor the SDL while receiving data in the FDD carrier, we need some kind of gap-assisted measurements to monitor the deactivated SDL SCell. On the other hand, once the SDL carrier is activated, maintaining the measurement gap for the SDL carrier would not be efficient. One efficient way of performing measurements with gaps for the deactivated SCells would be to update the PreMG conditions to activate and deactivate the PreMG based on the activation of the SDL carrier operating with LBCA-sw.

RAN4 to discuss how to perform efficient deactivated SCell measurements using gaps.

Other RRM requirements

SCell activation delay

- other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA

We agree that SCell activation delay requirements for activation of the SDL SCell are needed. SCell activation delay may be impacted if the availability of reference signals needed for activation of the SDL SCell changes through the switching pattern design. However, it is difficult to define the requirements without knowing the details of the switching pattern design, which is still fully open in RAN1. RAN4 may try to identify the potential impact to the existing requirements, but the detailed requirement may need to follow further RAN1 agreements.

Starting point for SDL SCell activation is to reuse the existing SCell activation delay requirements. Availability of SSB based on the RAN1 switching pattern to be taken into account in the delay requirements once the pattern design becomes clearer.

Furthermore, RAN4 needs to clarify which types of SCell activation are supported when the UE supports LBCA.

Clearly PUCCH SCell activation/deactivation delay requirements are not needed as the SDL SCell does not have uplink. It is also not necessary to define delay requirements for activation of multiple SCells. Furthermore, Rel-19 introduced fast SCell activation through EMR, which we do not see should be impacted as in general interaction between parallel WIs of the same release should be avoided.

PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI.

Considering the requirements that may be impacted, RAN4 should agree if all of these are in the scope of the WI or not:

- MAC-CE based SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at handover

- Fast SCell activation

- SCell activation with L3 reporting during activation

Furthermore, RAN4 needs to clarify whether both known and unknown SCell activation are in the scope.

RAN4 to clarify which of the existing SCell activation types and delay requirements are in the scope of the LBCA WI:

  - MAC-CE based SCell activation

  - Direct SCell activation at SCell addition

  - Direct SCell activation at handover

  - Fast SCell activation

  - SCell activation with L3 reporting during activation

RAN4 to clarify whether both known and unknown SCell activation is in the scope of the WI.

Regarding SCell deactivation delay, this does not require any measurements to be performed on the SCell to be deactivated, and therefore we do not see an impact on the deactivation delay requirements.

Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

Other delays (PCell and SCell related)

- FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

- Potential impacts to the PCell related requirement - FFS

Similar to SCell activation delay, the delay requirements listed in the WF should be studied when switching pattern is activated. RAN4 should rely on the RAN1 pattern design in defining whether there is any need for updates. Furthermore, RAN4 needs to clarify which requirements would be impacted.

For other delay requirements, make necessary edits based on RS availability through the switching pattern once the pattern design is clear. Furthermore, RAN4 to clarify which delay requirements are in the scope of the WI.

Assumptions for LBCA switching

Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting.

Even if LBCA requirements are defined for both SSB-based and SSB-less SCell, we think RAN4 can start by defining the requirements for SSB-based SCell.

Start from defining RRM requirements for SSB-based SCell operation.

Conclusion

The following Observations and Proposals were made:

Proposal 1: No further discussion needed about interruption on SDL or PCell due to switching time on PCell or SDL SCell.

Observation 1: RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

Observation 2: RAN4 to discuss the need for interruption requirements in the following scenario

  - UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Proposal 2: If a third carrier is configured without switching pattern, no interruption is allowed on the third carrier due to LBCA-sw operation on PCell and SDL SCell.

Proposal 3: If the switching pattern designed by RAN1 impacts the SSB/SMTC availability on the SCell, define active SCell measurement delay requirements based on the periodicity of the available SSB/SMTC. Details to be agreed once the switching pattern design gets clearer in RAN1.

Observation 3: If the UE cannot receive DL signals for FDD and SDL carriers simultaneously, then measurements without gaps cannot be used for LBCA-sw.

Proposal 4: RAN4 to discuss how to perform efficient deactivated SCell measurements using gaps.

Proposal 5: Starting point for SDL SCell activation is to reuse the existing SCell activation delay requirements. Availability of SSB based on the RAN1 switching pattern to be taken into account in the delay requirements once the pattern design becomes clearer.

Proposal 6: PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI.

Proposal 7: RAN4 to clarify which of the existing SCell activation types and delay requirements are in the scope of the LBCA WI:

a. MAC-CE based SCell activation

b. Direct SCell activation at SCell addition

c. Direct SCell activation at handover

d. Fast SCell activation

e. SCell activation with L3 reporting during activation

Proposal 8: RAN4 to clarify whether both known and unknown SCell activation is in the scope of the WI.

Proposal 9: Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation.

Proposal 10: For other delay requirements, make necessary edits based on RS availability through the switching pattern once the pattern design is clear. Furthermore, RAN4 to clarify which delay requirements are in the scope of the WI.

Proposal 11: Start from defining RRM requirements for SSB-based SCell operation.

- References

- R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple