3GPP TSG-RAN WG4 Meeting #114bis	R4-2504504

Wuhan, China, 07 – 11 April 2025

Agenda item:	7.28.1

Source:	Nokia

Title:	Collection of XR_Ph3 agreements for information

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Information

# 1	Introduction

This document contains the following rapporteur’s input to this meeting:

-	A list of XR agreements reached in RAN4 so far;

-	A short overview of the status from other groups, focusing on matters impacting RAN4.

# 2	List of Agreements

## 2.1	Scenarios for measurement skipping

Agreement from RAN4 #112 R4-2414043

Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

Agreement from RAN4 #112bis R4-2416864

Issue 1-1: Deployment scenarios < Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation

Agreement from RAN4 #113 R4-2420103

Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID

Agreement from RAN4 #114 R4-2502700

Issue 1-2: Types of measurements to be considered <Agreement>: - The following measurement types are considered for XR measurement skipping - Intra-frequency - Inter-frequency - LTE-inter- RAT (NR-EUTRAN FDD/TDD) - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

Agreement from RAN4 #113 R4-2420103

Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID

Agreement from RAN4 #114 R4-2502700

Issue 1-3: L3 Measurements without gaps with scheduling restrictions Issue 1-4: L1 measurements with scheduling restrictions and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

## 2.2	Measurements with gaps

Agreement from RAN4 #113 R4-2420103

## Issue 2-1: General approach for defining requirements for measurements with gaps <Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

Agreement from RAN4 #114 R4-2502700

## Issue 2-1: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. - Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table Gap types; First priority; Second priority; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

.

## 2.3	XR skipping requirements impact

Agreement from RAN4 #113 R4-2420103

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay

Agreement from RAN4 #114 R4-2502700

## Issue 3-1: Principles for measurement delay calculation <Agreement>: Follow NR-U-like approach for designing the formulas for measurement delay extension.

## 2.3	Time offset for measurement skipping processing

Agreement from RAN4 #113 R4-2420103

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

## 2.4	Need/feasibility of UE assistance information

Agreement from RAN4 #114 R4-2502700

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

## 2.5	List of impacted clauses and work split

Agreement from RAN4 #114 R4-2502700

Clause; Topic; Volunteer. 8.xr (new) Or 9.1; Time offset for DCI based measurement skipping; Nokia. 9.2.6; Intra-freq measurements with gaps; ZTE. 9.3.4; Inter-freq measurements with gaps; Ericsson. 9.4.2; Inter-RAT measurements E-UTRAN FDD measurements; Huawei. 9.4.3; Inter-RAT measurements E-UTRAN TDD measurements; Apple. NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580.

# 3	Workplan

Table 3-1: Workplan for the XR WI

| Date | RAN1 | RAN2 | RAN3 | RAN4 | RAN |
| --- | --- | --- | --- | --- | --- |
| 2024-02 | RAN1#116 Start discussion on a design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. |  |  |  |  |
| 2024-03 |  |  |  |  | RAN#103 Status Report (target of 2% corresponding to 0.5 TU spent over an initial allocation of 24.8) |
| 2024-04 | RAN1#116bis Continue the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Start agreeing on solution(s). | RAN2#125bis Endorse workplan Start the work on all 4 objectives except measurement gaps, for which RAN1 conclusions first need to be reached. For multi-modality and scheduling enhancements, start the study phase. | RAN3#123bis Start discussion on RAN3 related objectives on NR-DC, e.g. PDU set based handling, ECN marking, etc. Agree the DC bearer types to be supported. Designate responsible companies for BL Stage-2 & 3 CRs. |  |  |
| 2024-05 | RAN1#117 Continue developing the solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#126 Continue the work on all 3 objectives. | RAN3#124 Finalize the solution on required enhancements. Finalize Stage-2 CR(s) Agree the skeleton of Stage-3 CR(s). |  |  |
| 2024-06 |  |  |  |  | RAN#104 Status Report (target of 18% corresponding to 4.5 TUs spent over an initial allocation of 24.8) Take into account outcome of SA2 study if available. |
| 2024-08 | RAN1#118 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#127 Continue the work on all 3 objectives. For multi-modality and scheduling enhancements, conclude the study phase and convey the outcome to RAN#105. | RAN3#125 Agree Stage-2 CR(s) and Stage-3 CR(s) | RAN4#112 Approval of workplan Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |  |
| 2024-09 |  |  |  |  | RAN#105 Status Report (target of 30% corresponding to 7.5 TUs spent over an initial allocation of 24.8) Checkpoints: 1. Multi-modality (RAN2) 2. Scheduling enhancements (RAN2) |
| 2024-10 | RAN1#118bis Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Check if RAN1 involvement related to objective on multi-modality is needed. | RAN2#127bis Check with RAN1 the progress on measurement gaps and start discussing in RAN2. Continue the work on UL scheduling enhancements according to the refined objective. Start the work on uplink congestion signalling. | RAN3#125bis Discuss multi-modality when handling incoming LS (no additional TU allocated to the new objective). | RAN4#112bis Conclusions on scenarios for measurement skipping based on RAN1 conclusions. Discussion on the need for UE assistance information regarding measurements occasions needed. |  |
| 2024-11 | RAN1#119 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#128 Narrow down possible solutions. First version of Stage 2 CR | RAN3#126 Discuss LS on PDU Set Information Marking Support without QoS parameters, and usage of multi-modality information (no additional TU allocated to the new objective). | RAN4#113 Discussion on RRM requirements with measurement skipping. Discussion on the necessary UE assistance information. |  |
| 2024-12 |  |  |  |  | RAN#106 Status Report (target of 55% corresponding to 13.5 TUs spent over an initial allocation of 24.8) Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) |
| 2025-02 | RAN1#120 Finalize, if not already, the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Provide RAN1 input to RAN2 on RRC configuration parameters, UE capabilities, and Stage 2 TPs. | RAN2#129 Continue the work on the details of the agreed solutions. First versions of Stage 2 & 3 CR. | RAN3#127 Discuss LS on UL bit rate control, available data rate reporting and multi-modality awareness (no additional TU allocated to the new objective). | RAN4#114 Conclusion on RRM requirements with measurement skipping. Discussion on the need for UE assistance information regarding measurement skipping. Agreement of CR work split. |  |
| 2025-03 |  |  |  |  | RAN#107 Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) Status Report (target of 67% corresponding to 16.5 TUs spent over an initial allocation of 24.8) |
| 2025-04 | RAN1#120bis XR will not be discussed. | RAN2#129bis Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#127bis Start discussion on RAN3 related objectives on uplink rate control, PDU set based QoS handling enhancement, TTNB and burst size, exposure of available data rate etc. Designate responsible companies for Stage-2 & 3 BL CRs. | RAN4#114bis Conclusion on the necessary UE assistance information. First Draft CRs for RRM core requirements (draft CRs for 38.133 expected) First big Draft CR. |  |
| 2025-05 | RAN1#121 | RAN2#130 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#128 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in Stage 2 & 3 BL CRs. | RAN4#115 Discussion on remaining open issues for RRM core. |  |
| 2025-06 |  |  |  |  | RAN#108 Status Report (target of 91% corresponding to 22.5 TUs spent over an initial allocation of 24.8) |
| 2025-08 |  | RAN2#131 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN3#129 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN4#116 Discussion on remaining open issues for RRM core. Revision of CRs for RRM core requirements. Planning of RRM performance. Initial discussion on RRM test cases. |  |
| 2025-09 |  |  |  |  | RAN#109 Status Report (target of 100% corresponding to 24.8 TUs spent over an initial allocation of 24.8) CRs received for approval. |

# 4	Status from other groups

## 4.1	RAN1

During RAN1#120, the following was agreed:

Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Agreement from RAN1#119 is updated (red part)

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

- Alt1: If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band (and have same SCS) can be co-scheduled. In other words, if co-scheduled cells could be inter-band, skipping indication field cannot be configured.

Alt2: When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and

- if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field.

- if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell.

Following conclusion was made in RAN1#120:

Conclusion

It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

- Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

## 4.2	RAN2

Agreements affecting RAN4 in RAN2 #128

Agreements for RRM measurement gap skipping:

-	From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion.

-	“Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC.RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

-	RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1.