3GPP TSG-RAN WG4 Meeting #114bis	R4-2504509

# Wuhan, China, April, 7 - 11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR38.133 Introduction of measurement gap cancellation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Measurement gap cancellation is part of the Rel-19 work item NR XR Phase 3. Delay requirements for cancellation of a particular measurement gap occasion are missing in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for measurement gap occasion cancellation used for XR. Clause 8.x specifies requirements for the measurement gap occasion cancellation delay. Clause 9.1.y specifies applicability of the measurement gap occasion cancellation enhancement (applicability to measurement gap types is still under discussion). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing delay requirements and applicability for measurement gap occasion cancellation. The enhancement cannot be used for XR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new), 9.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR …CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 8.25.3	SSB based TCI state activation delay

In FR1, when the UE receives LTM candidate cell TCI state activation command at slot n, the UE shall have completed the LTM TCI state list update in

slot n + THARQ +3Nslotsubframe,µ + TOk*( Tfirst-SSB + TSSB-proc) / NR slot length.

In FR2, when all the target TCI states in the active TCI state list are known, when the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in

slot n + THARQ +3Nslotsubframe,µ + TOk*( Tfirst-SSB + TSSB-proc) / NR slot length.

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	TOk = 1 if the target TCI state is not in the serving cell active TCI state list or LTM candidate cell active TCI state list, 0 otherwise.

-	Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when the target SSB is to be measured outside MG, upon receiving PDSCH carrying MAC-CE activation command in slot n;

-	Tfirst-SSB is the time to first SSB occasion overlapping with MGL after slot n + THARQ +3Nslotsubframe,µ + 2 ms if the SSB is to be measured within MG, upon receiving PDSCH carrying MAC-CE activation command in slot n;

-	TSSB-proc = 2 ms.

## 8.x	Measurement gap occasion cancellation delay

### 8.x.1	Introduction

The requirements in this clause apply for the conditions defined in clause 9.1.y.2.

The UE shall complete the cancellation of the measurement gap occasion due to DCI indicating the cancellation of the measurement gap occasion within the delay defined in this clause. UE supporting [TBD capability option 2] shall in addition complete the activation of the measurement gap occasion due to DCI indicating the reactivation of the measurement gap occasion within the delay in this subclause.

### 8.x.1	Time offset for measurement gap occasion cancellation

The requirements in this clause apply for a UE supporting DCI-based measurement gap occasion cancellation.

The UE shall be able to cancel a measurement gap occasion and receive data without scheduling restrictions, if the DCI command, containing the measurement gap cancellation indication, is received no later than X prior to the start of the measurement gap occasion including MGTA. The value X is 3ms for a UE supporting [TBD capabilityName], and 5ms otherwise.

For UE supporting [TBD capability option 2], the UE shall additionally be able to activate a measurement gap occasion in order to perform measurements within the measurement gap, if the DCI command, containing the indication for reactivation of the measurement gap occasion, is received no later than Y prior to the start of the measurement gap occasion including MGTA. The value Y is [3] ms for a UE supporting [TBD capabilityName], and [5] ms otherwise.

--- End of change 1 ---

--- Start of change 2 ---

#### 9.1.13.3	Collision involving NCSGs

Collisions between occasions of concurrent measurement gap and NCSG or of two NCSGs may occur as specified in this clause if the two occasions are

-	two per-UE NCSGs, or

-	two per-FR NCSGs in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-FR NCSG and one per-UE measurement gap, or

-	one per-UE NCSG and one per-FR measurement gap, or

-	one per-FR NCSG and one per-FR measurement gap in the same FR.

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collision rule applies.

When the first occasion is NCSG, the ending point of the first occasion is the end of VIL2 and/or when the second occasion is NCSG, the starting point of the second occasion is the start of VIL1.The requirements with concurrentMeasGapsNCSG-r18 apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

### 9.1.y	Measurement gap occasion cancellation

#### 9.1.y.1	Introduction

[FFS]

#### 9.1.y.2	Applicability

Indication for cancellation of a measurement gap occasion as defined in clause 9.1.2 or scheduling restriction as defined in clause 10.6 of TS 38.213 is applicable for a UE in NR SA and carrier aggregation for the following types of measurements:

-	Intra-frequency within measurement gap

-	Inter-frequency within measurement gap

-	LTE-inter-RAT (NR-EUTRAN FDD/TDD) within measurement gap

The requirements for measurement gap occasion cancellation do not apply if the UE is required to do measurements for SRVCC.

NE-DC and EN-DC are not supported for measurement gap occasion cancellation.

Editor’s Note: the type of measurement gap applicable for measurement gap occasion cancellation, aside the legacy MG (Type-1 MG), is under discussion.

#### 9.1.y.3	Requirements

The UE is required to cancel a measurement gap occasion if the DCI command, indicating the cancellation of the measurement gap occasion, is received within a time offset described in clause 8.x.1 prior to the start of the measurement gap occasion including MGTA. If a cancelled measurement gap occasion overlaps with a SSB or CSI-RS used for L3 measurements without gaps or for L1 measurements, the UE shall not cause scheduling restrictions.

The UE supporting [TBD capability option 2] is required to activate the measurement gap occasion, if the DCI command, indicating the reactivation of the measurement gap occasion, is received within a time offset described in clause 8.x.1 prior to the start of the measurement gap occasion including MGTA.

--- End of change 2 ---