3GPP TSG-RAN WG4 Meeting #114bis            	R4-2504535

Wuhan, China, 7th-11th, April, 2024

Agenda item:	7.18.3

Source:	Samsung

Title:	Simulation results on BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In this contribution, the simulation results for BS demodulation with MMSE-IRC are provided for alignment based on agreed simulation assumption in [1].

# 2	Simulation Results

In this section, the initial simulation results are provided to check the performance gain of MMSE-IRC over MMSE receiver. the simulation results for the following test cases are provided

- Number of Rx: 2/4/8Rx

- Number of Interferes: 1 or 2 for 2Rx, and 2 for 4Rx/8Rx

- PUSCH mapping type: type A

- Interferes scenario: HomNet and HetNet

- Channel of interferes: TDLC300-100

- Channel of target cell:

  - TDLC300-100 for both HomNet and HetNet

  - TDLA30-10 or TDLC300-100 for HetNet, and TDLC300-100 or TDLB100-400 for HomNet

- Waveform:  CP-OFDM

- MCS: 16

- SCS and BW:

  - 15KHz:  5/20MHz

  - 30KHz: 10/100MHz

Table 1: PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS 16 | 14.65 | 11.89 | 2.76 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.2 | 13.72 | 2.48 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.61 | 13.72 | 2.89 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 18.17 | 15.59 | 2.58 |

Table 2.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 18.4 | 13.51 | 4.89 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 17.56 | 12.38 | 5.18 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 20.98 | 16.02 | 4.96 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 20.09 | 14.81 | 5.28 |

Table 3.  PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 15.05 | 12.18 | 2.87 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.86 | 14.26 | 2.6 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 17.01 | 14.05 | 2.96 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 18.86 | 16.17 | 2.69 |

Table 4.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 18.83 | 13.86 | 4.97 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 17.5 | 12.55 | 4.95 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 21.39 | 16.42 | 4.97 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 20.09 | 14.98 | 5.11 |

Table 5.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 14.75 | 12.05 | 2.7 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 15.09 | 12.37 | 2.72 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.69 | 13.9 | 2.79 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 17.01 | 14.16 | 2.85 |

Table 6.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 18.52 | 13.75 | 4.77 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.58 | 11.49 | 5.09 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 21.09 | 16.22 | 4.87 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 19.16 | 13.93 | 5.23 |

Table 7.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | MCS16 | 15.05 | 12.26 | 2.79 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 15.17 | 12.2 | 2.97 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 17.01 | 14.14 | 2.87 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 17.12 | 14.06 | 3.06 |

Table 8.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 18.82 | 13.96 | 4.86 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 16.8 | 11.82 | 4.98 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 21.39 | 16.48 | 4.91 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 19.33 | 14.24 | 5.09 |

Table 9.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.33 | 8.31 | 5.02 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 14.18 | 9.32 | 4.86 |

Table 10.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 17.81 | 9.46 | 8.35 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.45 | 7.92 | 8.53 |

Table 11.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.68 | 8.68 | 5 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 14.76 | 9.88 | 4.88 |

Table 12.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 18.27 | 9.87 | 8.4 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.49 | 8.27 | 8.22 |

Table 13.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.37 | 8.54 | 4.83 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.54 | 8.57 | 4.97 |

Table 14.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 21.08 | 9.94 | 11.14 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.11 | 7.82 | 8.29 |

Table 15.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.8 | 8.95 | 4.85 |
| 2 | 1 | 4 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.87 | 8.74 | 5.13 |

Table 16.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 21.39 | 10.36 | 11.03 |
| 2 | 1 | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.29 | 8.15 | 8.14 |

Table 17.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.69 | 4.34 | 6.35 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.38 | 5.17 | 6.21 |

Table 18.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 15.37 | 5.08 | 10.29 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.31 | 3.59 | 9.72 |

Table 19.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.4 | 4.55 | 6.85 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 12.11 | 5.48 | 6.63 |

Table 20.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.15 | 5.44 | 10.71 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.6 | 4.02 | 9.58 |

Table 21.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.82 | 4.63 | 6.19 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 10.86 | 4.57 | 6.29 |

Table 22.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 15.49 | 5.45 | 10.04 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.21 | 3.76 | 9.45 |

Table 23.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.6 | 4.87 | 6.73 |
| 2 | 1 | 8 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 11.77 | 4.5 | 7.27 |

Table 24.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 16.37 | 5.9 | 10.47 |
| 2 | 1 | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 13.39 | 4.09 | 9.3 |

# 3	Conclusion

In this contribution, the initial simulation results are provided.

# References

R4-2502272, WF on [114][323] NR_demod_Ph5_Part1_General_BS