3GPP TSG-RAN WG4 Meeting #114bis	R4-2504544

# Wuhan, China, 7 – 11 April 2025

Agenda item:		6.13.1

Source: 		Qualcomm Inc.

Title:	 	A-MPR and OOB blocking for n252

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, and decision on UE-to-UE coexistence limits was reached in previous meeting. Therefore, in this contribution we present our view on required A-MPR. Another band-specific important aspect without conclusion is OOB blocking requirements and whether co-banding is considered similar to n256 requirements.

## 2	Discussion

In RAN4#114 an agreement on UE-to-UE coexistence was reached and captured in WF [1].

The need for A-MPR was studied using the following assumptions:

- PA calibrated to 30 dB ACLR with 100 RB DFT-s-OFDM QPSK waveform with 22 dBm at antenna connector

- -28 dBc IQ-image and DC-leakage

- -60 dBc CIM3

The proposed A-MPR is summarized in Tables 1 to 4.

Table 1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.7 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 3.06 | > 9 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 6 | ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 7 | ≤ 8 |
| CP-OFDM QPSK | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |

Table 3: A-MPR regions for -30 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≤ 2.7 | A1 |
|  |  | ≤ 3.06 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 3.06 | ≥ 4.5 | A1 |
|  |  | ≥ 3.06 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 2.7 | A1 |
|  |  | ≤ 4.68 | 2.7 ≤ LCRB*12*SCS ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

Table 4: A-MPR for -30 dBm/MHz

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 4.5 |

Proposal 1: Consider results in Table 1 to Table 4 in defining A-MPR for band n252.

For out-of-band blocking different options were discussed for out-of-band blocking frequency ranges. n252 frequency range is overlapping with n256, and it is extremely likely that in practical UE designs the same filter is used to support both n252 and n256. However, when n256 was specified it was considered to share the same filter with n1/n65. In some implementations UE may use different Tx and Rx antenna and route the Rx via an n65/n66 Rx path. Therefore, n252 out-of-band blocking should be specified to accommodate co-banding this possible co-banding.

Proposal 2: Specify OOB boundaries for n252 as shown in Table 5

Table 5: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 3	Conclusion

In this contribution UE RF requirements for n252 were discussed. Following observations and proposals were made:

Proposal 1: Consider results in Table 1 to Table 4 in defining A-MPR for band n252.

Proposal 2: Specify OOB boundaries for n252 as shown in Table 5

Table 5: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 4	References

- R4-2502850, WF on NR and IoT NTN bands, Moderator (Aalyria)