3GPP TSG-RAN WG4 Meeting #114bis	R4-2504545

# Wuhan, China, 7 – 11 April 2025

Agenda item:		6.14.1

Source: 		Qualcomm Inc.

Title:	 	UE-to-UE coexistence and OOB blocking for B252

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, with main focus on UE-to-UE coexistence requirements towards bands n2 and n25. The requirement limits were finally concluded in RAN4#114. Another band-specific important aspect without conclusion is OOB blocking requirements and whether co-banding is considered similar to n256 requirements. As these requirements are relevant also for NTN IoT band 252, these requirements are discussed in this contributino.

## 2	Discussion

In RAN4#114 an agreement on UE-to-UE coexistence was reached and captured in WF [1].

Based on our evaluations these requirements can be met without power backoff.

Proposal 1: No A-MPR is needed for cat M1 or NB-IoT UEs to meet the -30 dBm/MHz and -40 dBm/MHz requirements.

At this point of time these requirements could therefore be also captured as general requirements. However, as these requirements are not applicable everywhere and higher power classes that are being currently specified in another work item may require A-MPR for these requirements. Still, in our view these should be specified as general requirements as this point of time. This will guarantee that UE meets the requirement independent of whether the NS is signalled. Later on, if need for A-MPR for higher power class arrives, the general UE-coex requirement can be limited to apply only for power class 3.

In addition, it is unnecessary to capture both -30 dBm/MHz and -40 dBm/MHz requirements as meeting -40 dBm/MHz guarantees meeting -30 dBm/MHz.

Proposal 2: Capture only the -40 dBm/MHz requirement as a general UE-coex requirement to TS 36.102

For out-of-band blocking different options were discussed for out-of-band blocking frequency ranges. B252 frequency range is overlapping with B256, and it is extremely likely that in practical UE designs the same filter is used to support both B252 and B256. However, when n256 was specified it was considered to share the same filter with B1/B65. In some implementations UE may use different Tx and Rx antenna and route the Rx via an B65/B66 Rx path. Therefore, B252 out-of-band blocking should be specified to accommodate co-banding this possible co-banding.

Proposal 3: Specify OOB boundaries for B252 as shown in Table 1

Table 1: Out of-band blocking for B252 and B256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 and 252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 3	Conclusion

In this contribution UE RF requirements for B252 were discussed. Following observations and proposals were made:

Proposal 1: No A-MPR is needed for cat M1 or NB-IoT UEs to meet the -30 dBm/MHz and -40 dBm/MHz requirements.

Proposal 2: Capture only the -40 dBm/MHz requirement as a general UE-coex requirement to TS 36.102

Proposal 3: Specify OOB boundaries for B252 as shown in Table 1

Table 1: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 and 252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 4	References