3GPP TSG-RAN WG4 Meeting #114bis	R4-2504546

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.10.2.2.1

Source: 		Qualcomm Inc.

Title:	 	NR NTN HPUE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF Tx requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

2.1 SAR solution

In RAN4#114 multiple options for SAR solution were discussed, ranging from UE autonomous dropping of transmissions, enhanced reporting of P-MPR and variety of duty cycle based solutions. However, no clear solutions candidate to be specified was found as captured in the WF [1].

Fundamentally, SAR can be controlled in the UE by either controlling transmission time or transmission power. Only UE has knowledge of the time and power = energy budget available to meet SAR, as network is not aware of e.g. usage of other radios like WiFi or Bluetooth. The remaining energy is also not constant, as both cellular radio and other radio usage is dynamic over time. Ideally, network would need regularly informed on the available energy headroom to be able to optimize scheduling so that user experience is the least impacted.

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

This target can be met by reporting the energy headroom to the network when UE start running close to no available headroom. As SAR measurement window can be very long, unnecessary signaling can be reduced e.g. if there has been a long time with no UL transmissions.

It is expected that in NTN use case most transmissions will be done with high UE output power. Therefore, it is reasonable to use maximum transmit power as a reference for remaining energy. We propose that UE reports to the network that it can transmit X seconds out of next Y seconds with full power. This has the following benefits

- Network has full information on the remaining energy, not only on power or time dimension of it

- Network can scale the time to match the used transmit power, as network also gets PHR

- Network can choose whether to use the budget more quickly or to reduce duty cycle to enable high power transmission over longer time span.

An alternative but equivalent option is to report that UE can transmit at X% duty cycle for next Y seconds.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

A reasonable granularity is required for values X and Y. In our view a starting point would be a range from 100 ms to 5s, with 100 ms granularity. Also to share the same understanding on what are the next Y seconds, the counting of the time can be based on the time instant when the report is sent.

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Finally, RAN2 help is needed to realize the reporting. Therefore, an LS should be sent to RAN2 to specify this energy headroom report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

In addition to maximizing the possibility for high power UE transmissions, also UE RF component feasibility needs to be considered. Specifically, RF filters and their power handling capabilities for PC2 and PC1.5 are designed with 50% duty cycle in mind. There may be instances where UE needs to protect it’s components. To enable re-use of same filter technologies, it would be beneficial not to mandate UE to follow higher than 50% duty cycle for PC2. Unlike the TN solution, this should be optional: UEs with filters with higher power handling capability can transmit longer times.

Proposal 5: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded.

2.2 ACLR requirements and MPR

Based on co-existence study results, PC3 ACLR could be sufficient to fulfil co-existence needs. However, this would diverge NTN and TN requirements from each other. With 30 dBc ACLR requirement the PA would be performing worse as a starting point, and therefore also A-MPR values would be slightly increased compared to selecting PC2 ACLR of 31 dB.

In our evaluations we observed that the same PA model can output approximately 0.25 dB more power if ACLR is set to 30 dB instead of 31 dB. This is not a significant difference and does not result in changes in component design.

Observation 3: ACLR choice between 30 and 31 dBc has approximately 0.25 dB impact on output power capability of a PA model.

We also looked into MPR and meeting emission requirements. If 30 dBc is selected and PA is calibrated to just meet 30 dBc ACLR with no changes to the transceiver and modem assumptions, the edge allocation definition would need to be extended to cover wider allocations. This is because the combination of the linear leakage and PA non-linearity no longer meets the 1 dB MPR. This can be offset by improving the linear leakage, which will result in worse EVM.

Of course, devices which have better ACLR would not need such relaxation nor transceiver parameter updates. It seems simpler to keep the 31 dBc ACLR assumption, which enables re-use of TN MPR with no further changes to the designs. Therefore, it is suggested to adopt 31 dBc ACLR both for PC2 and PC1.5 requirements, with PC1.5 requirements being applied for both handheld and non-handheld designs.

Observation 4: If 30 dBc ACLR is chosen for PC2 and PC1.5 but MPR is not relaxed, the ACLR relaxation may not be possible to be realized in implementations.

Situation is somewhat different for PC1, as the difference between TN ACLR and simulation results is much larger. We observed that output power capability for same PA model to reach 31 or 37 dBc ACLR changed almost 2 dB. That is, the components dimensioning would need to be different to sustain same output power with improved ACLR. In case 4x PC2 PAs would be used for PC1 design, MPR would need to increased to meet requirements.

Observation 4: ACLR choice between 31 and 37 dBc has almost 2 dB impact on output power capability of a PA model.

Also, TN PC1 usage has not become widespread and it is not clear if there are economies of scale to take advantage of. Therefore, it would be useful to relax the PC1 requirements to make implementation possible with 4 PC2 chains and increase the likelihood that designs will appear.

Proposal 6: NR NTN UE ACLR is 31 dBc for PC2 and PC1.5, both for handheld and non-handheld UEs.

Proposal 7: NR NTN UE ACLR is 31 dBc for PC1 non-handheld UEs.

When ACLR is aligned with TN requirements, also MPR can be directly re-used.

Proposal 8: NR NTN PC2 and PC1.5 MPR are the same as specified for TN UE in TS 38.101-1

When output power increases, MPR also typically increases. Also, it can be seen in the existing specifications that typically implementation with multiple Tx chains results in higher MPR. For example, edge allocation MPR for DFT-s-OFDM QPSK is 1 dB for PC3, 3.5 dB for PC2, 6.5 dB for PC1.5 and 8.5 dB for PC1.5 with 4Tx. Impact to other than edge allocations is smaller.

For edge allocations, the MPR is mainly due to SEM, and emissions are a combination of PA non-linearity and linear leakage from baseband. With PC1, it is expected that there would be approximately 2 dB increase for MPR to to meet SEM with 2 dB higher nominal power class compared to PC1.5.

As 4 Tx requirements have been already specified for PC1.5, it would be a logical starting point to use these PC1.5 requirements as starting point for PC1 4Tx. Some minor adjustments to the table may be needed, given than PC1.5 4Tx is based on 4x23 dBm PA, meaning that each PA is operating at its maximum power capability but to support 4Tx PC1 there is only 5 dB difference from PC2 to PC1.

Our initial simulation evaluations revealed also that in some cases that 5..8 RB allocation width at channel edge was still rather challenging and could require more power backoff. It therefore needs to be further evaluated if edge RB definition needs updates.

We aim to provide more detailed analysis also based on measurements in RAN4#115.

Proposal 9: Consider Table 6.2D.2-5 with 4Tx PC1.5 MPR with 2 dB increase to edge allocations as starting point for PC1 MPR and further evaluate need to adjust the MPR to accommodate PC1 output power.

Proposal 10: Further evaluate if L_CRB considered for edge allocations needs to be updated for PC1.

## 3	Conclusion

In this contribution UE RF Tx requirements for NR NTN were discussed. Following observations and proposals were made.

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

Proposal 5: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded.

Observation 3: ACLR choice between 30 and 31 dBc has approximately 0.25 dB impact on output power capability of a PA model.

Observation 4: ACLR choice between 31 and 37 dBc has almost 2 dB impact on output power capability of a PA model.

Proposal 6: NR NTN UE ACLR is 31 dBc for PC2 and PC1.5, both for handheld and non-handheld UEs.

Proposal 7: NR NTN UE ACLR is 31 dBc for PC1 non-handheld UEs.

Proposal 8: NR NTN PC2 and PC1.5 MPR are the same as specified for TN UE in TS 38.101-1

Proposal 9: Consider Table 6.2D.2-5 with 4Tx PC1.5 MPR with 2 dB increase to edge allocations as starting point for PC1 MPR and further evaluate need to adjust the MPR to accommodate PC1 output power.

Proposal 10: Further evaluate if L_CRB considered for edge allocations needs to be updated for PC1.

## 4	References

- R4-2503006, WF on HPUE UE requirement for IoT-NTN, Vivo