3GPP TSG-RAN WG4 Meeting #114bis	R4-2504547

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.10.2.2.2

Source: 		Qualcomm Inc.

Title: 		NR NTN HPUE Rx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

In previous meeting it was agreed to define RSD requirements similarly as in TN specifications. For n256 and n254 0 dB RSD was agreed, but n255 still needs further discussion.

For n255 situation is more challenging than for n256 or n254, as the duplex distance is much smaller. This means that both Tx leakage will be stronger, and Tx-Rx isolation is more challenging to reach without compromises to filter insertion loss. The filter passband may also be wider than just the single operating band, as currently many other L-bands are being specified, and co-banding these is likely.

Observation 1: It is highly likely that n255 can be co-banded together with the other L-bands being specified currently, resulting in wider filter passband.

In addition to filter isolation and Tx noise, it is necessary to consider also the Rx non-linearity in the RSD evaluation. Recently, 54 dBm IIP2 is often used in RSD evaluations for TN bands. An example is provided below, which fully ignores the Tx leakage and is therefore focused to show the impact of Rx non-linearity

Observation 2: Depending on duplexer isolations, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Proposal 1: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

When Tx noise, Rx non-linearity and realistic filter rejections are considered, we propose RSD values in Table 1 and Table 2

Proposal 2: Consider RSD values in Table 1 and Table 2 for PC2 when defining RSD requirements

Table 1: RSD for PC2 UE not supporting Tx diversity

|  | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| n255 | 1.3 | 1.3 | 1.3 | 1.3 |

Table 2: RSD for PC2 UE supporting Tx diversity

|  | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| n255 | 2.4 | 2.4 | 2.4 | 2.4 |

The values in Table 1 and Table 2 assume that Tx ACLR will be defined as 31 dBc. Further analysis is needed if ACLR requirement ends up being different.

Previously it was mentioned in RAN4 that n3 requirements could be re-used for n255. However, it should be noted that n3 has 3 dB relaxed reference sensitivity compared to n255. Therefore, even if the PA noise and Rx non-linearity levels would be exactly the same, the resulting RSD is larger.

Observation 3: n255 RSD is slightly larger than specified for some operating bands with similar duplex distance because n255 has more stringent reference sensitivity.

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: It is highly likely that n255 can be co-banded together with the other L-bands being specified currently, resulting in wider filter passband.

Observation 2: Depending on duplexer isolations, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Proposal 1: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Proposal 2: Consider RSD values in Table 1 and Table 2 for PC2 when defining RSD requirements

Table 1: RSD for PC2 UE not supporting Tx diversity

|  | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| n255 | 1.3 | 1.3 | 1.3 | 1.3 |

Table 2: RSD for PC2 UE supporting Tx diversity

|  | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| n255 | 2.4 | 2.4 | 2.4 | 2.4 |

Observation 3: n255 RSD is slightly larger than specified for some operating bands with similar duplex distance because n255 has more stringent reference sensitivity and similar level of added interference results in larger sensitivity degradation.

## 4	References