3GPP TSG-RAN WG4 Meeting #114bis	R4-2504548

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.10.2.3.1

Source: 		Qualcomm Inc.

Title: 		IoT NTN high power UE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved [1]. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

For UE RF architecture, NR TN HPUE conclusions need further consideration. For NR, PC2 is allowed to use dual Tx (tx diversity) implementation, but PC1.5 is specified to always apply dual Tx. For NB-NTN dual Tx operation may not be necessary for high output powers, though it still is one possible option for implementation. For NTN UL-MIMO has not been considered, as UL-MIMO typically needs good SNR to give benefits. For NB-IoT and cat M1 UL-MIMO has never been specified.

Also the signal characteristics are very different between NR-NTN and NB-NTN, as e.g. NB-IoT transmission bandwidth is only up to 180 kHz. This gives opportunities to take advantage of high power PAs similar to GSM PAs, which can be able to drive high power signal from single Tx.

Handheld satellite phones which have very high output power exist. For example, phones for Iridium system [6]. From the technical specifications it an be seen that this handheld device can provice 4 hours of talk time, while the EU type examination certification shows 37.7 dBm output power.

Figure 1: Snippets of materials available in [1]

The produce regulatory and safety card which is linked to the same web page confirms that this is conducted power provided to the antenna connector. The device weight is not different from a smartphone with a large display.

Observation 1: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

It should be also noted that PC1.5 is not available for IoT NTN. Therefore it is even more important that some higher power option than PC2 is specified. It is not necessary to fully align with TN PC1 requirements but further accommodations can be made to help the feasibility. Possible approaches can be e.g.

- Extending lower power tolerance for PC1 handheld devices

- Considering MPR0 waveform only for narrow allocation in the middle of the band as NB-IoT single tone has lower PAR than multi-tone transmission

- Considering lower nominal output power for IoT NTN PC1

Observation 2: Final requirements can be defined in a manner which facilitate easier feasibility.

Higher than PC2 power level would be extremely useful for IoT NTN, as it improves the link budget. This can make a huge difference in whether connection is possible only in perfect open-sky conditions, or whether connection is also possible e.g. in forest where trees cause additional path loss.

Observation 3: Link budget improvements beyond PC2 are desirable.

Finally, when it comes to component availability and performance, it is perfectly expected that not all components are available and supporting PC1 operation today – there has been no market for them as even standard does not allow PC1 operation in handheld UEs. Therefore, solving the chicken-egg problem can start by enabling the higher power in standards, after which component development is more likely to accelerate.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it is perfectly expected that all components are not commercially available today.

Proposal 1: PC1, including single Tx PC1, is considered feasible for NB-IoT handheld devices.

Proposal 2: RF requirements are defined for PC1 handheld devices.

Cat M1 ACLR for PC2 and PC1

Currently, NB-IoT does not support power class 2. Overall in LTE PC2 requirements have been defined only for channel bandwidths of 5 MHz and above, for example E-UTRA ACLR requirements for power class 2 have not been defined for 1.4 and 3 MHz channel bandwidths, and therefore there is no clear baseline ACLR value for category M1 UE either.

Based on the co-existence evaluations, PC3 ACLR would be sufficient to fulfil co-existence needs. Based on our evaluations the difference in absolute emission level is approximately the same 1 dB as the difference between the usual PC3 ACLR of 30 dBc and PC2 ACLR of 31 dBc. In some cases this difference is enough to justify an increase in MPR and/or A-MPR. Given that the link budget is already tight for NTN, additional power reduction is not good.

Therefore, it is proposed to specify 31 dBc ACLR for cat M1 UEs.

Proposal 3: Specify PC2 ACLR of 31 dB for cat M1 UEs for both PC2 and PC1.

NB-IoT PC2 RF requirements

When PC3 requirements for NB-IoT has been discussed, there was no common alignment on how PA is calibrated, what impairment levels are used and what are the assumptions for the baseband waveform filtering before it is put through the PA. This has resulted in large divergence in results, as observed e.g. for A-MPR proposals for B254, where A-MPR is not yet concluded. Therefore, it is necessary to align the assumptions for NB-IoT evaluations.

Observation 5: RAN4 needs to agree evaluation assumptions for NB-IoT

To evaluate PC2 MPR, we used a simulation model of a PC2 PA, which was pushed to the maximum output power of the model with 12-tone signal to set a MPR=0 reference point. This point was mapped to correspond to 26 dBm at antenna. We applied PC3 MPR values to this reference point, and verified that NB-IoT mask and UTRA ACLR requirements are met. The waveforms contained DC-leakage and IQ-image at 25 dBc level, and CIM3 was set to 60 dBc.

We only used 15 kHz SCS. Therefore, 3.75 kHz single-tone transmissions would still need further confirmation.

Based on this evaluation, we propose to use PC3 MPR also for PC2 at least for 15 kHz SCS.

Power tolerances

For maximum output power tolerance, nearly all specified LTE PC2 bands apply +/- 2 dB. For NR, +2/-3dB is normally used. In our view larger tolerances are more justified for NR as the signals have a lot of variety and implementations cover both 1Tx and 2Tx. However, for NB-IoT, the range of supported waveforms and e.g. their bandwidths is smaller and only 1 Tx implementation is considered. Therefore, more stringent tolerance is justified for NB-IoT.

Proposal 4: NB-IoT PC2 power tolerance is set to +/- 2 dB

There has also been proposals to set a wider range for the upper tolerance. Some proposals mentioned this should only apply for HD-FDD, like NB-IoT and no adjustment would be made FD-FDD variant of cat M1, with the justification that HD-FDD solution likely has smaller insertion loss.

In our view this is not the only reason why an implementation may have higher transmit power at antenna connector. The implementation may be use better PA or better technologies in filters, which may similar result in ability to have higher output power. Correspondingly, HD-FDD does not always mean higher output power, as devices supporting both FD-FDD and HD-FDD for same operating band (e.g. FD-FDD cat M1 and NB-IoT) may not implement separate filters for these. Therefore, we think it is necessary to apply same power tolerances to HD-FDD and FD-FDD

Proposal 5: Power tolerances are applied uniformly to HD-FDD and FD-FDD.

For LTE, PC1 power tolerance is consistently set to +2/-3 dB. We propose to use this also for NB-IoT, both for handheld and non-handheld.

Proposal 6: NB-IoT PC1 power tolerance is set to +2/-3 dB both for handheld and non-handheld devices.

For upper power tolerance, it is also important to note that adding a tolerance in power class definition is not sufficient, but the tolerance needs to be also applied to configured maximum output power, for which a separate Pcmax tolerance is defined:

NB-IoT PC1 requirements for handheld and non-handheld devices

For PC1, using 25 dBc IQ-image, there is a risk of needing high MPR/A-MPR to meet general requirements like NB-IoT mask. Therefore, for PC1 we propose to assume in simulation [30] dBc IQ-image and LO-leakage instead of 25 dBc. Design striving towards PC1 power level could be assumed to be slightly better than lower power classes.

Proposal 7: For NB-IoT PC1, for MPR and A-MPR evaluations assume [30] dBc IQ-image and LO-leakage and 60 dBc CIM3.

These proposals still leave open how to address the baseband filtering. This is not crucial for deciding MPR, but is needed for A-MPR as there are some additional emission requirements immediately outside the channel bandwidth e.g. in band 254.

Proposal 8: Further discuss assumptions for baseband waveform filtering to be used in A-MPR evaluations.

We made initial evaluations via simulations to NB-IoT PC1. Further measurement verification is still needed. In these simulation-based evaluations we assumed 30 dBc IQ-image and DC-leakage and 60 dBc CIM3. Due to the limited time between the RAN4 meetings, the evaluation framework was simplified to still use PC2 PA with 26 dBm at antenna, but the relative difficulty to model PC1 was implemented by making the NB-IoT mask 5 dB more stringent, mapping the 26 dBm to match 31 dBm performance.

In these initial evaluations, it was observed that additional MPR to support PC1 operation may be needed for full 12-tone transmission, and potentially also for allocations which extend to edge of the NB-IoT channel. These include edge positions of 1- and 3-tone allocations and both possible positions for 6-tone allocation.

These evaluations were simplified and based only on simulations. We aim to do more evaluations including also measurements for RAN4#115.

Proposal 9: As a starting point, consider up to 1 dB larger MPR for PC1 operation for tone allocations which extend to the edge of the NB-IoT channel, including also fully allocated 12-tone transmission.

Proposal 10: Further measurement confirmation is needed for PC1 MPR.

P-MPR

In previous meeting P-MPR was discussed to be agreed to be added to configured maximum output power. Generally, filter for higher power classes have been designed with 50% duty cycle in mind. There may be some occasions when the UE may need to protect it’s components. On the other hand, there may also be UEs with filters which can tolerate high output power. To enable re-use of same filter technologies, it would be beneficial not to mandate UE to follow higher than 50% duty cycle for PC2. Unlike the TN solution for NR, this should be optional.

Further discussion in needed for PC1.

Proposal 11: Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2. Capture this allowance into specification.

Proposal 12: Further evaluate potential specification impact from filter power handling for PC1.

## 3	Conclusion

In this IoT NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: Handheld battery powered satellite phones with much higher conducted output power than PC1 are available today.

Observation 2: Final requirements can be defined in a manner which facilitate easier feasibility.

Observation 3: Link budget improvements beyond PC2 are desirable.

Observation 4: Component performance and availability is likely to improve after standards enable using said higher power. Therefore, it is perfectly expected that all components are not commercially available today.

Proposal 1: PC1, including single Tx PC1, is considered feasible for NB-IoT handheld devices.

Proposal 2: RF requirements are defined for PC1 handheld devices.

Observation 5: RAN4 needs to agree evaluation assumptions for NB-IoT

Proposal 5: Power tolerances are applied uniformly to HD-FDD and FD-FDD.

Proposal 6: NB-IoT PC1 power tolerance is set to +2/-3 dB both for handheld and non-handheld devices.

Proposal 7: For NB-IoT PC1, for MPR and A-MPR evaluations assume [30] dBc IQ-image and LO-leakage and 60 dBc CIM3.

Proposal 8: Further discuss assumptions for baseband waveform filtering to be used in A-MPR evaluations.

Proposal 9: As a starting point, consider up to 1 dB larger MPR for PC1 operation for tone allocations which extend to the edge of the NB-IoT channel, including also fully allocated 12-tone transmission.

Proposal 10: Further measurement confirmation is needed for PC1 MPR.

Proposal 11: Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2. Capture this allowance into specification.

Proposal 12: Further evaluate potential specification impact from filter power handling for PC1.

## 4	References

[1]  Iridium 9555, https://www.iridium.com/products/iridium-9555/