3GPP TSG-RAN WG4 Meeting #114bis	R4-2504549

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.10.2.3.2

Source: 		Qualcomm Inc.

Title: 		IoT NTN high power UE Rx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved [1]. One of the objectives is high power UE operation for NTN. In this contribution UE RF requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

In previous meeting it was agreed to define RSD requirements similarly as in TN specifications. For band 256 [0] dB RSD was agreed, but further study was requested on duplexer isolation and PA interference.

In addition to filter isolation and Tx noise, it is necessary to consider also the Rx non-linearity in the RSD evaluation. Recently, 54 dBm IIP2 is often used in RSD evaluations for TN bands. An example is provided below, which fully ignores the Tx leakage and is therefore focused to show the impact of Rx non-linearity.

It can be seen that even with good Tx-Rx isolation of -55 dBm, second order intermodulation starts to be close to the refsens level when output power is increased from 23 to 26 dBm. It can be also seen that when if the total bandwidth of the receiver is wider, and therefore the total noise power is larger, the IMD level causes less impact to the sensitivity. Furthermore, eMTC is based on single receiver, so there is no diversity branch helping to reach.

Impact of Rx IIP2 is clearly visible in LTE TN specifications in TS 36.101. For example, looking at requirements for band 4 which has 400 MHz Tx-Rx separation, reference sensitivity is relaxed for 1.4 and 3 MHz channel bandwidths compared to what is calculated purely from difference in bandwidth.

It can be seen that 1.4 MHz channel bandwidth has 1.5 dB relaxation and 3 MHz channel bandwidth has 0.5 dB relaxation applied.

As NTN bands have more stringent reference sensitivity as a starting point, similar level of Tx noise and Rx IIP2 will result in larger RSD.

Observation 1: Depending on duplexer isolations, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Observation 2: When reference sensitivity power level is lower, the same level of additional interference results in higher RSD.

Observation 3: eMTC is based on single Rx, diversity receiver does not improve performance like for NR

Proposal 1: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

With all impairments considered the reference sensitivity budget looks as follows:

Different PC3 refsens impacts the required RSD. Therefore we propose the following RSD values.

Proposal 4: For PC2 cat M1 FD-FDD, apply 1.2 dB RSD for bands n253 and n255, and apply 1.1 dB RSD for bands n256 and n254.

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made:

Observation 1: Depending on duplexer isolations, Rx IIP2 can cause refsens degradation and it needs to be considered in the RSD evaluation.

Observation 2: When reference sensitivity power level is lower, the same level of additional interference results in higher RSD.

Observation 3: eMTC is based on single Rx, diversity receiver does not improve performance like for NR

Proposal 1: In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Proposal 2: For PC2 cat M1 FD-FDD, apply 1.2 dB RSD for bands n253 and n255, and apply 1.1 dB RSD for bands n256.

## 4	References