3GPP TSG-RAN WG4 Meeting #114bis	R4-2504553

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.30.2

Source: 		Qualcomm Inc.

Title:	 	IoT NTN UL Capacity Enhancements

Document for:		Approval

## 1	Introduction

IoT NTN UL capacity enhancement work is on-going in RAN1. RAN4 has had initial discussion on whether any specified scheme results in RAN4 specification impact. In this contribution RAN4 specification impact is further discussed.

## 2	Discussion

#### Issue 2-1: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - No RF impact on OCC2 for IoT NTN.

#### Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - No RF impact for <1ms gap length.

  - FFS for longer gaps.

#### Issue 2-3: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether solutions (e.g., CFO grouping) for solving frequency inaccuracy between OCC UEs would have potential RF impact.

In RAN4#114 following agreements were reached:

RAN1 is still working on the open items under issue 2-2 and issue 2-3. Most likely outcome is that RAN1 may change their specifications on transmission gaps and segmentation when these happen together with OCC. Given that RAN4 requirements only reference RAN1 specifications, there would be no direct impact to RAN4 specifications.

IoT NTN scheduling is fully controlled by the network. Network decides which UEs it pairs together, how many repetitions are configured and therefore has control over transmission gaps and segmentation. This means also that network has means to avoid worst case configurations.

A requirement to re-synchronize Tx frequency periodically would not be helpful, as the baseline frequency error requirement does not change and it would be sufficient to synchronize to just meet the frequency error. As part of normal operation, UE will synchronize with network and maintain the frequency error.

Observation 1: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 2: It is part of normal operation of the UE to maintain sync with the network and not violate frequency error requirements.

The work item for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was just updated in RAN meeting in December 2024 [1]. UE RF specification is not listed as an impacted specification despite RAN spending multiple meetings discussing potential RF impact. This has been visible also in status reports to RAN. Therefore, it can be concluded that RAN does not expect UE RF specification impact from IoT NTN capacity enhancements.

Proposal 1: There is no RAN4 UE RF specification impact from IoT NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1:

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

Question:

From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

RAN4 has also received an LS from RAN1 asking about feasibility of DM-RS transmissions:

RAN1 meeting minutes provide further background information, as the reached working assumption includes the following: “Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.”

For NB-IoT DMRS duration is one symbol. The duration of the blanked symbol with 3.75 kHz SCS is approximately 267us. Our internal analysis confirmed that there is no issue in maintaining phase continuity over a single blanked symbol. This is aligned with agreement in previous meeting that there is no RF impact on gaps under 1ms.

Proposal 2: Reply back to RAN1 that it is feasible to introduce support of TDM DMRS as per the RAN1 description.

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements were discussed and following observation and proposal was made:

Observation 1: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 2: It is part of normal operation of the UE to maintain sync with the network and not violate frequency error requirements.

Proposal 1: There is no RAN4 UE RF specification impact from IoT NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

Proposal 2: Reply back to RAN1 that it is feasible to introduce support of TDM DMRS as per the RAN1 description.

## 4	References

[1]	RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)