# 3GPP TSG-RAN WG4 Meeting #114		        R4-2504556

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.30.2

Source: 	Huawei, HiSilicon

Title:	On TDM DMRS for 3.75kHz SCS OCC for IoT NTN

Document for:	Approval

# 1	Introduction

RAN1 has made the following working assumption on the DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1:

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

In the LS [1], RAN1 asks RAN4 if it is feasible to introduce such support of TDM DMRS in Rel-19.

# 2	Discussion

The TDM DRMS scheme can be illustrated as below [2]:

Figure 1: TDM of DMRS for 3.75kHz [2]

From the perspective of one UE, the DMRS symbol is muted for two consecutive slots in every 4 slots. The duration of a DMRS symbol is 275µs for 3.75kHz SCS. As discussed before, sufficient phase continuity can be maintained for gap length < 1ms. No RF impact is expected as seen from the agreement in the WF from last meeting [3].

#### Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - No RF impact for <1ms gap length.

  - FFS for longer gaps.

Proposal 1: Reply to RAN1 that no RF impact is expected to support the TDM DMRS scheme for 3.75kHz SCS OCC for NPUSCH format 1.

Proposal 2: The text proposal for the reply LS is as follows:

----------------------------------------------------- TP for Reply LS --------------------------------------------------------

RAN4 has concluded that: No RF impact for <1ms gap length. The duration of a DMRS symbol is 275µs for 3.75kHz SCS. Hence, no RF impact is expected from DMRS muting in the TDM DMRS scheme. From a RAN4 perspective, it is feasible to introduce support of TDM DMRS as described in RAN1’s LS, in Rel-19.

----------------------------------------------------- End of TP --------------------------------------------------------

RAN1 is likely to design mechanisms to avoid the OCC pattern being interrupted by longer gaps, such as UCG, NPRACH occasions, segmentation gap, etc. Hence, it’s better for RAN4 to wait for RAN1’s progress before discussing the potential RF impact of longer gaps.

Proposal 3: RAN4 to wait for RAN1’s progress before discussing the potential RF impact of longer gaps.

# 3	Conclusion

Triggered by RAN1’ LS, the following proposals are made:

Proposal 1: Reply to RAN1 that no RF impact is expected to support the TDM DMRS scheme for 3.75kHz SCS OCC for NPUSCH format 1.

Proposal 2: The text proposal for the reply LS is as follows:

----------------------------------------------------- TP for Reply LS -----------------------------------------------------

RAN4 has concluded that: No RF impact for <1ms gap length. The duration of a DMRS symbol is 275µs for 3.75kHz SCS. Hence, no RF impact is expected from DMRS muting in the TDM DMRS scheme. From a RAN4 perspective, it is feasible to introduce support of TDM DMRS as described in RAN1’s LS, in Rel-19.

----------------------------------------------------- End of TP --------------------------------------------------------

Proposal 3: RAN4 to wait for RAN1’s progress before discussing the potential RF impact of longer gaps.

# References

- R4-2503109 LS on TDM DMRS for 3.75kHz SCS OCC, RAN1, RAN4#114bis

- R1-2500085 Discussion on UL capacity enhancements for IoT NTN, RAN1#120

- R4-2502373 Way forward for IoT_NTN_Ph3, MTK, RAN4#114