3GPP TSG-RAN WG4 Meeting #114bis	R4- 2504557

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.24.2.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

As per the WF [2] in the last meeting, RAN4 will continue the discussions on the following RF requirements for A-IoT device 1:

Transmitter:

- Backscattered output power

- Modulation quality

- Occupied bandwidth

- Emission requirements

Receiver:

- Reference sensitivity

- Maximum input power

# 2	Discussion

### 2.1 Overview

Similar to RFID tags, a practical AIoT device would be made of an IC chip and an antenna printed on certain substrate such as plastic, paper, fabric, etc. The same IC chip with different antenna designs will exhibit different performances. The most commonly used antenna design is a dipole antenna.

Figure 1：Commercial UHF RFID tags (source: www.rfidlabel.com)

The complexity of evaluating the radiated performances largely come from the orientation property of the antenna, including directivity and polarisation.

Unless otherwise stated, the radiated performances discussed below are evaluated in the far-field of Tx/Rx antenna under free space propagation.

### 2.2 Receiver RF Requirements

Receiver Sensitivity:

In the study phase, around -36dBm sensitivity was reported by companies. Assuming that NF=24dB, and the device can receive signal anywhere in band n8 DL (i.e. BW=35MHz), the SNR at the device antenna connector would be:

SNR = -36 – (-174+24+10log10(35e6) = 38.6 dB

It’s unlikely that a device needs such high SNR to demodulate the R2D signal. Instead of using conventional LLS method, it’s proposed to allow companies to report the feasible sensitivity level obtained by measurements.

Proposal 1: In order to determine the receiver sensitivity requirement, allow companies to report the achievable sensitivity levels based on measurements.

The device cannot access the network unless being triggered by the reader. Unlike other 3GPP RATs, there’re no continuous DL data transmissions for AIoT. One way to evaluate the receiver performance is to measure the success rate of the device that correctly responds to the inventory procedure.

Similar to the Select-Query-Ack commands for RFID, the inventory procedure consists of the message sequence of Paging, R2D trigger, MSG1 (RN16), MSG2 (ACK) and MSG3 (device ID).

Proposal 2: The AIoT inventory procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. RMC) are TBD.

Differentiating tags based on sensitivity performance is common for RFID. GS1 has developed a comprehensive grading system for tagged-items, which defines a set of “grades” based on sensitivity and backscatter power at various orientations [3].

Based on our initial evaluation of some prototype devices, the Rx sensitivity of around  -30 to -35 dBm is achievable. In order to improve DL coverage and to encourage vendors to develop higher performance products, we propose to define a higher sensitivity level in addition to the basic level.

Proposal 3: Consider to define multiple levels of sensitivity requirements, for example:

Table 1: Device Sensitivity Levels

| Sensitivity Level | REFSENS (dBm) |
| --- | --- |
| L1 | max gain direction：≤ [-30~-35] other directions：TBD |
| L2 | max gain direction：≤ [-40~-45] other directions：TBD |

Orientation variation:

Due to its simplicity, dipole antennas are widely used in RFID tags. The antenna gain pattern of a dipole is the well-known donut shape with nulls along the antenna axis. Consequently, the OTA sensitivity of the tag exhibits similar orientation property.

Figure 2: Orientation sensitivity from a commercial UHF RFID tag datasheet (Source: General specification for CTU07601-465)

There can be multiple ways to account for the orientation property in the Rx sensitivity requirements. For example,

Option 1: Only define EIS in max gain direction, effectively determined by chip demod sensitivity - max antenna gain

Option 2: TRS by scanning the device from different angles and averaging the results.

Option 3: Sensitivity requirements at different orientation angles, similar to GS1 [3], such as:

azimuth: max gain direction  +/- 0/30/60 degrees

elevation: 0/90 or  0/30/60/90 degrees

Option 4: max allowed degradation of X dB at Y%-tile CDF, similar to spherical coverage

For example, the HPBW (half-power beamwidth) of an ideal half-wave dipole is around 78 degrees. Hence, for 43% (=78/180) spherical coverage, the degradation is ≤ 3dB. If polarisation mismatch is added, the degradation is ≤ 5.2dB.

Proposal 4: RAN4 to discuss how to define the Rx sensitivity requirements for different orientation angles.

Frequency variation:

Due to ultra-low complexity, the device can only have simple antenna and matching network. The antenna gain would have larger variation across operating frequencies, compared with conventional UEs.

Figure 3 Sensitivity vs frequency from a commercial UHF RFID tag datasheet (Source: General specification for CTU07601-465)

Proposal 5: Additional margin should be included in the sensitivity requirements to account for the performance variation in the operating frequency range.

Maximum input power:

The device receiver has very limited dynamic range, which is estimated to be around 30 to 40dB. Larger range is possible such as using AGC, but at the cost of increased complexity and power consumption. Given the sensitivity requirement, we propose the max input level as:

Proposal 6: It’s recommended to set the maximum input level to [-3] dBm for AIoT devices.

This does not mean that the device wills top working for higher input power levels, but the performance might be worsen.

### 2.2 Transmitter RF requirements

Backscatter output power:

For A-IoT D1T1-B, the CW node is outside the topology. And the CW node is assumed not to be co-located with the active reader. The CW2D and D2R path loss are likely to be different.

The backscatter loss or reflection gain can be defined as: the backscattered power – incident CW power.

Note that the backscatter power shall be the sum of the power of the left and right side band, excluding the carrier itself.

Given reciprocity, if the antenna directivity has been evaluated in the receiver sensitivity requirements, it does not need to be assessed again in the Tx requirements. Hence, only the max gain direction might need to be considered for both the incident and backscattered signal.

When considering UL link budget, the path loss of CW2D and D2R as well as the backscatter loss at the device should be included. The worst case for the UL budget seems to be when the device is located in the middle of CW2D and D2R propagation path. Hence, it’s meaningful to evaluate the backscatter loss at medium level of CW incident power.

Proposal 7: Consider to evaluate the backscatter loss at given CW incident power. The limit on the backscatter loss and the corresponding incident power level are TBD.

Modulation signal quality:

In general, the modulation signal quality sets an upper bound of the SNR seen by the receiver. The potential requirement on the modulation quality depends on the Tx signal design as well as the demodulation processing at the receiver.

As per RAN1 agreement, the device may support OOK or BPSK for D2R. It’s worth noting that for backscatter communication, the modulation is implemented in analogue domain by switching impedances, which is very different from digital modulation seen in other 3GPP RATs.

For OOK modulation in backscatter communications, some performance metrics such as delta-RCS (Radar Cross-Section), change in reflection coefficient have been studied [6]. However, its impact on the D2R demodulation performance has yet to be quantified.

Observation 1: For OOK modulation in backscatter communications, some performance metrics such as delta-RCS (Radar Cross-Section), change in reflection coefficient have been studied [6]. However, its impact on the D2R demodulation performance has yet to be quantified.

For BPSK modulation, one might consider the conventional EVM metric, which compares the received symbols with the predefined points in the reference constellation. Due to the use of line code for D2R, there’re one (R=1) or multiple (R>1) pairs of chips for each information bit. As shown in Figure 4 below, a pair of (+1, -1) chips correspond to information bit 1, and a pair of (-1,+1) chips is for information bit 0, in case of R=1.  During backscatter, the carrier phase is expected to be reverted between the two chips in a pair. However, for a practical device, the phase change might not be 180 degrees. Furthermore, the backscatter power for the two chips can be different as well, owing to the switch of the load impedance. In other words, the amplitude of the BPSK symbols can vary.

Figure 4: One option of D2R time domain waveform generation [5]

Given the nature of line coding, the demodulator at the reader is likely to jointly process pairs of chips (i.e., two BPSK symbols) rather than one BPSK symbol at a time. The Euclidian distance between the two BPSK symbols might be more important than the phase difference alone.

Observation 2: Due to the nature of line coding, the D2R demodulator is likely to process the OOK/BPSK symbols in pairs. The Euclidian distance between the two received OOK/BPSK symbols play an important role.

Proposal 8: When evaluating potential modulation quality metrics, the reference demodulation process should be confirmed. The effect of the quality metric on the demodulation performance should be quantified.

Occupied bandwidth:

The OBW requirement depends on the D2R signal design and the discussion on the system channel bandwidth.

SEM + Spurious emissions:

The NR SEM could be used as the starting point, since the devices operate on NR bands. Further simplification is possible, given the relatively low backscatter power. The exact mask also depends on D2R signal design, system channel BW, etc.

The NR general spurious emission limits can be used as the starting point, and further discuss the OOB boundary.

Proposal 9: Define EIRP emission requirement to be evaluated at the max gain direction.

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Proposal 1: In order to determine the receiver sensitivity requirement, allow companies to report the achievable sensitivity levels based on measurements.

Proposal 2: The AIoT inventory procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. RMC) are TBD.

Proposal 3: Consider to define multiple levels of sensitivity requirements, for example:

Table 1: Device Sensitivity Levels

| Sensitivity Level | REFSENS (dBm) |
| --- | --- |
| L1 | max gain direction：≤ [-30~-35] other directions：TBD |
| L2 | max gain direction：≤ [-40~-45] other directions：TBD |

Proposal 4: RAN4 to discuss how to define the Rx sensitivity requirements for different orientation angles.

Proposal 5: Additional margin should be included in the sensitivity requirements to account for the performance variation in the operating frequency range.

Proposal 6: It’s recommended to set the maximum input level to [-3] dBm for AIoT devices.

Proposal 7: Consider to evaluate the backscatter loss at given CW incident power. The limit on the backscatter loss and the corresponding incident power level are TBD.

Observation 1: For OOK modulation in backscatter communications, some performance metrics such as delta-RCS (Radar Cross-Section), change in reflection coefficient have been studied [6]. However, its impact on the D2R demodulation performance has yet to be quantified.

Observation 2: Due to the nature of line coding, the D2R demodulator is likely to process the OOK/BPSK symbols in pairs. The Euclidian distance between the two received OOK/BPSK symbols play an important role.

Proposal 8: When evaluating potential modulation quality metrics, the reference demodulation process should be confirmed. The effect of the quality metric on the demodulation performance should be quantified.

Proposal 9: Define EIRP emission requirement to be evaluated at the max gain direction.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN#106

- R4-2502858 WF on requirements for system parameters and A-IoT device, CMCC, RAN4#114

- GS1 Tagged-Item Performance Protocol (TIPP) Tagged-Item Grading: Grade Definitions Guideline

- R4-2413030 On the RF requirements for Ambient IoT devices, Huawei, HiSilicon, RAN4#112

- R1-2409418 On general aspects of physical layer design for Ambient IoT, Huawei, HiSilicon, RAN4#119

- On the measurement of the Delta Radar Cross Section for UHF tags, S. Skali, et al, IEEE International Conference on RFID, 2009