3GPP TSG-RAN WG4 Meeting # 114-Bis						    		R4-2504559

Wuhan China 07-11 April 2025

Title: Discussion on coexistence simulation parameters for GEO and LEO VSAT

Agenda Item: 7.11.2

Source: Sharp

Document for: Decision

# 1	Introduction

# At RAN4#113@Orland, we agreed the way forward R4-2419909[1] of the Ku-band co-existence study model of Ku band VSAT phased array antenna model and VSAT Max Tx antenna gain. The WF for the VSAT Max Tx antenna gain is as follows.

### Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

At RAN4#113@Orlando it was proposed in R4-2419070 [2] to use a Conducted power (before Ohmic loss) per antenna element (dBm) of 4.44dBm, and some initial consensus was made on it.

At RAN4#114@Athens regarding UE RF, the Maximum EIRP was agreed in R4-2502288 [3] to be 88.2dBm. In light of this agreement, we believe it is necessary to update the flat-panel model used for the co-existence study.

At RAN4#114@Athens it was agreed in [4] that R4-2502315[5] would be used as the container to hold the running agreements on Ku band simulation assumption. The current state of the agreements as captured in R4-2502315 for table 6a.2.2.2-1 and 6a.2.2.2-2 are copied below for reference.

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| 1 | VSAT Phased array antenna Characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 0/ 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | TBD (dependent on decisions on EIRP) |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). |  |  |

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | Phased array antenna |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |
| Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO |
| Tx antenna gain | Option 1 - 36.6 dBi, Option 2 – 36.2 dBi |
| Antenna elevation angle | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

# 2	Discussions

In the approved WF [1], 41.3 dBi Tx antenna gain has been agreed for GEO. However, in the same WF, 45 x 45 elements has been agreed as phased array antenna configuration, which leads to 10*log1045*45+3.5≈36.6 dBi Tx antenna gain, where 3.5 dBi is per-element antenna gain.

In our understanding, the conflict in Tx antenna gain values (i.e. 41.3 dBi vs 36.6 dBi) lies in the difference in the target scenario, in that the 45 x 45 antenna element configuration is for GEO and LEO, and whereas 41.3 dBi Tx antenna gain is only for GEO. Therefore, we need different antenna configuration for phased array.

To achieve a Tx antenna gain of 41.3 dBi, 78 x 78 antenna elements are required with 3.5 dBi element gain. Thus, for GEO, we propose a new antenna configuration of 78 x 78 with the size of 84 cm square (similar to the size of a circle with 100 cm diameter).

Proposal 1: A new antenna configuration of 78 x 78 with the size of 84 cm square (similar to the size of a circle with 100 cm diameter) is used only for GEO.

On the other hand, for LEO, 88.2 dBm EIRP is too large for a 45 x 45 antenna configuration. Based on our understanding, 36.6 dBi + 37.5 dBm = 74.1 dBm EIRP for Option 1 or 36.2 dBi + 37.5 dBm = 73.7 dBm for Option 2 is enough for LEO. In addition to that, since there are some use cases that require only low data rates (i.e. small aperture antenna is enough) in GEO, an antenna configuration of 45 x 45 is also considered valid in GEO. Here, the Tx power is calculated using 4.44 dBm per-element Tx power multiplied with 45 x 45. (i.e. 37.5dBm = 4.44 + 10*log10(45*45) dBm)

Proposal 2.1: EIRP is set as 74.1 dBm for Option 1 and 73.7 dBm for Option 2 for GEO and LEO.

Proposal 2.2: Conducted power (before Ohmic loss) per antenna element is set to 4.44 dBm for GEO and LEO.

To resolve the conflict as identified above, it is necessary to differentiate the simulation parameters between coexistence parameters sets, where a first set is provided for a VSAT device that is connected to an only GEO, and a second set is provided for a VSAT device that is connected to GEO and LEO. Further to this differentiation, and as it agreed in R4-2502290 at RAN4#114 meeting, we will maintain the simulation parameters in a single document. The starting document at RAN4#114 meeting is available in R4-2502315, and the table(s) in that document are used as a baseline for the proposed revisions below.

Proposal 3.1: Revise the table 6a.2.2.2-1 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 3.2: Revise the table 6a.2.2.2-1 of R4-2502315 to add Note 5

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| 1 | VSAT Phased array antenna Characteristics |  | Only GEO |  | GEO and LEO |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  | Antenna pattern |  | Table 2-2 and 2-3 (TR 38.921) |  | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 |  | Element gain (dBi) (Note 1) |  | 0/ 3.5 |  | 0/ 3.5 |
| 1.3 |  | Y axis / Z axis element 3 dB beam width of single element (degree) |  | 90° for Y axis 90° for Z axis |  | 90° for Y axis 90° for Z axis |
| 1.4 |  | Y axis / Z axis element front-to-back ratio (dB) |  | 30 dB |  | 30 dB |
| 1.5 |  | Antenna polarization |  | Circular (RHCP or LHCP; as a starting point) (NOTE5） |  | Circular (RHCP or LHCP; as a starting point) (NOTE5） |
| 1.6 |  | Antenna array configuration (Y axis × Z axis)(Note 3) |  | 78 x 78 elements |  | 45x45 elements |
| 1.7 |  | Number of supported polarizations, P |  | 1 |  | 1 |
| 1.8 |  | Y axis / Z axis radiating element spacing |  | 0.5 lambda |  | 0.5 lambda |
| 1.9 |  | Array Ohmic loss (dB) (Note 1) |  | 2 |  | 2 |
| 1.10 |  | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) |  | TBD (dependent on decisions on EIRP) |  | 4.44 |
| 1.11 |  | Maximum coverage angle in the horizontal plane (degrees) |  | 0~360 degrees |  | 0~360 degrees |
| 1.12 |  | Vertical coverage range (degrees) between beam direction and normal direction |  | 0~60 degrees |  | 0~60 degrees |
| 1.13 |  | Normal direction |  | Toward X+ axis |  | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Parameters for the phased array antenna should be defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-2: Other NTN UE parameters (Phased array antenna)

|  |  |  | Only GEO | GEO and LEO |  |
| --- | --- | --- | --- | --- | --- |
| 2 | Characteristics |  | Phased array antenna | Phased array antenna |  |
| 2.1 | Frequency band |  | Ku-band (i.e. 11 GHz DL and 14 GHz UL) | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |  |
| 2.2 | Antenna type and configuration |  | Directional Section 6.4.1 of [6] with 100 cm equivalent aperture diameter | Directional Section 6.4.1 of [6] with 60 cm equivalent aperture diameter |  |
| 2.3 | Polarization |  | Circular (NOTE2) | Circular (NOTE2) |  |
| 2.4 | Antenna temperature |  | 150 K | 150 K |  |
| 2.5 | Noise figure |  | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO); Other options not precluded | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |  |
| 2.6 | Tx transmit power |  | 50 W for GSO and 10 W (40 dBm) for NGSO | 50 W for GSO and 10 W (40 dBm) for NGSO |  |
| 2.7 | Antenna elevation angle |  | 25˚ and 90˚ | 25˚ and 90˚ |  |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Proposal 4.1: Revise the table 6a.2.2.2-2 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 4.2: Move Rx Antenna gain, and Tx antenna gain from table 6A.2.2.2-2 of R4-2502315 to 6A.2.2.2-X of R4-2502315

Proposal 4.3: Revise the table 6a.2.2.2-2 of R4-2502315 to add Note 2

Proposal 4.4: Add a left column to table 6a.2.2.2-2 of R4-2502315 indicating row numbering

Parameters for the parabolic antenna should be defined in Table 6a.2.2.2-X. The following describes the WF items in R4-2419909 (i.e. 100 cm aperture (41.3dBi), 88.2 dBm EIRP).

Table 6a.2.2.2-X: NTN UE parameters (Parabolic antenna)

|  |  | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| X | Characteristics | Parabolic antenna | Parabolic antenna |
| X.1 | Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| X.2 | Antenna type and configuration | Directional Section 6.4.1 of [6] with 100 cm equivalent aperture diameter | Directional Section 6.4.1 of [6] with 60 cm equivalent aperture diameter |
| X.3 | Polarization | Circular (NOTE2) | Circular (NOTE2) |
| X.4 | Rx Antenna gain | 39.0 dBi | 34.3 dBi |
| X.5 | Antenna temperature | 150 K | 150 K |
| X.6 | Max EIRP | 88.2dBm | 74.1 dBm for Option 1 and 73.7 dBm for Option 2 |
| X.7 | Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO | 50 W for GSO and 10 W (40 dBm) for NGSO |
| X.8 | Tx antenna gain | 41.3 dBi | Option 1 - 36.6 dBi, Option 2 – 36.2 dBi |
| X.9 | Antenna elevation angle | 25˚ and 90˚ | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Proposal 5: Create the table 6a.2.2.2-X of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

# 3	Summary

Proposal 1: A new antenna configuration of 78 x 78 with the size of 84 cm square (similar to the size of a circle with 100 cm diameter) is used only for GEO.

Proposal 2.1: EIRP is set as 74.1 dBm for Option 1 and 73.7 dBm for Option 2 for GEO and LEO.

Proposal 2.2: Conducted power (before Ohmic loss) per antenna element is set to 4.44 dBm for GEO and LEO.

Proposal 3.1: Revise the table 6a.2.2.2-1 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 3.2: Revise the table 6a.2.2.2-1 of R4-2502315 to add Note 5

Proposal 4.1: Revise the table 6a.2.2.2-2 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 4.2: Move Rx Antenna gain, and Tx antenna gain from table 6A.2.2.2-2 of R4-2502315 to 6A.2.2.2-X of R4-2502315

Proposal 4.3: Revise the table 6a.2.2.2-2 of R4-2502315 to add Note 2

Proposal 4.4: Add a left column to table 6a.2.2.2-2 of R4-2502315 indicating row numbering

Proposal 5: Create the table 6a.2.2.2-3 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

# 4	References

R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, RAN4 #113

R4-2419070, Discussion on coexistence study for NR NTN Ku Band, Huawei, HiSilicon, RAN4 #113

R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

R4-2502290,  Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4 #114

R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), RAN4 #114

38.811, Study on New Radio (NR) to support non-terrestrial networks