3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504565

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              Discussion on L3 measurement enhancement by optimizing Rx BSF

Agenda item:			7.17.2.1

Document for:	Discussion

Introduction

In last meeting, RAN4 had extensively discussions on L3 measurement enhancement by optimizing Rx BSF. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

From our understanding, if we treat BSF measurement as an event, and the BSF measurement-related event is appropriately configured, the corresponding thresholds, hysteresis and TTT settings should be considered. And for the measurement event triggering, UE should continuously check if an entry condition is fulfilled for the measurement results during TTT. And under such mechanism, too-early, ping-ping, and other unintended events can be ensured that won’t happen AMAP. However, robust issue is not such top in our case. In this sense,

Proposal 1: If the BSF measurement is treated as an event, the corresponding hysteresis and timeToTrigger parameters can be added along with, otherwise, suggest not to add the two parameters.

2.2 Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Compared to TTT, the discussion on conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” is more important.

From our understanding, the quit condition is necessary. Based on the discussion happened in previous meetings, we prefer that UE fall back to normal measurement mode after UE has reported qualified measurement results. Since UE in wireless communication system needs to monitor the channel quality not only for serving cell, but also for some neighbor cell. That is to say, although UE is doing business with serving cell, it also needs to find some candidate neighbor cell and monitor them to shorten traffic interruption when serving cell quality becomes poor. Such thing may happen at cell edge. Fast L3 measurement can expedite neighbor cell related action, avoid UE suffering from long interruption due to neighbor cell search/measurement latency. And if UE has reported valid measurement results, that can mean the HO expectation is realized, and can fallback to single panel. Besides, in some cases, e.g., UE already moves to cell center, there is no need to receive signals by using multiple panel simultaneously for saving power.

However, there might be an un-avoided possibility that: Once UE reports the qualified measurement result, it fallbacks to the normal measurement, while NW is not aware of such UE behaviour, and due to, e.g., performance maintenance/monitoring demands per serving cell or some other reasons, UE is expected to continue BSF measuring over a period of time (e.g.,..) with BSF related configurations. From this, measurement wont be done accurately. To solve the issue:

- The maximum period of time is determined using a timer. For L3 BSF measurement, upon UE the qualified measurement results reporting, the timer runs, and when the time expires, the BSF measurement is deactivated, UE fallback to the normal measurement.

- UE preference indication via UAI. e.g., overheatingAssistance. Based on the explicit UAI indication, NW will know the UE measurement status and it has overheating issue.

Proposal 2: Apart from the threshold conditions, the UE is allowed to fallback to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results. To enable such quit mechanism, the following solutions can be considered:

- Define the maximum period of time of UE to continue BSF measuring after it has reported the qualified measurement result as a timer.

- UE preference indication via UAI. e.g., overheatingAssistance.

2.3 Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

From our understanding, the legacy Rx beam sweeping procedure assumes that UE does not move or rotate significantly/quickly from the start of the beam sweep until it is complete, otherwise, the worst case is that the UE reports false beams, which were the best ones when the measurement was performed, but become rather weaker when the measurement period is complete, and the result is reported. In this sense, one of the benefits on faster beam sweeping in our case is that short delay in cell detection and measurement will shorten the difference between valid RSRP measurement report and actual condition when the report is provided, which can improve the system performance.

However, there is a tradeoff between mobility performance and number of samples to be measured

- In the previous release, there were two candidate values for the N, that is N=4 and N=8. The N=8 was finally compromised by taking mobility performance verification based on SLS and UE antenna implementation (spherical coverage) into consideration. Actually, from our understanding, from system performance point of view only, the N=4 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies.

If N/measured sample is reduced, the performance degradation issue may unavoidably arise. While, since no SLS results were obtained based on the new sweeping behaviour, we cant observe how much performance loss the UE will experience with different number of Rx beam sweeping, and cant conclude how to reduce the N, i.e, the total number of required samples for measurement accordingly under the new scenario. But we prefer to use N=4, the corresponding total samples is 20= 5 × 4 as baseline, at least one of the candidate values to define the delay requirements of inter-/intra-frequency measurement for Rel-19 L3 multi-Rx measurement, since as we mentioned before, the performance based on N=4 can be accepted by the companies in the previous release.

Proposal 3: Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). N=4 can be as baseline.

3 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for L3 measurement enhancement by optimizing Rx BSF. The following observations and proposals are obtained:

Proposal 1: If the BSF measurement is treated as an event, the corresponding hysteresis and timeToTrigger parameters can be added along with, otherwise, suggest not to add the two parameters.

Proposal 2: Apart from the threshold conditions, the UE is allowed to fallback to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results. To enable such quit mechanism, the following solutions can be considered:

- Define the maximum period of time of UE to continue BSF measuring after it has reported the qualified measurement result as a timer.

- UE preference indication via UAI. e.g., overheatingAssistance.

Proposal 3: Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). N=4 can be as baseline.

4 References

[1] R4-2502593 “WF on RRM requirements for NR_RRM_Ph5_Part2”, CATT