TSG-RAN Working Group 4 (Radio) meeting #114bis	  R4-2504569

Wuhan, China, 7 – 11 April 2025

Source:	Ericsson

Title:	A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour

Agenda item:	5.3.1

Document for:	Approval

# Background

In this contribution we consider the A-MPR regions, presenting A-MPR results, we make a proposal for increase range for common PUCCH configuration, and consider the optionality in earlier releases.

# Discussion: A-MPR for NS_05 and handling of PUCCH

## The A-MPR regions for NS_05

The A-MPR regions for NS_05 agreed in [1] are shown below.

Table 1: A-MPR regions for NS_05

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | RBstart | LCRB |
| 10 | FC < 1930.7 | >0 | ≥ 1.5*RBstart + 4.7 MHz/12/SCS | ≤ 1.8 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |
| 15 | 1927.5 ≤ FC < 1932.5 | >0 | ≥ 1.45*RBstart + 4.68 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | ≤ 3.6 MHz/12/SCS |
| 15 | 1932.5 ≤ FC < 1938.7 | ≤ 0.54 MHz/12/SCS | ≥ 12.96 MHz/12/SCS | ≤ 2.16 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |
| 20 | FC < 1945.7 | >0 | ≥ 1.5*RBstart + 4.14 MHz/12/SCS | ≤ 5.04 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and ≤ 1.5*RBstart + 4.14MHz/12/SCS |
| NOTE 1: The A-MPR values are specified in Table TBD NOTE 2: The sub-carrier spacing SCS expressed in MHz |  |  |  |  |  |

Proposed A-MPR values will be presented in this contribution. Nevertheless, we make the following observation, obvious but will be used below to facilitate configuration of common PUCCH in a cell:

Observation 1: for RB allocations starting in the Region A2 with ≤ 5.04 MHz/12/SCS at the lower carrier edge, the required A-MPR is reduced when the RBstart is such that the IMD3 falls in the -25 dBm/MHz range below 1906.6 MHz as compared to that needed when the IMD3 falls in the -30 dBm/MHz range.

## PUCCH transmissions for meeting limits without A-MPR

In [2] it is proposed to study improvements of the A-MPR regions such that the PUCCH resource sets can be configured without A-MPR using PRB offset (clause 9.2.1 in 38.213). The region A2 ends at RBstart = 28 (5.04MHz), which implies a required PRB offset of 28 PRB from the lower carrier edge to avoid A-MPR for common PUCCH. Based on observation 1, one solution would be to split Region A2 into two regions. Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A3 to be between 10 (1.80MHz) < RBstart ≤ 28 (5.04 MHz).

Proposal 1: Split the agreed Region A2 into two new regions: Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A3 to be between 10 (1.80MHz) < RBstart ≤ 28 (5.04 MHz).

This would enable to place the common PUCCH in region A3 with less backoff than in current Region A2 (and the new proposed Region A2) which will contain a higher Backoff need to meet the requirements.

The NR channels in n1 should also accommodate dynamic sharing with LTE. For LTE the PUCCH offset must be set to 24 PRB to avoid A-MPR for PUCCH transmissions (corresponding to a 26-27 PRB offset for NR with its smaller internal GB) as needed to meet the PHS limit below 1915.7MHz. For sharing with LTE, it is beneficial to configure the NR PUCCH outside the LTE PUCCH with a PRB offset less than 26-27 PRB rather than inside not to further fragment the NR PUSCH transmissions, i.e. avoid a further reduction of the PUSCH region with lower or no A-MPR inside the PUCCH allocation. The PUSCH scheduled outside the PUCCH allocation requires an A-MPR > 2 dB within Region A2.

A smaller PRB offset for PUCCH would be beneficial also without DSS even though legacy UEs not supporting the modified A-MPR indicated by NS_05/NS_05U may not benefit from the A-MPR re-evaluated.

## Simulation requirements and assumptions

The DECT additional Spectrum Emission Mask requirements:

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

The following assumptions for the simulations have been applied for the last 10 years:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -28dBc

- Image: -28dBc

- CIM3: -60dBc

- CIM5: -70dBc

In practice, an IQ image rejection exceeding 34 dB is needed for meet EVM requirements for 256QAM and some of the UL non-contiguous CA emissions requirements are impossible to meet unless an LO leakage better that 28dBc is achieved. If MPR simulation assumption would use more current representative performance levels of added noises, the A-MPR needed in Region A2 and Region A3 would be reduced. In fact, by this we may lower and even avoid any A-MPR for Region A2 and Region A3, at least for DFTS-OFDM QPSK that is used for determining A-MPR for PUCCH. Hence, we propose following changes:

- Carrier Leakage: -34dBc

- IQ Image rejection: -34dBc

Proposal 2: For the simulation assumptions consider the following new assumptions

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -34dBc

- Image: -34dBc

- CIM3: -60dBc

- CIM5: -70dBc

## Simulation Results

Table 2 shows the modification we would like to see to the regions, where introduction of Region A3 is done and some modifications to the values in Region A1 and A2.

Table 2: A-MPR regions for NS_05 with exception for PUCCH

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | RBstart | LCRB | RBstart | LCRB |
| 10 | FC < 1930.7 | ≥0 | ≥ 1.5*RBstart + 4.7 MHz/12/SCS | ≤ 1.8 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | ≥0 | ≥ 1.45*RBstart + 4.68 MHz/12/SCS | ≤ 3.6 MHz/12/SCS | ≤ 3.6 MHz/12/SCS |  |  |
| 15 | 1932.5 ≤ FC < 1938.7 | ≤ 0.54 MHz/12/SCS | ≥ 12.96 MHz/12/SCS | ≤ 2.16 MHz/12/SCS | ≤ 2.16 MHz/12/SCS |  |  |
| 20 NOTE 3 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ 5.76 MHz/12/SCS |
| 20 NOTE 4 | FC < 1945.7 | ≥0 | ≥ 1.1*RBstart + 4.14 MHz/12/SCS | ≤ 1.80 MHz/12/SCS | ≤ 5.76 MHz/12/SCS and < 1.1*RBstart + 4.14MHz/12/SCS | > 1.80 MHz/12/SCS and ≤ 5.04 MHz/12/SCS | ≤ [0.9] MHz/12/SCS |
| NOTE 1: The A-MPR values are specified in Table TBD NOTE 2: The sub-carrier spacing SCS expressed in MHz NOTE 3: Not applicable for DFT-s-OFDM QPSK for FC < 1945.7 MHz NOTE 4: Only applicable for DFT-s-OFDM QPSK (for accommodating PUCCH transmissions with 0.5dB A-MPR with a reduced PRB offset from the carrier bandwidth edge) |  |  |  |  |  |  |  |

Table 3: Reworking A-MPR for NS_05

| CBW = 20 MHz, DFT-s-ORFM QPSK Fc < 1945.7 MHz | Region A1: A-MPR | Region A2: A-MPR | Region A3: A-MPR |
| --- | --- | --- | --- |
| IQ Image= -28 dB and Carrier Leakage = -28dBc | 4 | 4 | 2 |
| IQ Image = -34 dB and Carrier Leakage = -34dBc | 4 | 3 | 0.5 |

As suspected, the approved Region A2 is sub-optimal, both with legacy assumptions, and current performing assumptions. One thing is clear and that is as a starting point to split the Region A2 in two Regions, where we call the new regions Region A2 and Region A3. Although we had hopped of avoiding A-MPR common PUCCH completely, it is possible to reduce the required PRB offset to 10 PRB by using assumptions for MPR simulations that are more representative of current performance. We propose

Proposal 3: For accommodating PUCCH transmissions with 0.5dB A-MPR with a reduced PRB offset from the carrier bandwidth edge, we propose to improve the simulation assumptions when doing PUCCH transmissions in DFT-s-OFDM QPSK case.

This means that the PRB offset could be reduced to 10 PRB as compared to the 28 PRB implied by the agreed WF [1]. We assume that the exceptional assumptions would only be used for smaller allocations for with the MPR is reduced by tighter IQ image rejection and reduced carrier leakage.

An exception for PUCCH could be created by modifying the A3 region for the 20 MHz channel bandwidth as shown in the table 2 above, the ‘smaller allocation’ exempted is a tentative 5 PRB to cover common PUCCH transmissions as well as smaller PUSCH QPSK transmissions for centre frequencies below 1945.7 MHz:

# Optionality in earlier releases

It has been agreed to introduce the updated emission limits and associated A-MPR from Rel-19 [1]. The modified MPR behaviour can be used for allowing UEs of earlier releases indicate support of the reduced A-MPR and make the network aware of this, at least for UEs compliant with Rel-16 and later.

Rel-19 UEs must be compliant with the Rel-19 version of the 38.101-1 and since this is the open release, the bit shall be set:

whereas UEs compliant with “this version of the specification” earlier than Rel-19, may set the bit to indicate support of the lower A-MPR

in accordance with the original intention of the modified MPR behaviour to maintain backwards compatibility. This means that the bitmap for n1 would always be included by UEs compliant with Rel-19 or later.

Proposal 4: for indication of modified MPR behaviour for NS_05/NS_05U, UEs compliant with Rel-19 or later shall set the relevant bit to “1” and thus always include the bitmap for band n1 such that backwards compatibility for modified MPR indication is maintained.

A version of the specification should not specify/define behaviour of a UE compliant with a version of another release. The modified MPR behaviour, or the release independence for that matter, should not do this either.

# References

1.    R4-2420481, “WF on PHS protection update”, KDDI, OPPO

2.    R4-2500231, “(NR_newRAT-Core) Further study on NR A-MPR reevaluation scenarios on n1 for Japan”, KDDI