3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504583

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.10.3.2

Source:	Ericsson

Title:	On the UE RF requirements for less than 5 MHz CBW for NTN

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In RAN4#114 meeting, the WF has been approved in [2]. In this contribution, the only UE RF requirement that we focus on is the reference sensitivity requirement.

# Discussion

On the REFSENS requirements topic, the following was captured in the approved WF [2]:

Issue 2-2-1: Reference sensitivity

- Agreement

  - For band n254, [2.2] dB improvement compared with 5 MHz CBW;

  - For band n256, [1.7]dB improvement compared with 5 MHz CBW;

  - FFS for band n255

For the reference sensitivity requirement (REFSENS), 3 MHz CBW shall be added to Table 7.3.2-1 in TS38.101-5. For narrower channel bandwidths, such as 3MHz, the impact of IIP2 (i.e. self-mixing and effect of the 2nd order non-linearity) is more pronounced: narrower the channel bandwidth, larger the REFSENS degradation due to Tx OOB emission at the Rx frequency, and better IP2 performance (receiver linearity) is needed. Duplexing distance/band gap is an important factor for Tx-Rx isolation performance and for determining the second-order response. Every extra 1dB duplexer isolation corresponds to 2dB relaxation in the IIP2 requirement.

Due to a very large duplexing distance for band n254, it is quite safe to say that 2.2dB REFSENS improvement is feasible. Also, for bands n256 and n255, due to a fairly large duplexing distance 2.2dB REFSENS improvement seems to be feasible under the assumption of RXIIP2 = 54dBm.

Regarding the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

Proposal 1: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Specify 2.2dB improvement compared with 5 MHz CBW for all bands;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

# Conclusion

Based on the discussion in the previous section we have made the following proposals:

Proposal 1: For the REFSENS requirements, 3 MHz CBW shall be added to Table 7.3.2-1:

- Specify 2.2dB improvement compared with 5 MHz CBW for all bands;

- For the uplink configuration for REFSENS, 3 MHz should be added to Table 7.3.2-2, where 15RBs could be adopted.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2502282, “WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#114.