3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504584

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.10.2.2.1

Source:	Ericsson

Title:	On the UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for NR-NTN HPUE from the approved WF in RAN4#114 [2].

# Discussion

The following issues have remained for further study:

Issue 2-3: NR-NTN PC1

Agreement:

- RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19.

For TN, 4Tx support has been specified for non-handheld UEs in Rel-18 for up to PC1.5. The main reason for such a limitation in power class was the default PA configuration assumption of 4 x 23 dBm. The support for PC1 would require the support of 4 x 26 dBm PA configuration, where at least for TN it is assumed that it will not be widely available in the commercial UEs in Rel-19. For that reason, the proposals from the companies for the support of PC1 with 4Tx have been made in the discussion on Rel-20 scope. In our view, it would make the most sense to discuss both TN and NTN 4Tx support for PC1 in Rel-20.

Proposal 1: Postpone the introduction of 4Tx support for PC1 to Rel-20, which is aligned with TN.

Issue 2-4-1: PC2 ACLR

Agreement:

- PC2 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-2: PC1.5 ACLR

Agreement:

- PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-3: PC1 ACLR

Agreement:

- PC1 ACLR is [31 or 37] dBc for NR-NTN UE

- Further study averaging value, i.e. 34.

For TN, the NR ACLR requirements have been specified as follows:

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

These requirements have been derived having in mind the co-existence between the different UEs, ensuring that not too much power is leaked to adjacent channels, with the goal to primarily protect the DL reception of other UEs. No isolation distance has been assumed between the UEs when these requirements have been derived.

If these requirements are not met, the minimum performance in the network cannot be guaranteed.

In this WI, it was agreed to specify requirements assuming 0 km isolation distance as well. Since the definitions of power classes are the same between NTN and TN, the same ACLR requirements shall be specified for NTN as well, which was also confirmed by our co-existence simulation results [3]. Relaxing ACLR requirements should not be used as a “tool” to potentially decrease the MPR requirements, as their purpose is to ensure the co-existence both between NTN UEs and between NTN and TN UEs. The consequences of relaxing the ACLR requirements would be detrimental to the TN performance (and unpredictable).

Proposal 2: Specify the same ACLR requirements as for TN networks: 37 dB for PC1, 31 dB for PC1.5 and 31 dB for PC2.

Issue 2-6: MPR/A-MPR

Agreement:

- For PC 2 and PC 1.5 MPR:

- Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc.

- Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc.

- For PC 1 MPR:

- Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc:

  - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies

  - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec.

- Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc

- Aim for MPR/A-MPR analysis in #114bis for different power classes

- For PA model calibration:

  - DFT-s-OFDM QPSK 20 MHz

  - 100RB0

  - 4dB post PA loss

  - 1dB MPR

  - Carrier leakage 28dBc

  - IQ image:28 dBc

  - CIM3: 60 dBc

  - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB

  - SEM: existing PC3 SEM

  - EVM: 17.5%

If both the ACLR and the SEM requirements are reused from the TN, the same MPR requirements should be specified as well.

Proposal 3: Specify the same MPR requirements as for the TN for each of PC1, PC1.5 and PC2.

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: Postpone the introduction of 4Tx support for PC1 to Rel-20, which is aligned with TN.

Proposal 2: Specify the same ACLR requirements as for TN networks: 37 dB for PC1, 31 dB for PC1.5 and 31 dB for PC2.

Proposal 3: Specify the same MPR requirements as for the TN for each of PC1, PC1.5 and PC2.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2502954, “WF on HPUE requirments for NR-NTN”, Samsung, RAN4#114.

- R4-2501401, “On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE”, Ericsson, RAN4#114.