3GPP TSG-RAN WG4 Meeting #114bis	R4-2504598

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.26.4.1

Source:	MediaTek Inc.

Title:	Simulation assumptions for R19 LP-WUS

Document for:	Discussion

# Introduction

In the last meeting, RAN4 discussed the simulation assumptions for LP-WUS/WUR in FR1, and some agreements were achieved as captured in the WF [1]. In this contribution, we further discuss the remaining aspects in simulation assumption for RRM performance evaluation for LP-WUS/WUR UE, including the general parameters, cell specific parameters and UE specific parameters. In addition, we provide some initial simulation results based on the simulation assumption provided.

# Simulation assumptions and results

One of the issues discussed in RAN4 was the definition of the target SNR for LP-WUR RRM measurements as captured in the WF [1].

Issue 2-1-1: SINR setting Agreement: Include -6 dB SINR in simulation assumptions SINR and -6 dB SINR is low priority and depending on company input; Treat -0.5dB and 2dB SINR as low priority cases; Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation Agreement: Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR Issue 2-1-6: On SCS in simulation Agreement: Using SCS = 30KHz as the base in the simulation Issue 2-1-7: On OOK interference Agreement: Define a specific LP-SS binary sequences of the serving and interference cell for simulation alignment. Issue 2-1-4: On LP-SS sequence for simulation purpose Agreement: For RAN4 simulation alignment purpose: For M=1, L=4 (if supported), the set of LP-SS binary sequences is: Serving cell: [0 1 0 1] Interference cell: [0 1 1 0] For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1] For M=1, L=8 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 0 1 0 1] Interference cell: [1 0 1 0 1 0 0 1] For M=2, L=8 (if supported), the set of LP-SS sequence is: Serving cell: [0 1 0 1 1 0 0 1] Interference cell: [0 1 1 0 0 1 0 1] For M=2, L=12 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 0 1 1 0 0 1 1 0 0 1] Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1] For M=2, L=16 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1] Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1] For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between: Serving cell: [0110100110101010] Interference cell: [0110101010011010] For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: Serving cell: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] ### ub Issue 2-2-1: On simulation campaign Agreement: delta RSRP and delta RSRQ (the delta RSRP is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point) For simulation alignment purpose, interested companies can also report the “mean” value. For the number of samples: It is up to companies whether to provide result for all or some of the numbers of samples.

For the RRM simulation purpose only, it was agreed to use -3dB, 0.5dB, 2dB dB Ês/Iot value for serving cell measurement for both OOK based and OFDM based LP-WUR. These values were selected based on the previous agreement made in RAN1 (RAN1#117). In the last RAN4 meeting, -6dB was also included as side condition requirements to align with RAN1 agreements. Considering the worst-case scenario, we provide our results based on the two values of SNR (-3dB and -6dB) for RRM measurement accuracy as shown in Table 1.

Proposal 1: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |  |  |
| M=1, L=6 | [2] | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | [2] | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | [2] | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | [2] | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | [2] | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Set 2 from [3] | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

The simulation results are demonstrated in terms CDF percentage for the measured RSRP accuracy for LP-SS, including 5%, 50%, 95% as well as the Absolute accuracy. Note that Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point following the agreement in Issue 2-2-1 from the WF [1].  The simulation results above demonstrate the RSRP error using different number of samples in TDL-C channel. Regarding the LP-SS binary sequence, from the table above, it is based on the corresponding LPSS sequences agreed in [2]. However, for M=4, L=32, we provide the results based on the LP-SS sequence set 2 discussed in RAN1 [3] as captured below:

Agreement For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between: - Set 1: [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] - Set 2 [0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0] [0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0] [0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0] [0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

It can be noticed that even with 3 sample measurement LP-WUS capable UE can achieve satisfactory results for the given simulation assumption in Table 2, 3 and 4. However, to select number of samples for RRM performance evaluation for LP-WUS capable UE, we propose to use at least 3 or 5 samples to guarantee achieving the required performance.

Proposal 2: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Furthermore, the overall simulation assumption list for RRM performance evaluation for LP-WUS/WUR UE is provided below including the general parameters, cell specific parameters and UE specific parameters.

Table 2: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | 3/5 |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 3: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-4 with M = 1,2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 |
| LP-SS binary sequence |  | Indicated in accuracy table | Indicated in accuracy table |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK based receiver 10 ppm | N/A |
| Timing error |  | T= 5us for OOK-4 with M=1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
| Ês/Iot | dB | -3dB / -6dB | N/A |
| Propagation conditions | - | FR1: TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 4: UE-specific parameters

| [Receiver Filter] | 3rd Order Butterworth with 3.84MHz bandwidth |
| --- | --- |
| [Receiver ADC bit width] | 4-bit ADC |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |

In addition, for the simulation results obtained for OOK-based LP-WUR, it is our understanding that the ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only (even though the transmitted LP-SS is an I/Q signal). Therefore, RAN4 needs to align on how the RSRP measurement results are obtained for OOK-based LP-WUR, otherwise results in RAN4 might not be comparable.

Proposal 3: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: RSRP error (dB) in TDL-C channel for OOK shown in Table 1.

Table 1: RSRP error (dB) in TDL-C channel for OOK

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 samples |  |  |  | 5 samples |  |  |  |  |  |
|  |  | 5% | 50% | 95% | Absolute accuracy | 5% | 50% | 95% | Absolute accuracy |  |  |
| M=1, L=6 | [2] | -2.32 | -0.37 | 0.94 | 2.32 | -1.87 | -0.38 | 0.67 | 1.87 | -3 | Note: Absolute accuracy is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point |
|  |  | -1.9 | 0.54 | 2.28 | 2.28 | -1.22 | 0.54 | 1.93 | 1.93 | -6 |  |
| M=2, L=8 | [2] | -1.54 | 0 | 1.41 | 1.54 | -1.1 | 0.02 | 1.05 | 1.1 | -3 |  |
|  |  | -1.48 | 0.43 | 2.1 | 2.1 | -0.97 | 0.43 | 1.73 | 1.73 | -6 |  |
| M=2, L=16 | [2] | -1.29 | 0.13 | 1.38 | 1.38 | -0.93 | 0.1 | 1.09 | 1.09 | -3 |  |
|  |  | -1.05 | 0.54 | 2.05 | 2.05 | -0.68 | 0.52 | 1.67 | 1.67 | -6 |  |
| M=4, L=16 | [2] | -1.48 | 0.33 | 2.05 | 2.05 | -0.82 | 0.34 | 1.41 | 1.41 | -3 |  |
|  |  | -1.37 | 0.72 | 2.61 | 2.61 | -0.66 | 0.73 | 2.04 | 2.04 | -6 |  |
| M=4, L=32 | [2] | -1.29 | 0.05 | 1.28 | 1.29 | -1.11 | 0.02 | 1.19 | 1.19 | -3 |  |
|  |  | -1.01 | 0.49 | 2.01 | 2.01 | -0.82 | 0.45 | 1.76 | 1.76 | -6 |  |
| M=4, L=32 | Set 2 from [3] | -1.1 | 0.02 | 1.12 | 1.1 | -1.03 | -0.01 | 1.08 | 1.08 | -3 |  |
|  |  | -1.07 | 0.23 | 1.56 | 1.56 | -0.91 | 0.19 | 1.43 | 1.43 | -6 |  |

Proposal 2: For OOK-based LP-WUR, use 3 or 5 samples for RSRP measurements.

Proposal 3: The ideal RSRP and the measured RSRP are derived based on the assumption that the OOK-based LP-WUR is equipped with I-branch only, and RAN4 needs to align the understanding for the obtained results.

# References

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo.

- R4-2502600, WF on RRM requirements for NR_LPWUS

- R1-2501583, Summary #5 of discussions on LP-WUS and LP-SS design, vivo