3GPP TSG-RAN WG4 Meeting # 114bis	R4-2504602

Wuhan, CN, Apr 07 – Apr 11, 2025

Agenda Item: 7.22.3.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated/event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not precluded Issue 1-3: Same or different periodicities of current beam and new beam(s)? <Way forward > Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo) - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities. - According to RAN1 LS (R1-2410914) Option 2: (Huawei) - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1 Option 3: (vivo) - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements. RAN1 LS [3] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

In legacy, the number of samples (M) is decided by whether RAN1 would have time domain restriction for channel measurements. For example, M=1 if higher layer parameter of time domain restriction (timeRestrictionForChannelMeasurement) is configured, and M=3 otherwise. In RAN4 we can reuse same values as legacy, it can be revisited if RAN1 make a different agreement otherwise.

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. This number can be revisited if RAN1 make a different agreement otherwise.

Until the latest RAN1 progress related to RAN1 LS [3], there’s no RAN1 conclusion to support different periodicity of current and new beam. Therefore, RAN4 does not need to discuss something has not been supported in RAN1. Even if RAN1 later will support different periodicities, in our understanding, we could consider the maximum period of current beam and new beam. For example, if the SSB period of current beam and new beam is different, we could define the maximum period as Max{TSSB_current, TSSB_new} for TL1-RSRP_event triggered_measurement_Period_SSB.

Proposal 2: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not.

- If RAN1 agree to support the different periodicities of current and new beam, consider the maximum periodicity:

  - Max {TSSB_current, TSSB_new} for TL1-RSRP_event triggered_measurement_Period_SSB

  - Max {TCSI-RS_current, TCSI-RS_new} for TL1-RSRP_event triggered_measurement_Period_CSI-RS

Issue 1-4: Components of Event triggered measurement reporting delay? <Way forward > Option 1: (Apple, vivo, Samsung, E///, MTK) - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel - FFS on whether other components are needed. - FFS the definition of Tfirst UL channel Option 2: (CMCC, Xiaomi) - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. Option 3: (Nokia) - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period). Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

We also share our view on the evaluation period for both cases supported by RAN1, “no time window” and “with time window with configurable M (event instances threshold)”.

- In the case of “no time window”, this is the basic feature (M instance = 1) in our understanding since the M event instances (M > 1) is only applicable under configurable time window, subject to UE capability defined in RAN1.

- In the case of “time window with configurable M”, UE initiate beam reporting only if the number of instances for event 2 (i.e., the quality of new beam becomes a threshold value better than current beam) is larger than or equal to event instances threshold. Also, the evaluation should be done within time window. Otherwise, UE should re-evaluate whether the event condition is met since the time window is changed.

For the case of no time window, RAN4 can define the requirements for reporting delay based on the L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS as agreed in the previous meeting in Issue 1-2. However, whether to consider the uncertainty time to send beam reporting notification on 1st PUCCH to be part of event triggered reporting delay, we can either follow legacy L3 event triggered reporting principle (i.e., to exclude this uncertainty time) or to include the uncertainty time for UL reporting.

For the case of when time window is configured, during the configured time window there might be N number of instances to check for event 2, where N can be ≥ M counter. Based on RAN1 design, UE initiate beam reporting only if at least M number of instances satisfy event 2. Therefore, if we want to define the requirements for this case, RAN4 need to consider the worst-case scenario. For example, when only M out of N instances meet event 2 condition, for which these M instances could occur anywhere within the configured time window as shown the figure below. There can be two ways to define the requirements for this case. One way is that the reporting delay with configured time window can reuse the same principle of Time To Trigger (TTT) in the legacy requirements. Another way is that the reporting delay can be defined based on the multiple (M) times of L1-RSRP measurement period within the time window.

Figure 1: the case of time window with configurable M

Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 4: For the case of no time window:

- Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period + Tfirst UL channel where,

- TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

- Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 5: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 6: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

- Option 2: Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: event instances threshold is configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM <Way forward > Option 1: (Qualcomm, Nokia, MediaTek(descoped)) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. Option 2: (Apple, Huawei, Ericsson) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement. Option 3: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Option 4: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

Since there’s no L1-RSRP measurement for CSI-RS on inter-cell in legacy. To save time, we prefer to postpone the discussion until the enhancement of existing requirements are finished.

Proposal 7: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

RAN1 Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded Issue 1-11: Whether to specify new unified TCI switching delay requirements? <Way forward > Option 1: (Qualcomm, Apple, China Telecom, Samsung (For known conditions for TCI states), Ericsson, MediaTek) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. Option 2: (Xiaomi, Nokia) - Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress. Option 3: (LGE) - For unified TCI switching delay requirements, RAN4 needs to discuss whether active TCI state switching delay can be reduced or not Option 4: (ZTE) - For MAC-CE based DL TCI switching delay: no need to consider TL1-RSRP and Tfirst-SSB+TSSB-proc Option 5: (Samsung) - Not consider unknown case.

In our understanding, after UE send L1 event beam reporting to NW, it can be expected that there will be TCI state switching command to target event beam from NW. Follow the legacy known condition of TCI state, if NW send TCI state switching command within 1280ms upon the last transmission of the RS resource for beam reporting or measurement. Then UE will treat this target TCI state as known TCI state. In known TCI state, there’s discussion to reduce additional SSB reception from RAN1 agreement. Since no RAN1 conclusion for now, we could wait for RAN1 further agreement. If RAN1 agree to reduce additional SSB reception for TCI state of UE-initiated/event-driven beam, then RAN4 could consider skipping the fine timing sync (additional SSB reception) for known TCI state. Otherwise, reuse the existing unified TCI state delay requirement.

Proposal 8: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

## 2.2 Capability on UE-initiated/event-driven beam management

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > Option 1: (Apple, Ericsson) - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added. Option 2: (China Telecom) - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI. Option 3: (CMCC, MediaTek) - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4). Issue 1-15: Compatibility of capabilities with LTM <Way forward > Option 1: (Apple, Ericsson) - Discuss it later. Option 2: (CMCC, Huawei, Nokia, Samsung) - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately. Option 3: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. TS 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NGRAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequencyvSS-RSRP, SS-RSRQ, SS-SINR, CSIRSRP, CSI-RSRQ, and CSI-SINR for NG-RANvinter-frequency cells. …; …; …. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.

In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in TS 38.133 section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO.

Proposal 9: Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

Since we’re targeting FR2 and sTRP with intra- and inter-cell beam management, the number of events for UE-initiated beam reporting should be specified for intra-frequency.

We prefer to specify a unified capability that indicates the total number of L1 events supported for mobility and MIMO. This allows UE with more flexibility to utilize the whole capability of L1 events compared to share the total number of L1 events to separate UE capability.

Proposal 10: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. This number can be revisited if RAN1 make a different agreement otherwise.

Proposal 2: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not.

- If RAN1 agree to support the different periodicities of current and new beam, consider the maximum periodicity:

  - Max {TSSB_current, TSSB_new} for TL1-RSRP_event triggered_measurement_Period_SSB

  - Max {TCSI-RS_current, TCSI-RS_new} for TL1-RSRP_event triggered_measurement_Period_CSI-RS

Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 4: For the case of no time window:

- Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period + Tfirst UL channel where,

- TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

- Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 5: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 6: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

- Option 2: Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: event instances threshold is configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 7: Reuse legacy measurement period for event 2 on UE-initiated/event-driven beam management.

- SSB based L1-RSRP measurement for both intra-cell and inter-cell.

- CSI-RS based L1-RSRP measurement for intra-cell.

  - Descope CSI-RS based L1-RSRP measurement for Inter-cell case.

Proposal 8: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

Proposal 9: Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

Proposal 10: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1 (Samsung)

[3] R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management