3GPP TSG-RAN WG4 Meeting #114bis							         		R4-2504604

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Initial power boosting result for NB-IoT-NTN for a PC2 NR PA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.10.2.3.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In R4#114 we proposed the IoT NTN HPUE requirement for smartphone to be common for implementations with a GSM PA or a PC2 1Tx NR PA in [1]. A way forward [2] captured a number of agreements on NB-IoT NTN and eMTC-NTN HPUE work that are the basis of the present work. In this contribution, we provide preliminary measurement results on a PC2 1Tx PA. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boosting and more power should be achievable.

# Discussion

## 2.1 Way forward input

The way forwards [2] captured an agreement on IoT-NTN HPEU topics:

Issue 1-1-1: ACLR for NB-Iot based Iot-NTN for PC1/2;

Agreement: Re-use current PC3 ACLR (UTRA_ACLR = 37dBc) for PC1/2.

Issue 1-1-3: SEM for NB-Iot based NTN for PC1/2

Agreement: Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN

Due to the lack of time, we only measured for UTRA ACLR and recognize that further limitations may occur with the NB-IoT SEM or in-band requirements.

Issue 1-2-3: The reference components need to be considered.

Agreement: The reference components need to be considered.

- For NB-Iot based Iot-NTN, both GSM PA and NR 1Tx PC2 PA are taken into consideration in developing unified for one power class;

- For eMTC-based IoT-NTN, HPUE for PC2 and NR 1Tx PC2 PA

The measurements were performed on a PC2 1Tx Mid-band PA calibrated for 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boosting and more power should be achievable.

## 2.2 Measurement Results

The measurement results are for a limited number of NB-IoT waveforms:

- Pi/2 BPSK waveforms with 1SCS at the edge or in the middle of the channel.

- QPSK waveforms with 1SCS at the edge or in the middle of the channel.

- 1RB QPSK waveform.

- Transceiver impairments were set at the nominal NR 28dB and the 34dB 256QAM image and carrier leakage level.

- The power levels are at the PA output and 2-3dB post PA losses assuming HD-FDD will need to be discussed further to have a common set of evaluation assumptions.

- The PA is designed for PC2 1Tx NR power with up to 4dB post PA losses (30dBm 0dB MPR level at the PA output)

Table 1 provides the measurements of the five waveforms per output power, current increase vs MPR0 and margins versus UTRA ACLR for 28dB and 34dB levels of impairments.

Table 1: UTRA ACLR margin versus PA output power for various NB-IoT waveforms

|  | 28dB impairment |  |  |  | 34dB impairment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pout | relative Current | UTRA ACLR margin |  | Pout | relative Current | UTRA ACLR margin |  |
| modulation | dBm |  | low | high | dBm |  | low | high |
| 1SCS middle Pi/2 BPSK | 30.0 | 100% | 31.8 | 31.8 | 30.0 | 100% | 32.0 | 32.0 |
|  | 31.0 | 114% | 32.3 | 32.3 | 31.0 | 116% | 32.9 | 32.9 |
|  | 31.9 | 140% | 26.0 | 26.0 | 31.4 | 132% | 27.1 | 27.1 |
| 1SCS edge Pi/2 BPSK | 30.0 | 100% | 29.0 | 29.0 | 30.0 | 100% | 28.8 | 28.8 |
|  | 31.0 | 114% | 29.5 | 29.5 | 31.0 | 117% | 29.7 | 29.7 |
|  | 31.9 | 141% | 23.3 | 23.3 | 31.4 | 131% | 23.1 | 23.1 |
| 1SCS middle QPSK | 30.0 | 100% | 27.7 | 27.7 | 30.0 | 100% | 26.6 | 26.6 |
|  | 31.0 | 115% | 27.2 | 27.2 | 31.0 | 119% | 26.7 | 26.7 |
|  | 31.6 | 135% | 20.9 | 20.9 | 31.2 | 131% | 21.1 | 21.1 |
| 1SCS edge QPSK | 30.0 | 100% | 25.6 | 25.6 | 30.0 | 100% | 25.0 | 25.0 |
|  | 31.0 | 114% | 25.7 | 25.7 | 31.0 | 118% | 25.4 | 25.4 |
|  | 31.6 | 136% | 18.4 | 18.4 | 31.4 | 131% | 18.7 | 18.7 |
| 1RB QPSK | 30.0 | 100% | 17.4 | 17.4 |  |  |  |  |
|  | 31.0 | 117% | 14.6 | 14.6 |  |  |  |  |
|  | 31.6 | 136% | 7.3 | 7.3 |  |  |  |  |

Observations:

- All waveforms show significant margin to UTRA ACLR close to 32dBm, with the 1RB cases still having some margin.

- There is slightly higher margin for 1SCS Pi/2 BPSK versus QPSK, 1RB QPSK showing the lowest margin.

- There is slightly higher margin for 1SCS in the middle versus the edge.

- The 34dB image and carrier leakage case does not show significant benefits and 28dB seem a good starting point.

- The power was limited due to external current limitation on the PA and it should be feasible to reach higher power once those limitations are controlled.

- Provided that SEM or in-band requirement do not create further limitations and assuming close to 3dB post PA losses for HD-FDD vs NR FDD case, PC1.5 power (29dBm) 0dB MPR might be reachable.

Proposal: To start a full power boosting evaluation on a PC2 1Tx PA to achieve PC1.5 power levels and possibly higher for NB-IoT-NTN, it is important that we agree on key parameters:

- PA calibration level: for PC2 1Tx NR PA with propose 29dBm 1dB MPR for 31dB NR ACLR with a 20MHz QPSK DFT-s-OFDM 100RB0 waveform. Which correspond to 4dB post PA loss in a FD-FDD environment.

- Image and carrier leakage: we propose to use the NR agreed level at 28dB.

- HD-FDD post PA losses: we propose to evaluate 2.5dB and 3dB.

# Conclusion

In this contribution, we share a preliminary set of NB-IoT-NTN HPUE results and make the following proposals to start a complete power boosting evaluation campaign based on a PC2 1Tx NR PA.

Proposal: To start a full power boosting evaluation on a PC2 1Tx PA to achieve PC1.5 power levels and possibly higher for NB-IoT-NTN, it is important that we agree on key parameters:

- PA calibration level: for PC2 1Tx NR PA with propose 29dBm 1dB MPR for 31dB NR ACLR with a 20MHz QPSK DFT-s-OFDM 100RB0 waveform. Which correspond to 4dB post PA loss in a FD-FDD environment.

- Image and carrier leakage: we propose to use the NR agreed level at 28dB.

- HD-FDD post PA losses: we propose to evaluate 2.5dB and 3dB.

# References

[1] R4-2500108 Candidate TN PA for IoT-NTN HPUE in smartphones, Skyworks Solutions Inc., R4#114

[2] R4-2503006 WF on HPUE UE requirement for IoT-NTN, vivo, R4#114