3GPP TSG-RAN WG4 Meeting #114bis	R4-2504607

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.3.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for SBFD operation

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective [1] is to specify the sub-band full duplex operation. RAN4 has reached a few agreements on RRM [2,3,4,5] in the last few meetings. In this paper, we further discuss some open issues related to CSI-RS based measurements with SBFD operation.

# CSI-RS based L1 measurements

- Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case

  - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

  - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

  - Option 3: Consider the UE measurement capability

- FFS whether and how to account for the UE capability on CSI processing time.

- Further discuss the accuracy requirements for the following cases

  - Case 1: X1 >= X3 and X2 >= X3

  - Case 2: X1 >= X3 and X2 < X3

  - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

RAN4 made several agreements related to CSI-RS based L1 measurements in the last meeting where one of the main issue discussed was related to processing of punctured CSI-RS in frequency domain in SBFD symbols.

The main FFS issues in the above agreements are related to DUD configuration when the CSI-RS PRBs are spread across the two DL sub-bands. Basically, how to specify the measurement accuracy and measurement delay requirements in such cases.

We think, like DU configuration, even DUD configuration should ensure that atleast the minimum number of CSI-RS RBs are contiguous. In other words, to reuse the existing measurement delay and accuracy requirements, at-least one of the DL sub-band should have consecutive CSI-RS PRBs which is not less than the existing threshold, else the UE cannot meet the existing measurement delay and accuracy requirements.

Proposal 1: In DUD configuration, existing delay and accuracy requirements for CSI-RS based measurements apply only if the number of consecutive CSI-RS PRBs in atleast one of the DL subband is no less than the existing thresholds, i.e., 24 RBs for RLM/BFD/CBD and 48 RBs for L1-RSRP/L1-SINR.

The next issue is on CSI-RS resource configuration for RLM/BFD measurements. Based on the RAN1 reply LS [6], it is clear the dynamically scheduled UL transmission in the SBFD symbols is prioritized over periodic CSI-RS resources configured for RLM/BFD measurements. This would lead to UE missing some CSI-RS measurements for RLM/BFD. In order to accommodate such missed CSI-RS measurements, we think the CSI-RS based measurements and evaluation periods should be extended by the number of missed CSI-RS occasions due to collisions with the dynamically scheduled UL.

Proposal 2: Extend the CSI-RS based measurement and evaluation period such as RLM OOS/IS, BFD, CBD, L1-RSRP, L1-SINR etc., by the number of missed CSI-RS occasions due to collisions with the dynamically scheduled UL.

# Conclusion

Proposal 1: In DUD configuration, existing delay and accuracy requirements for CSI-RS based measurements apply only if the number of consecutive CSI-RS PRBs in atleast one of the DL subband is no less than the existing thresholds, i.e., 24RBs for RLM/BFD/CBD and 48RBs for L1-RSRP/L1-SINR.

Proposal 2: Extend the CSI-RS based measurement and evaluation period such as RLM OOS/IS, BFD, CBD, L1-RSRP, L1-SINR etc., by the number of missed CSI-RS occasions due to collisions with the dynamically scheduled UL.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis

[4] R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#113

[5] R4-2502699, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#114

[6] R1-2501560, “Reply LS on CSI-RS measurement with SBFD operation”, RAN1#120