3GPP TSG-RAN WG4 Meeting #114bis	R4-2504608

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.3.1

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements. RAN4 has already reached quite a few agreements [2,3,4,5] in the last meetings. In this paper, we further discuss various issues related to L1-CLI requirements.

# L1 based UE-to-UE CLI measurement requirements

As we know, RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new measurement report quantities and has finalized upon method #1, #2 and#3 to be used to perform CLI measurements.

Scheduling Restrictions

RAN4 has already started working on defining the requirements for Method#1 and #2 and in the last meeting, it was also agreed to define requirements for Method#3. We think the delay and accuracy requirements for Method#3 could be same as Method#1, however, the scheduling restrictions may be different. Same as Method#1 and2, since the UE performs measurements in the receive mode, scheduling restrictions will apply on the UL, i.e., the UE is not expected to transmit on UL during the RSSI measurement symbols. For DL, the scheduling restrictions do not apply to the RSSI measurement symbols if UE is able to support FDM-ed DL reception and RSSI measurement if RAN1 will define optional UE capability for FDMed DL reception and RSSI measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restrictions apply to the RSSI measurement symbols.

Proposal 1: For Method#3, UL scheduling restriction always apply, and DL scheduling restrictions apply if the UE does not support the FDMed DL reception and RSSI measurement capability.

Proposal 2: For Method#3, the scheduling restrictions apply only on the RSSI symbols to be measured.

Measurement Restrictions

RAN4 made the following agreement on measurement restrictions during the last meeting:

- Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

  - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

While the above agreement addresses the measurement restrictions when the L1 CLI measurement resources overlap with other measurement resources configured for L1 or L3 measurements, the case of overlapping L1 CLI measurement resources is not clear. Based on the recent agreement from RAN1, we may have the following cases of L1 CLI measurement collisions:

- Case 1: Method#1 and Method#2

  - L1-CLI-RSSI resources configured in DL sub-band collide with L1-SRS-RSRP measurement resources configured in UL sub-band

- Case 2: Method#2 and Method#3

  - L1-SRS-RSRP resources configured in UL sub-band collide with L1-CLI-RSSI measurement resources configured in UL sub-band

- Case 3: Method#1 and Method#3  Invalid case based on RAN1 agreement

  - L1-CLI-RSSI resources configured in DL sub-band collide with L1-CLI-RSSI measurement resources configured in UL sub-band

We think Case 1 & 2 are valid cases and may be discussed in RAN4 while Case 3 is invalid based on RAN1 agreement:

A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Observation 1: L1-CLI-RSSI resources (configured according to Method#1 or Method#3) may collide with L1-SRS-RSRP resources configured for L1 CLI measurements

Since it’s already agreed that the L1-CLI-RSSI measurements are performed with DL timing while L1-SRS-RSRP measurements are performed with UL timing, it’s clear that the UE cannot perform these two measurements simultaneously. So, it should be left to UE implementation which measurements the UE performs.

Proposal 3: It’s up-to UE implementation whether to perform L1-CLI-RSSI measurements or L1-SRS-RSRP measurements when the resources configured for the two measurements are colliding in time.

Measurement requirements

For measurement capability, similar to R16 CLI requirements, the L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined within the DL active BWP of the UE, so that the UE is not required to perform any RF retuning for such measurements.

Proposal 4: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Moving on to the side conditions and corresponding measurement delays and accuracy requirements, we think the R16 SRS bandwidth (48PRBs) can be assumed for SBFD as well, but the side conditions on SRS Es/Iot need to be re-evaluated. For instance, R16 requirements were defined to support dynamic TDD where the DL was not protected from the aggressor UL, but for SBFD, the UL transmission takes place in the UL sub-band. So, the actual leakage to the DL sub-bands may be very low due to filtering at the UE side (which was no possible in dynamic TDD). Using SRS SNR of 1db as specified in R16 may show very little leakage in the DL subband and may not cause any degradation. So we think, to have meaningful measurements of the SRS RSRP, the SRS SNR should be considered at a sufficiently high level, e.g., at-least 10db.  .

Proposal 5: Reuse R16 assumptions on SRS bandwidth (i.e., 48 PRBs) for L1-SRS-RSRP measurements.

Proposal 6: RAN4 to consider higher side conditions of at-least 10 dB on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (1 dB).

RAN1 LS on L1 based UE-to-UE CLI measurement and reporting

RAN1 sent an LS to RAN4 [6] asking whether it’s reasonable to use R16 SRS-RSRP and CLI-RSSI measurement reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI. RAN1 also want to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

We think the CLI measurements reporting ranges and granularities specified in R16 is sufficiently large and it’s reasonable to reuse them for L1-CLI measurements.

Proposal 7: Agree with RAN1 to use R16 SRS-RSRP and CLI-RSSI measurement reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI. The range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI is also okay.

# Conclusion

Proposal 1: For Method#3, UL scheduling restriction always apply, and DL scheduling restrictions apply if the UE does not support the FDMed DL reception and RSSI measurement capability.

Proposal 2: For Method#3, the scheduling restrictions apply only on the RSSI symbols to be measured.

Observation 1: L1-CLI-RSSI resources (configured according to Method#1 or Method#3) may collide with L1-SRS-RSRP resources configured for L1 CLI measurements

Proposal 3: It’s up-to UE implementation whether to perform L1-CLI-RSSI measurements or L1-SRS-RSRP measurements when the resources configured for the two measurements are colliding in time.

Proposal 4: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Proposal 5: Reuse R16 assumptions on SRS bandwidth (i.e., 48 PRBs) for L1-SRS-RSRP measurements.

Proposal 6: RAN4 to consider higher side conditions of at-least 10 dB on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (1 dB).

Proposal 7: Agree with RAN1 to use R16 SRS-RSRP and CLI-RSSI measurement reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI. The range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI is also okay.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis

[4] R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#113

[5] R4-2502699, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#114

[6] R1-2501620, “LS on L1 based UE-to-UE CLI measurement and reporting”, RAN1#120