3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504616

Wuhan, China, 7 – 11th April 2025

Source:	BT

Title:	TP for TR 38.753 Section 6.2 MU-MIMO

Agenda item:	7.16.2

Document for:	Endorsement

Introduction

In RAN4#113 work split for TR 38.753 was agreed [1]. In this contribution we provide draft for TP on Section 6.2 MU-MIMO.

Text proposal

--------------------------------------------- Start of text proposal -----------------------------------------------

MU-MIMO

This section provides comparison of spatial channel models for MU-MIMO cases.

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 8 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed | Follow, Random |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |  |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

--------------------------------------------- End of text proposal -----------------------------------------------

References

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.