# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504638

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.11.7

Source: 	Moderator: Intelsat

Title: 	Topic summary for [114bis][313] NR_NTN_Ku_Band_Coexistence

Document for:	Discussion and Decision

# Introduction

This document is a collection of analysis and proposals submitted by various companies on the topic of Ku Band Co-existence analysis.

The compilation provides

- Common parameters and scenarios that are inline between companies

- Open issues that require further discussions and

- Other open issues that have not been resolved.to date

The seven documents in this collection were submitted to RAN4 working Group 114-bis meeting.

# Contributions

## Xiaomi Contribution R4-2503320

Xiaomi in this contribution discusses the need to simplify the simulation cases to just 5 worst cases in oppose to the 8 cases suggested previously.

The previously identified cases are shown in the following Table 1 and the suggested cases are show in Table 2

Table 1 Aggressor and victim of each scenario

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

However, for the coexistence study, not all of the scenarios are the worst case which can determine the ACLR and ACS for NTN system. In order to have a clear view for which scenario can be the worst case, Xiaomi summarized the following Table 2 from existing study results of TR38.863.

Table 2 Xiaomi's suggested 5 worst cases for co-existence simlation

|  | Metrices | ACIR requirement | 2GHz | 17GHz DL | 27GHz UL |
| --- | --- | --- | --- | --- | --- |
| 1 | NTN UE ACLR | NTN UL interfering TN UL | 13 dB |  | 13 dB |
| 2 | NTN SAN ACS | TN UL interfering NTN UL | 28 dB |  | 1.5 dB |
| 3 | NTN UE ACLR | NTN UL interfering TN DL | 26 dB |  | 0 |
| 4 | NTN SAN ACS | TN DL interfering NTN UL | 37 dB |  | 16 dB |
| 5 | NTN UE ACS | TN DL interfering NTN DL | 28 dB | N/A |  |
| 6 | NTN SAN ACLR | NTN DL interfering TN DL | 23 dB | 0 |  |
| 7 | NTN SAN ACLR | NTN DL interfering TN UL |  | 0 |  |
| 8 | NTN UE ACS | TN UL interfering NTN DL |  | 2.5 dB |  |

Xiaomi proposes the following:

Proposal 1: In order to control the workload of coexistence study, RAN4 can evaluate scenarios:

- NTN UL interfering TN UL

- NTN UL interfering TN DL

- NTN DL interfering TN DL

- TN DL interfering NTN DL

- TN DL interfering NTN UL

And for the co-existence simulations at 11 GHz and 14GHz, which were previously agreed, Xiaomi are proposing the following:

Proposal 2: In order to control the workload of coexistence study, RAN4 can ignore the 90 degrees cases for coexistence study. [25 degree is the worst case]

Proposal 3: Suggest RAN4 to evaluate phased array antenna for GEO VSAT UE and circular VSAT UE for LEO for the first step.

Proposal 4: Suggest RAN4 to consider 3.5dB element gain for GEO phased array antenna VSAT UE.

Proposal 5: Considering NTN VSAT noise figure 2.5 dB for GEO VSAT UE and 6 dB for LEO VSAT UE.

## CATT contribution R4-2503454

CATT in their contribution R4-2503454 have proposed 6.9 dBm for the conducted power per antenna element for VSAT phase array antenna. Furthermore, CATT is proposing 39.2 dBm for VSAT maximum receiver gain for a 100 cm parabolic antenna.

### Sub-topic 2-1: VSAT antenna pattern Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel. Agree to use 38.921 unless there is a need to use sub array then 38.803 The following parameters are agreed: Table 2-3 VSAT phased array antenna characteristics Ku band VSAT phased array antenna. 1; Ku band VSAT phased array antenna Characteristics. 1.1; Antenna pattern; Table 2-2 and 2-3 (TR 38.921). 1.2; Element gain (dBi) (Note 2); 0 or 3.5 ?. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization; Circular（RHCP or LHCP）. 1.6; Antenna array configuration (Y axis × Z axis)(Note 4); 45x45 elements. 1.7; Number of supported polarizations, P; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda. 1.9; Array Ohmic loss (dB) (Note 2); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); [4.44 dBm] TBA. 1.11; maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; normal direction; Toward X+ axis. Note 1: Void Note 2: The element gain in row 1.2 includes the loss given in row 1.9. Note 3: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 4: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Agree 3.5 dB element gain

As above, the Conducted power (before Ohmic loss) per antenna element is [4.44 dBm] TBA, since 40dBm (10W) VSAT max Tx power was agreed in WF in RAN4#112bis [2], and a 45 x 45 = 2025 element antenna[1] was agreed for VSAT phased array (flat-panel) antenna. Then, the conducted power (before Ohmic loss) per antenna element (dBm) is 10*log10(10*1000)-10*log10(45*45) = 6.9357dBm. We propose the conducted power (before Ohmic loss) per antenna element (dBm) = 6.94dBm.

Proposal 1: Adopt 6.94dBm for conducted power (before Ohmic loss) per antenna element (dBm) for VSAT phased array antenna.

### Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

As above, the 100 cm aperture (41.3dBi) parabolic antenna was agreed for Ku-band NTN coexistence, the VSAT Max TX antenna gain is 41.3 dBi for 14GHz UL, but the VSAT Max RX antenna gain for 11GHz DL is missing. For parabolic antenna, one 100 cm aperture parabolic antenna can support both 14GHz UL and 11GHz DL at the same time, but the antenna gain for DL and UL is different.

3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

So, using Eq.1, for 14GHz,  2θ-3dB14GHz=1.2631 degree, and for 11GHz, 2θ-3dB11GHz=1.6076 degree.

We can use 41.3 dBi for 14GHz and 1.2631degree 3dB beamwidth as reference, the antenna gain for 11GHz can be calculated by following Eq.2.

G11GHz=G14GHz-20log102θ-3dB11GHz2θ-3dB14GHz=41.3-20*log101.60761.2631=39.2                                            (Eq.2)

So, we propose that VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna is 39.2dBi.

Proposal 2: Adopt 39.2dBi for VSAT Max Rx antenna gain for 100 cm aperture parabolic antenna.

## Ericsson contribution R4-2504263

Ericsson in this contribution discusses the need to focus on the worst-case scenario for the co-existence and to focus on 35-degree elevation angle. Furthermore, Ericsson seeks clarification on the choice of 6 dB Noise Figure for Mobile VSATs connected to the LEO satellites.

On the VSATantenna size, Ericsson is seeking clarification whether 100 cm VSATs are feasible or should a 60 cm VSAT be considered.

The minimum antenna elevation angle for NTN UE should be greater than 35 degrees, based on extracts from ETSI EN 303 980 and ETSI EN 303 981.

With 35 degrees elevation angle, the 5%-tile throughput CDF is stable while the CDF for 25 degrees cannot be concluded as the throughput is almost zero. To avoid large discrepancies between companies results, it would be relevant to assume 35 degrees.

- Proposal 1	RAN4 to consider minimum antenna elevation angle of 35 degrees for NTN UE in the simulation assumptions.

- Proposal 2	RAN4 to check if 6 dB NF is still valid for LEO NTN scenario for Mobile VSAT cases.

Observation 3	A 60 cm VSAT antenna seems to be an ideal assumption for Mobile VSAT applications where portability is crucial.

- Proposal 3	RAN4 to clarify whether the agreed VSAT antenna parameters in previous meetings are applicable for Mobile VSAT and discuss feasibility of 100 cm VSAT antenna size.

RAN4 to treat the co-existence simulation assumptions for Ku-band provided in the Annex [R4-2504263], as a running document

## Ericsson contribution R4-2504264

Ericsson in this contribution have provide preliminary simulation results based on the parameters which have been agreed top date.

For the GEO Case, Ericsson have produced the following results for elevation angles of 90, 25 and 35 degrees see following Table 4:

Table 4 Ericsson's ACIR results for GEO at various elevation angles

| ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |
|  |  |  |  | 90 degrees | 25 degrees | 35 degrees |
|  |  |  |  | Mobile VSAT | Mobile VSAT | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 2.5 dB NF |
| VSAT UL | TN UE UL | 1 | 14 GHz | 1 | 12 |  |
| TN UL | VSAT UL | 2 | 14 GHz | 16 | 22 |  |
| VSAT UL | TN DL | 3 | 14 GHz | 0 | 0 |  |
| TN DL | VSAT UL | 4 | 14 GHz | 6 | 36 | 26 |
| TN DL | SAN DL | 5 | 11 GHz | 36 | 32 | 30 |
| SAN DL | TN DL | 6 | 11 GHz | 0 | 0 | 0 |
| SAN DL | TN UE DL | 7 | 11 GHz | 0 | 0 | 0 |
| TN UL | SAN DL | 8 | 11 GHz | 22 | 16 | 16 |

And similarly, Ericsson have produced ACLR results for the LEO at 600 km at various elevation angles, see following Table 5

Table 5 Ericsson's ACIR results for LEO 600 at various elevation angles

| ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |
|  |  |  |  | 90 degrees | 25 degrees | 35 degrees |
|  |  |  |  | Mobile VSAT | Mobile VSAT | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 2.5 dB NF |
| VSAT UL | TN UE UL | 1 | 14 GHz | 0 | 8 |  |
| TN UL | VSAT UL | 2 | 14 GHz | 16 | 12 |  |
| VSAT UL | TN DL | 3 | 14 GHz | 0 | 0 |  |
| TN DL | VSAT UL | 4 | 14 GHz | 9 | 31 | 22 |
| TN DL | SAN DL | 5 | 11 GHz | 35 | 33 | 30 |
| SAN DL | TN DL | 6 | 11 GHz | 0 | 0 | 0 |
| SAN DL | TN UE DL | 7 | 11 GHz | 0 | 0 | 0 |
| TN UL | SAN DL | 8 | 11 GHz | 26 | 21 | 20 |

Ericsson have provide the following observation, based on their simulations:

Observation 1	In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 10 dB at 35 degrees when compared to 25 degrees elevation angle.

Observation 2	In scenario 4, higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 9 dB at 35 degrees when compared to 25 degrees elevation angle.

Discussion point: what is the gap between the TN and NTN frequency used for the simulations?

## Samsung contribution R4-2504338

Samsung in this contribution suggests simplifying the co-existence scenarios and to focus on selected cases to minimise the analysis time and the workload.

Table 6 Interference scenario for co-existence simulation

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

To simplify the co-existence work Samsung suggests the following simplifications:

Proposal 1: For VSAT ACLR, to prioritize Scenario 1 over Scenario 3 if time is limited

Proposal 2: For VSAT ACS, to prioritize Scenario 8 over Scenario 5 if time is limited.

Proposal 3: For SAN ACS, to prioritize Scenario 4 over Scenario 2, if time is limited

Samsung is also suggesting that for a GEO and NGSO(LEO) and for a 45x45 element antenna, the required gain of36.6 and 37.2 dBI can be reached with antenna element gains of 3.1 And 3.5 dBi:

Proposal 4: For 45x45 antenna array, the Tx antenna gain of 36.6 dBi and 36.2 dBi can be reached with an element gain of 3.1 dBi and 3.5 dBi respectively.

Discussion point: the above calculation to be clarified further

Proposal 5: To clarify the understanding that with 50W for GSO and 10W for NGSO, and antenna gain of 36.6 /36.2 dBi as given in [2], the peak eirp will sums up to 83.6 / 83.4 dBm for GSO and 76.6 / 76.2 dBm for NGSO.

Discussion point: the above calculation to be clarified further

Proposal 6: For 45x45 antenna array, with Tx power of 50W for GSO and 10W for NGSO, the power per element will be 15.9 dBm and 8.9 dBm.

Proposal 7: To clarify the polarization loss between circular and linear polarizations.

## JSAT Contribution R4-2504477

JSAT in their contribution R4-2504477 have proposed a more realistic models for GEO Satellite parameters. This is shown in the following Table 6a.2.2.1-2.

More importantly JSAT have provide a matrix or a table which illustrates various combinations of VSATs and GEO/LEO Satellites that can be used for simulation. This matric table is shown in the following Table 6a.2.2.2-X

On the ACLR and ACS simulations, JSAT are proposing to have a 300 MHz separation  between the Terrestrial and Non-Terrestrial Uplink interferers and 200 MHz separation between Downlink  Terrestrial and Non-Terrestrial interferers

Proposal 1: As shown in the table below, the GEO satellite parameters should be updated to more realistic values in Ku-band, and it is proposed that these be used for the coexistence study.

Table 6a.2.2.1-2: (new) Satellite parameters for co-existence study

| Satellite orbit |  | Set-2 GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 2.5 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 39.0 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 2.5 m | 0.714 m | 0.714 m |
| G/T |  | 12.5 dB K-1 | 11.4 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 40.0 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: Create the table 6a.2.2.2-X of R4-2502315 to clarify the combinations of VSAT models and satellite models to be used for coexistence study. In the following matrix, we identify (via the X check mark) 4 configurations for VSAT models and 3 configurations for SAN that should be included in the simulations.

Original JSAT proposal:

Table 6a.2.2.2-X: VSAT-SAN combination patterns

| VSAT model SAN | 45 x 45 Phased Array | 60 cm Parabolic | 78 x 78 Phased Array | 100 cm Parabolic |
| --- | --- | --- | --- | --- |
| GEO | X | X | X | X |
| LEO 600km | X | X | N/A | N/A |
| LEO 1200km | X | X | N/A | N/A |

Improved JSAT proposal (after combination with THALES proposals from R4-2504658):

Table 6a.2.2.2-X: VSAT-SAN combination patterns

| VSAT model SAN | 45 x 45 Phased Array | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array | 100 cm Parabolic (1st step for coexistence) |
| --- | --- | --- | --- | --- |
| GEO Set-1 (original) | X | X |  |  |
| GEO Set-2 (new) |  |  | X | X |
| LEO 600km | X | X | N/A | N/A |
| LEO 1200km | X (could be descoped) | X (could be descoped) | N/A | N/A |

Observation 1: It has been agreed in the WF that the coexistence study will be conducted using 11 GHz and 14 GHz for both TN and NTN.

Observation 2: It has been agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study.

Observation 3: The only TN frequency allocation subject to coexistence studies with the NTN uplink in the 13.75-14.5 GHz range is 14.8-15.35 GHz, meaning that TN and NTN frequencies are separated by more than 300 MHz. For the uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

Observation 4: The only TN frequency allocation subject to coexistence studies with the NTN downlink in the 10.7-12.75 GHz range is 10.0-10.5 GHz allocated to 12 countries in Region 2, meaning that TN and NTN frequencies are separated by more than 200 MHz.

Observation 5: Since the TN and NTN frequencies are separated by more than 300 MHz in the case of NTN Uplink and more than 200 MHz in the case of NTN downlink, the impact of NTN VSAT and SAN on the TN is expected to be significantly smaller. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Proposal 3: When considering ACS/ACLR for VSAT UE and SAN based on the coexistence test results, RAN4 should consider that the actual frequency assignments of TN and NTN are separated by more than 300 MHz for NTN Uplink and 200 MHz for NTN Downlink. Given this separation, it should be noted that relaxation may be necessary when deriving core requirements from the simulation results.

## Sharp Contribution R4-2504559

At RAN4#113 it was agreed to study a 100cm parabolic VSAT with a max Tx antenna gain of 41.3 dBi for GEO only coexistence testing. Additionally, it was agreed that for VSAT phase array antenna, a 45x45 element array configuration would be used, which is for LEO and GEO coexistence testing. However, the calculated Tx gain for a 45x45 element array is 37.5 dBi (i.e. 36.6dBi =  4.44 + 10*log10(45*45), where 4.44 dBi is per-element antenna gain). To resolve this difference (41.3 vs 37.5 dBi), a VSAT phase array used for GEO only testing should use a different (i.e. larger ) number of elements so as to derive a realistic conducted power per antenna element, and results in approximately 88.2 dBm max EIRP. Therefore, we propose that a new antenna configuration of 78 x 78 elements with the size of 84 cm square (similar to the size of a circle with 100 cm diameter) is used only for GEO. To manage this new 78 x 78 elements configuration, we also propose to split the tables of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 1: A new antenna configuration of 78 x 78 with the size of 84 cm square (similar to the size of a circle with 100 cm diameter) is used only for GEO.

Proposal 2.1: EIRP is set as 74.1 dBm for Option 1 and 73.7 dBm for Option 2 for GEO and LEO.

Proposal 2.2: Conducted power (before Ohmic loss) per antenna element is set to 4.44 dBm for GEO and LEO.

Proposal 3.1: Revise the table 6a.2.2.2-1 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 3.2: Revise the table 6a.2.2.2-1 of R4-2502315 to add Note 5

Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations

| C | VSAT Phased array antenna Characteristics | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 0/ 3.5 | 0/ 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) (NOTE5） | Circular (RHCP or LHCP; as a starting point) (NOTE5） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 78 x 78 elements | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | TBD (dependent on decisions on EIRP) | 4.44 |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Parameters for the phased array antenna should be defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-2: Other NTN UE parameters (Phased array antenna)

|  |  | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| 2 | Characteristics | Phased array antenna | Phased array antenna |
| 2.1 | Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| 2.2 | Antenna type and configuration | Directional Section 6.4.1 of [6] with 100 cm equivalent aperture diameter | Directional Section 6.4.1 of [6] with 60 cm equivalent aperture diameter |
| 2.3 | Polarization | Circular (NOTE2) | Circular (NOTE2) |
| 2.4 | Antenna temperature | 150 K | 150 K |
| 2.5 | Noise figure | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO); Other options not precluded | NTN VSAT Type 4 and 5: 2.5 dB (LEO and GEO) and 6 dB (LEO); Other options not precluded |
| 2.6 | Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO | 50 W for GSO and 10 W (40 dBm) for NGSO |
| 2.7 | Antenna elevation angle | 25˚ and 90˚ | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Proposal 4.1: Revise the table 6a.2.2.2-2 of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

Proposal 4.2: Move Rx Antenna gain, and Tx antenna gain from table 6A.2.2.2-2 of R4-2502315 to 6A.2.2.2-X of R4-2502315

Proposal 4.3: Revise the table 6a.2.2.2-2 of R4-2502315 to add Note 2

Proposal 4.4: Add a left column to table 6a.2.2.2-2 of R4-2502315 indicating row numbering

Parameters for the parabolic antenna should be defined in Table 6a.2.2.2-X. The following describes the WF items in R4-2419909 (i.e. 100 cm aperture (41.3dBi), 88.2 dBm EIRP).

Table 6a.2.2.2-X: NTN UE parameters (Parabolic antenna)

|  |  | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| X | Characteristics | Parabolic antenna | Parabolic antenna |
| X.1 | Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| X.2 | Antenna type and configuration | Directional Section 6.4.1 of [6] with 100 cm equivalent aperture diameter | Directional Section 6.4.1 of [6] with 60 cm equivalent aperture diameter |
| X.3 | Polarization | Circular (NOTE2) | Circular (NOTE2) |
| X.4 | Rx Antenna gain | 39.0 dBi | 34.3 dBi |
| X.5 | Antenna temperature | 150 K | 150 K |
| X.6 | Max EIRP | 88.2dBm | 74.1 dBm for Option 1 and 73.7 dBm for Option 2 |
| X.7 | Tx transmit power | 50 W for GSO and 10 W (40 dBm) for NGSO | 50 W for GSO and 10 W (40 dBm) for NGSO |
| X.8 | Tx antenna gain | 41.3 dBi | Option 1 - 36.6 dBi, Option 2 – 36.2 dBi |
| X.9 | Antenna elevation angle | 25˚ and 90˚ | 25˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

Proposal 5: Create the table 6a.2.2.2-X of R4-2502315 to account Only GEO phased array antenna configuration separately from LEO and GEO phased array antenna configuration.

## Thales’s contribution R4-2504658

Thales in this contribution, are proposing to select 78x78 cm phased array antennas for GEO connectivity. Furthermore, Thales is proposing 45x45 cm phase array antenna for LEO and GEO connectivity. LEO at 600 km is a more popular case and Thales is proposing to prioritize this scenario.

Thales’s proposals are as follow:

Observation 1: 100cm aperture VSAT configuration with 88.2dBm has been proposed for a new class of GSO, from a potential Set-2 configuration with antenna gain and transmission power compared with original GSO Set-1 configuration agreed in previous meeting.

Observation 2: For this reason, it does not make sense to consider a multiplication of coexistence simulations. The idea is that 100cm aperture VSAT will connect with GSO Set-2 only.

Proposal 1: Phased array 78 cm x 78 cm VSAT terminal and 100 cm VSAT terminal should consider connectivity with GSO configuration Set-2 only (with lower antenna aperture e.g. 2.5m and lower G/T e.g. 12dB/K compared with original GSO configuration Set-1 agreed in previous meeting).

Discussion point: the above needs further clarification

Discussion point: Set 1 was previously discussed for the simulation, so we need to have Set 2?

Proposal 2: Phased array 45 cm x 45 cm VSAT terminal should consider connectivity with original GSO configuration Set-1 and also with LEO (LEO@600km & LEO@1200km).

Proposal 3: LEO@600km should be prioritized compared with LEO@1200km.

Proposal 4: Companies to discuss if Bessel function (for the equivalent circular aperture of 50, 60 or 100 cm) can be still used for the simplification of the VSAT antenna assumptions.

Proposal 5: Prioritize circular aperture of 60 & 100 cm VSAT antenna and map the transmission powers and respective NFs in a table representing the potential combinations of NTN scenarios.

Proposal 6: Companies to further discuss if the calibration scenarios could be avoided (at least for scenarios using equivalent circular aperture).

Observation 3: In the case of phase array VSAT scenarios, some TN-NTN calibration phase might be required in order to double-check interference of adjacent lobes if implemented similarly by all companies.

# Summary of Common Parameters

The common theme amongst the 7 contributions were simplification of the co-existence scenarios and to focus on the worst-case interference scenarios

Further themes were the Satellite and VSATs simulation parameters which are converging.

## Common Proposals:

Ignore the 90-degree case and to focus on 25-degree or 35-degree elevations

From the 8 identified interference scenarios to focus on the following 5 cases

  - NTN UL interfering TN UL

  - NTN UL interfering TN DL

  - NTN DL interfering TN DL

  - TN DL interfering NTN DL

  - TN DL interfering NTN UL

Phased Array antennas to be evaluated for GEO and LEO

Equivalent Antenna aperture of 100 cm which relates to Parabolic Antenna

78x78 78 element phased array VSAT antenna for GEO connectivity and 45x45 element phased array antenna for LEO and GEO connectivity

VSAT phase array antennas for GEO and LEO to have different antenna elements and configurations

LEO at 600 Km to be prioritized against LEO at 1200 Km

# Summary of Open Issues

For the elevation angle we need to agree on 25 or 35 degree

Mismatch between Horizontally Polarized GEO Satellite and the Circularly Polarized VSAT to be clarified

Confirm or agree on definitive Noise Figure for VSAT connecting to GEO and LEO Satellites

Agree on definitive elemental gain of phased array antennas either 2.5dBi, 3.1dBi or 3.5dBi

Agree on conducted power of VSAT phased array antenna elements 6.9 dBm, 6.94 dBm or 4.4 dBm

Agree on maximum Rx antenna gain

GEO updated Satellite parameters, proposed by JSAT to be further discussed and agreed

The following matrix to be discussed and agreed for Co-existence simulation

| VSAT model SAN | 45 x 45 Phased Array | 60 cm Parabolic | 78 x 78 Phased Array | 100 cm Parabolic |
| --- | --- | --- | --- | --- |
| GEO | X | X | X | X |
| LEO 600km | X | X | N/A | N/A |
| LEO 1200km | X | X | N/A | N/A |

Interfering carriers in Uplink direction to have 300 MHz separation and 200 MHz separation in the Downlink direction.

In the following table, EIRP for GEO to be set at 74.1 dBm and 73.7 dBm for GEO and LEO case

[Sharp]: Two issues to consider here:

1: If we agree to EIRP for GEO to be set at 74.1 dBm and 73.7 dBm for GEO and LEO case then it should be included in the tables 6a.2.2.2-2 of RP-2402315. However, the table below is from the Sharp proposal to split the tables 6a.2.2.2-1 of 2315 into Only GEO and GEO and LEO, and further it is proposed by Sharp to also split and 6a.2.2.2-2 into Only GEO and GEO and LEO, and further to create a new table 6a.2.2.2-x. In the table below (6a.2.2.2-1), there is not a row for max EIRP, however in the proposal from Sharp table 6a.2.2.2-x would have a row for max EIRP

2: Do we agree to split the tables 6a.2.2.2-1 and 6a.2.2.2-2 of 2315 into Only GEO and GEO and LEO and add a new table 6a.2.2.2-x

| C | VSAT Phased array antenna Characteristics | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 0/ 3.5 | 0/ 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) (NOTE5） | Circular (RHCP or LHCP; as a starting point) (NOTE5） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 78 x 78 elements | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | TBD (dependent on decisions on EIRP) | 4.44 |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

SET 2 connectivity for GEO Satellites with 78x78 elements and/or SET1 connectivity for LEO/GSO with 45x45 elements

Companies to discuss if Bessel function (for the equivalent circular aperture of 50, 60 or 100 cm) can be still used for the simplification of the VSAT antenna assumptions

Prioritize circular aperture of 60 & 100 cm VSAT antenna and map the transmission powers and respective NFs in a table representing the potential combinations of NTN scenarios.

Companies to further discuss if the calibration scenarios could be avoided (at least for scenarios using equivalent circular aperture)

# Other Parameters to be Clarified

In Ericsson’s simulations, R4-2504264, what is the gap between the TN and NTN frequency used for the simulations?

# Moderator’s Suggested Way Forward

The Common Parameters are the parameters that are in line with various companies’ views and the open issues are parameters that need further discussion and agreement.

## Common Parameters

- Focus on 5 worst case scenarios

- NTN UL interfering TN UL

- NTN UL interfering TN DL

- NTN DL interfering TN DL

- TN DL interfering NTN DL

- TN DL interfering NTN UL

- Simulate 35-degree elevation in line with ETSI standards, ignore the 90-degree elevation

- Focus ion LEO at 600 Km and use the following table for the Simulation Matrix

| VSAT model SAN | 45 x 45 Phased Array | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array | 100 cm Parabolic (1st step for coexistence) |
| --- | --- | --- | --- | --- |
| GEO Set-1 (original) | X | X |  |  |
| GEO Set-2 (new) |  |  | X | X |
| LEO 600km | X | X | N/A | N/A |
| LEO 1200km | X (could be descoped) | X (could be descoped) | N/A | N/A |

- Assume Equivalent Antenna aperture of 100 cm which relates to Parabolic Antenna

- Adopt 78x78 78 element phased array VSAT antenna for GEO connectivity and 45x45 element phased array antenna for LEO and GEO connectivity

- VSAT phase array antennas for GEO and LEO to have different antenna elements and configurations

- Assume Satellites and VSATs are circularly polarised , ignore linearly palsied case

- Select modest and realistic antennae element gain of 2.5 dBi

- Select modest antenna conducted power of VSAT to be 4,4 dBm

- Select EIRP for GEO to be set at 74.1 dBm and 73.7 dBm for GEO and LEO case as in the following table

| C | VSAT Phased array antenna Characteristics | Only GEO | GEO and LEO |
| --- | --- | --- | --- |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 1) | 0/ 3.5 | 0/ 3.5 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB |
| 1.5 | Antenna polarization | Circular (RHCP or LHCP; as a starting point) (NOTE5） | Circular (RHCP or LHCP; as a starting point) (NOTE5） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 78 x 78 elements | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | TBD (dependent on decisions on EIRP) | 4.44 |
| 1.11 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees |
| 1.12 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees |
| 1.13 | Normal direction | Toward X+ axis | Toward X+ axis |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. |  |  |  |

## Parameters to be Discussed

- Whether to use SET 1 or SET 2 in GEO and LEO Simulations use of the Bessel Functions to simulate VSAT Phase Array Antenna patterns and performance

- Required Calibration scenarios that are needed

- Whether to prioritize circular aperture of 60 & 100 cm VSAT antenna

- The use of Bessel function to simulate the performance and radiation pattern of phased array VSATs

- The satellite parameters defined in SET 1 do not appear to be feasible for the actual satellite implementation. Would it be appropriate to proceed with the coexistence study using these values?

# References

R4-2413519

R4-2500574