3GPP TSG-RAN4 Meeting #114bis	R4-2504663

# Wuhan, China, 7th – 11th April 2025

Agenda item:			6.1

Source:	Moderator (CMCC)

Title:	Topic summary for [114bis][105] HPUE_NR_bands

Document for:	Information

Introduction

This summary focuses on Rel-19 HPUE for NR FR1 TDD/FDD single band, including agenda 6.5.

Topic #1: HPUE in a single FDD band

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2504095 | China Unicom | TR Reserved for post-meeting approval |
| R4-2504185 | Ericsson | Observation 1: while Public Safety limits can be met by applying large A-MPR in several regions of the carrier, this has significant impact on possible configuration of e.g. common PUCCH with NS_12 – NS_15 indicated in the cell for any power class, and the back-off for PUSCH/SRS transmissions can be up to 14 dB for PC2 operation. Notwithstanding, it is also recognised that harmful interference to Public Safety is unacceptable. Observation 2: if introduced for n5, the NS_14 and NS_15 network signalling values could be defined using the modified MPR behaviour bitmap, support mandated for PC2 (the bit shall be set) but optional for PC3 (the bit may be set). The gNBs would then be aware of the support of the NS values, which is necessary. |
| R4-2504186 | Ericsson | Draft CR to 38.101-1: Introduction of requirements for PC2 operation in band n26 |
| R4-2504555 | Qualcomm Inc. | Proposal 1: It’s feasible to adopt the PC2 requirements in draft CR R4-2503019 for band n26. Proposal 2: It’s feasible to adopt the PC2 A-MPR requirements for NS_48 and NS_49 in R4-2501984 for both 1Tx and 2Tx. Proposal 3: Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed. |

Open issues summary

Sub-topic 1-1 Power Class 2 (PC2) operation in band n26

WF during last meeting:

Issue 1: n26 PC2 There are still A-MPR requirements that need to be defined for n26 PC2, specifically NS-12, NS-13, NS-14, NS-15 and RSD. - Set the analysis (including 3MHz) in the draft CR R4-2503019 as baseline. - These NS and RSD values need to be defined based on the analysis results from TR 38.896, and any additional relevant input.

Issue 1-1: PC2 A-MPR for n26

- Proposals

  - Option 1: PC2 requirements in draft CR R4-2503019 for band n26 could be adopted (Ericsson, Huawei)

  - Option 2: Others

- Recommended WF

  - Further check option 1.

Sub-topic 1-2 Power Class 2 (PC2) operation in band n5

Issue 2-1: PC2 A-MPR for n5

- Proposals

  - Option 1: if introduced for n5, the NS_14 and NS_15 network signalling values could be defined using the modified MPR behaviour bitmap, support mandated for PC2 (the bit shall be set) but optional for PC3 (the bit may be set). (Ericsson)

Table 6.2.3.1-1A: Mapping of network signalling label NR band; Value of additionalSpectrumEmission. 0; 1; 2; 3; 4; 5; 6; 7. n1; NS_01; NS_100; NS_05; NS_05U; NS_48; NS_49; Reserved. […]. n5; NS_01; NS_100; NS_14 (NOTE 3); NS_15 (NOTE 3); Reserved. […]. n26; NS_01; NS_100; NS_12; NS_13; NS_14; NS_15; Reserved. […]. NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. NOTE 3: Support of the additional requirement indicated by this network signalling value is indicated by modified MPR behaviour for this band (Annex L.1).. # L.1 Indication of modified MPR behavior This annex contains the definitions of the bits in the field modifiedMPR-Behaviour indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. NOTE 1: In the present release, the modifiedMPR-Behaviour is indicated [7] by an 8-bit bitmap per supported NR band. Table L.1-1: Definitions of the bits in the field modifiedMPR-Behaviour NR Band; Index of field (bit number); Definition (description of the supported functionality if indicator set to one); Notes. n5; 0 (leftmost bit); Additional requirements for network signalling values NS_14 and NS_15 as defined in the respective Clause 6.5.3.3.19 and Clause 8 of 38.101-1 v19.0.0.; This bit shall be set to 1 by a UE supporting Power Class 2 as indicated by ue-PowerClass in BandNR. This bit may be set to 1 by a UE supporting Power Class 3 as indicated by ue-PowerClass in BandNR..

  - Option 2: Others.

- Recommended WF

  - Discuss option 1 firstly.

Sub-topic 1-3 NS_05 NS_48 NS_49 PC2 A-MPR clean-up

WF during last meeting:

Issue 2: A-MPR clean-up For NS_48 and NS_49, and to account for PC2 2Tx rev-IMD, further study the approach below: - Void Note 7 in Table 6.2.3.1-1, - Replace the [2] dB PC2 2Tx A-MPR relaxation with a maximum of 0.5dB relaxation for cases where the specified PC2 (1Tx) A-MPR < PC3 A-MPR +3dB, - To avoid tightening the Rel-17 and Rel-18 requirements, these changes are applicable to Rel-19 only.

Issue 3-1: A-MPR clean-up

- Proposals

  - Option 1: Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed. (Huawei):

  - Option 2: Others.

- Recommended WF

  - Discuss option 1 firstly.

Issue 3-2: A-MPR for NS_48 NS_49

- Proposals

  - Option 1: Adopt the PC2 A-MPR requirements for NS_48 and NS_49 in R4-2501984 for both 1Tx and 2Tx. (Huawei):

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |

  - Option 2: Others.

- Recommended WF

  - Discuss option 1 firstly.