# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504700

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.10.5

Source:	Samsung

Title:	Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod

Document for:	Approval

Introduction

This t-doc provides WF for [114bis][323] NTN_testing_NGSO_channel_model_and_demod.

WF

Sub-topic 1-1 TE emulated channel model for satellite mobility

Issue 1-1-1 TE emulated channel model comparison with GMAT

- agreement

  - Keep previous agreement: RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102

    - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix

  - Not need to compare the Kepler’s equation and equation of motion model with GMAT

Issue 1-1-2 FFS to consider the Earth’s rotation during signal flight time for more accurate estimations of propagation delay

- Proposals

  - Option 1

ΔtDL=RDLtTc1-1cvUEtT・RDLtTRDLtT

  - Option 2

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

  - Option 4

Δt=DelayERt=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

ρtR ECEF=rtRECEF-UEtRECEF

- agreement

  - Consider the earth rotation during signal flight time impact into TE-emulated channel modelling

    - Take the proposals from option 1 or option2/4 for DL propagation delay calculation

    - Companies are encouraged to cross check the equations to derive the total propagation delay and its error.

      - Note: The details of option 1, option 2 and option 4 can be referred as R4-2503710, R4-2503247 and R4-2504344, respectively

  - Not consider the earth rotation for doppler shift calculation

Issue 1-1-3 Time instance to calculate the propagation delay

- Proposals

  - Option 1

    - For downlink , calculate delay from the satellite at time t to the UE at time t+ΔT;

    - For uplink, calculate the delay from UE at time t-ΔT to the satellite at the time t, where ΔT is the signal transmission time

- agreement

  - Option 1 as baseline assumption.

Issue 1-1-4 Accuracy requirement to solve Kepler’s equation based on Newton-Raphson method

Issue 1-1-5 Number of iterations to solve Kepler’s equation based on Newton-Raphson method

Issue 1-1-6 Interpolation method for the Doppler shift and Propagation delay

Issue 1-1-7 Accuracy requirements for maximum delay errors to obtain the propagation delay and Doppler shift trajectory

- agreement

  - The number of iterations to solve Kepler’s equation based on Newton-Raphson method is up to TE implementation

  - The interpolation method for the Doppler shift and Propagation delay trajectory is up to TE implementation

    - Note: the earth rotation during signal flight time impact should be considered to calculate the propagation delay

  - To derive accuracy requirement,

    - Baseline assumption

      - number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method

      - Calculate the Propagation delay and Doppler shift every 1ms

    - For accuracy requirement

      - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side

      - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz)

        - The time period to calculate the error is 1s

    - TE should meet the minimum accuracy requirement defined.

    - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

Issue 1-1-8 Assumption of Elevation angle range for RF test

- agreement

  - The elevation angle used for RF requirement test can be decided by RF session pending on test purpose

Issue 1-1-9 Initial ephemeris information for LEO-600 satellite

- Proposals:

  - Option 1： Use table 1 as the initial ephemeris information for LEO-600 satellite scenario

Table 1	Initial ephemeris information for LEO-600 stating from elevation angles of 30 degrees.

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | [-3451.1273] |
| positionY-r17 | km | [5703.5635] |
| positionZ-r17 | km | [2071.0809] |
| velocityVX-r17 | km/s | [0.87360] |
| velocityVY-r17 | km/s | [-2.06700] |
| velocityVZ-r17 | km/s | [7.21231] |

- agreement

  - Option 1

Issue 1-1-10 Initial ephemeris information for LEO-1200 satellite

- Proposals:

  - Option 1：

    - If RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario, reuse TS 38.508 Table 7.6.2.1-3a as the initial ephemeris information

Table Initial ephemeris information for LEO-1200 stating from elevation angles of 30 degrees (Copy from TS 38.508 Table 7.6.2.1-3a, serving cell 30 degrees elevation angle conditions).

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | -3816.2176 |
| positionY-r17 | km | 6364.098 |
| positionZ-r17 | km | 1544.9954 |
| velocityVX-r17 | km/s | 0.51036 |
| velocityVY-r17 | km/s | -1.39884 |
| velocityVZ-r17 | km/s | 7.09518 |

    - RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario if there are test cases whose test time becomes more than 250 seconds

- agreement

  - Option 1

Sub-topic 1-2 RAN4 requirements

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation

- agreement

  - Following the agreement in the last meeting

    - Consider Keplerian model and equation of motion models in UE side as baseline assumption for RAN4 requirement

      - Note: There is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information

Issue 1-2-2 Simulation Assumption for channel model

- agreement

  - Only the Rel-17 NTN fading channel models with timing-varying Doppler Shift and propagation delay is considered for this WI

Issue 1-2-3 Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- Proposals:

  - Option 1:

    - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

    - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

  - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

  - Option 3:

    - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with.

    - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state.

  - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements.

- Recommended WF:

  - Further discussion

Issue 1-2-4 FFS to revisit the UE assumption of residual doppler is 0.1ppm

- agreement

  - Keep the UE assumption of residual doppler is 0.1ppm

Issue 1-2-5 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model

- agreement

  - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

  - FFS on test 1-3 and 1-4

    - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable

Issue 1-2-6 Test Scope for IoT NTN with applying TE-emulated time vary doppler shift and propagation delay models

- agreement

  - No requirement for IoT NTN PUSCH with TE-emulated time vary doppler shift and propagation delay channel model

Issue 1-2-7 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model

- agreement

  - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1

    - FFS If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model

Issue 1-2-8 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model

- agreement

  - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model

  - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model

    - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1

    - FFS to cover test 3

Issue 1-2-9 Simulation Assumption for cellSpecificKoffset-r17 with applying the time-varying Doppler shift and propagation delay model

- agreement

  - Adopt the selected Rel-17 NTN test cases and their associated simulation assumptions as the baseline for this WI

Issue 1-2-10 Simulation Assumption for IoT NTN PDSCH requirement

- agreement

  - Apply the time-varying Doppler shift and propagation delay model with LEO-600 for agreed test cases using the same parameters

Issue 1-2-11 Simulation Assumption for IoT NTN NPUSCH requirement

- agreement

  - No requirement for IoT NTN PUSCH with TE-emulated time vary doppler shift and propagation delay channel model

Issue 1-2-12 Applicability rule

- agreement

  - Add new applicability description which test cases use the time-varying Doppler shift and propagation delay model in Rel-19 TS 36.102 and TS 38.101-5

  - Details of applicability rule are pending on the agreed test cases

Sub-topic 1-3 Inclusion in Specfication

Issue 1-3-1 Specification of channel Model

- agreement

  - Capture the TE-emulated channel model into the normative part of Appendix, details can be discussed in the CR drafting stage

- Reference

[1] R4-2500586, “Topic summary for [114bis][323]  NTN_testing_NGSO_channel_model_and_demod “, Moderator (Samsung)

- Appendix (for information)

  - A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

A.1.1	Keplerian model based estimation

Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

A.1.2	Equation of Motion based estimation

Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if the satellite position/velocity is updated every 1 second.

Step 2	Solve the equation of motion with the 4th-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

Where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

  - A.2	Varying Doppler shift and propagation delay generation

A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since it is assumed that UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |

ρtECEF=ρt,xECEIρt,yECEIρt,zECEI=rt,xECEF-UEt,xrt,yECEF-UEt,yrt,zECEF-UEt,z

Range(t)=ρtECEI=ρt,xECEI2+ρt,yECEI2+ρt,zECEI2

RangeRate(t)=ρtECEF⋅vtECEFRnage(t)=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFRange(t)

Delay(t)=Range(t)c

Doppler(t)=-RangeRate(t)×fcc

Additionally, the elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0