# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504723

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.11.7

Source: 	Moderator: Intelsat

Title: 	Way Forward for  [114bis][313] NR_NTN_Ku_Band_Coexistence

Document for:	Discussion and Decision

# Introduction

Following a productive discussion at RAN4 #114-bis in Wuhan, several agreements were reached on the simulation of co-existence between Terrestrial and Non-Terrestrial use cases. Remaining parameters to be discussed at future RAN4 meetings.

# Agreed Way Forward

The following parameters were agreed:

- To focus on 4 worst case scenarios

- NTN UL interfering TN UL

- NTN DL interfering TN DL

- TN DL interfering NTN DL

- TN DL interfering NTN UL

- Simulate 90 and 35-degree elevation angles in line with ETSI standards

- Use the following Table 1 for the simulation matrix

The following Table 1 illustrates agreed simulation matrix:

Table 1 Simulation Matrix

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array |
| --- | --- | --- | --- |
| GEO | X | X | X |
| LEO 600km | X | X |  |

Following Table 2 illustrates agreed LEO and GEO Satellite parameters:

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 4 m | 0.714 m |
| G/T |  |  | 16.5 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 44 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

And the following Table 3 illustrates the agreed Phased Array and Parabolic Antenna parameters:

Table 3 Phase Array and Parabolic Antenna Parameters

| 1 | VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- | --- |
| 1.1 | Antenna pattern | TR 38.921, TR 38.922 | TR 38.921, TR 38.922 | Circular Aperture TR 38.863 |
| 1.2 | Element gain (dBi) (Note 1) | 3.5 | 3.5 | N/A |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis | 90° for Y axis 90° for Z axis | N/A |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB | 30 dB | N/A |
| 1.5 | Antenna polarization (Note 5) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) | Circular (RHCP or LHCP) |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 3) | 45x45 elements | 78x78 elements | 60 cm diameter aperture |
| 1.7 | Number of supported polarizations, P | 1 | 1 | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda | 0.5 lambda | N/A |
| 1.9 | Array Ohmic loss (dB) (Note 1) | 2 | 2 | N/A |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2) | 6.9 | 9.1 | N/A |
| 1.11 | Tx Transmit power | 10 Watts for GEO and for LEO | 50 Watt for GEO | 10 Watts for GEO and for LEO |
| 1.12 | Maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees | 0~360 degrees | 0~360 degrees |
| 1.13 | Vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees | 0~60 degrees | 0~60 degrees |
| 1.14 | Normal direction | Toward X+ axis | Toward X+ axis | Toward X+ axis |
| 1.15 | Max EIRP Value | 76.6 dBm | 88.2 dBm | 76.6 dBm |
| 1.16 | Noise Figure | 6 dB | 6.0 dB for GEO | 2.5 dB for both LEO and GEO |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization |  |  |  |  |

# References

R4-2413519

R4-2500574

R4-2502315

R4-2504477