# 3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504725

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.9.5

Source:	Astrum Mobile, Qualcomm, ZTE, Huawei

Title:	Way Forward for [114bis][318] 5G_Broadcast_GSO_NTN_band

Document for:	Approval

# Band definition and system parameters

### Sub-topic 2-1: New Suffix

Issue 2-1-1: Whether to use a new suffix for new SDO band in TS 36.102 and TS 36.108

- Agreement:

  - UE RF: To use suffix in TS 36.102 to specify the corresponding SDO band, system parameters, UE RF requirements  for LTE based 5G broadcast over GSO satellite.

  - SAN RF: To consider suffix in TS 36.108 to specify the corresponding SDO band, system parameters, SAN RF requirements (if needed) for LTE based 5G broadcast over GSO satellite.

### Sub-topic 2-2: Operating band, transmission bandwidth, channel arrangement

Issue 2-2-1: Operating band

- Agreement:

  - The new SDO band operating in 1467 to 1492 MHz can be numbered as 246

Issue 2-2-2: Transmission bandwidth configuration(NRB)

- Agreement:

  - To define 50RB transmission bandwidth configuration for SCS=15kHz for 10MHz channel bandwidth in TS 36.102 and TS 36.108

Issue 2-2-3: Channel bandwidth

- Agreement:

  - To define PMCH of 10MHz bandwidth configuration for 15kHz SCS in TS 36.102 and TS 36.108 as following table.

Table 5.3C-2: 5G broadcast PMCH bandwidths per operating band in TS 36.102 and TS 36.108

| Operating Band | PMCH bandwidth |
| --- | --- |
|  | 10MHz |
| 246 | Yes |

Issue 2-2-4: Channel spacing

- Agreement:

  - To reuse the same PMCH channel spacing for LTE-based 5G Broadcast operating for satellite.

    - Nominal channel spacing between adjacent broadcast channels is defined as follows

        - Nominal Channel spacing = PMCH bandwidth

    - where PMCH bandwidth is the broadcast bandwidth indicated by upper layer signaling pmch-Bandwidth in the MBSFN area (see TS 36.331 [7]).

Issue 2-2-5: Channel raster, carrier frequency and EARFCN

- Agreements:

  - The channel numbers for LTE based 5G terrestrial broadcast band are proposed as 228051 -<1>- 227480

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 246 | 100 | 1467 | 228051 | 228051 -<1>- 228300 | N/A |  |  |

# UE RF requirements

### Sub-topic 3-1: UE RF requirements

Issue 3-1-1: Reference sensitivity

- Agreement:

  - Add a new chapter 7.3C

  - Specify Reference sensitivity as -97 dBm for 10MHz PMCH.

Issue 3-1-2: Maximum input level

- Agreement:

  - Add a new chapter 7.4C

  - To specify the maximum input level requirements as -40dBm which is in line with NTN UE maximum input level requirement of 36.102.

Issue 3-1-3: ACS

- Agreement:

  - Add a new chapter 7.5C

  - Take the following ACS requirements as the baseline.

Table 7.5C-1: Adjacent channel selectivity

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| ACS | dB | 33 |

Table 7.5C-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |
| PInterferer | dBm | REFSENS +45.5dB |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 7.5+0.0075 / -7.5-0.0075 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.1.3. |  |  |

Table 7.5C-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -53.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 5 |
| FInterferer (offset) | MHz | 7.5+0.0075 / -7.5-0.0075 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.1.3. |  |  |

Issue 3-1-4: In-band blocking

- Agreement:

  - Add a new chapter 7.6C.2 for in-band blocking

  - To specify in-band blocking for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102 as the baseline.

Table 7.6C.2-1: In band blocking parameters

| Rx parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| BWInterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5+0.0025 |
| FIoffset, case 2 | MHz | 12.5+0.0125 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |

Table 7.6C.2-2: In-band blocking

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 246 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: BW refers to 10 MHz |  |  |  |  |

Issue 3-1-5: Out-of-band blocking

- Agreement:

  - Add a new chapter 7.6C.3 for out-of-band blocking

  - To specify out-of-band blocking for 10MHz PMCH for LTE based 5G broadcast utilizing satellite in TS 36.102 as following tables.

Table 7.6C.3-1: Out-of-band blocking parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.6C.3-2: Out of band blocking

| Operating band | 1 | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | FInterferer (CW) | MHz | FDL_low -40 to FDL_low -85 | FDL_low -85 to FDL_low -110 | FDL_low -110 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 75 | FDL_high +75 to FDL_high +100 | FDL_high +100 to +12750 MHz |
| 246 | PInterferer | dBm | -44 | -30 | -15 |

Issue 3-1-6: Narrow band blocking

- Agreement:

  - Narrow-band blocking requirements are not applicable to LTE based 5G terrestrial broadcast.

Issue 3-1-7: Spurious response

- Agreement:

  - No spec impact.

Issue 3-1-8: Intermodulation characteristics

- Agreement:

  - Add a new chapter to capture Intermodulation characteristics for LTE-based 5G Broadcast.

Issue 3-1-9: Spurious emissions

- Agreement:

  - No spec impact.

Issue 3-1-10: Reference measurement channels

- Agreement:

  - To specify the RMC of 15kHz SCS for 10MHz PMCH in TS 36.102 as following tables as the baseline. FFS on other SCSs

Table A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | TBD | TBD |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 3 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | TBD | TBD |
| MBMS UE Category |  | ≥ 1 | ≥ 2 |
| Note 1: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

# SAN RF requirements

### Sub-topic 4-0: SAN RF requirements set

- Agreement:

  - For all requiremens listed in sub-topic 4-1, necessity for certain requirements require further verification to consider requirements structure approach in TS36.104 (for TN broadcast), as well as TS38.108 (NTN SAN specification).

  - All SAN RF related agreements are considered as baseline, subject to verification next RAN4 meeting.

  - Proponent are encouraged to provide more details on their current and/or future satellite characteristics.

### Sub-topic 4-1: SAN RF requirements

Issue 4-1-1: Satellite Access Node output power

- Agreement:

  - No specification impact.

Issue 4-1-2: RE power control dynamic range

- Agreement:

  - No specification impact.

Issue 4-1-3: Total power dynamic range

- Agreements

  - FFS on whether total power dynamic range requirement applies for LTE based 5G broadcast over Satellite

Issue 4-1-4: Transmit ON/OFF power

- Agreement:

  - No specification impact.

Issue 4-1-5: Transmitted signal quality

- Agreement:

  - No specification impact.

Issue 4-1-6: Occupied bandwidth

- Agreement:

  - No specification impact.

Issue 4-1-7: Adjacent Channel Leakage Power Ratio

- Agreement:

  - Reuse the legacy requirements for GEO

Issue 4-1-8: EVM

- Agreement:

  - To specify the EVM requirements for LTE based 5G broadcast over satellite based on the following table.

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| NOTE 2: Only QPSK and 16QAM apply. |  |

Issue 4-1-9: Out-of-band emissions

- Agreement:

  - Reuse the legacy requirements.

Issue 4-1-10: Transmitter spurious emissions

- Agreement:

  - Reuse the legacy requirements.

Issue 4-1-11: Transmitter intermodulation

- Agreement:

  - No specification impact, not applicable to SAN.

Issue 4-1-12: Window length

- Agreement:

  - To reuse EVM window length for 10MHz defined in 36.104.

### Sub-topic 4-2: SAN RF conformance requirements

- Agreement:

  - As per WID: RAN4 to further discuss whether SAN RF conformance requirements and tests will be defined.