# 3GPP TSG-RAN WG4 Meeting #114bis		  R4-2504726

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.15.2

Source:	Qualcomm Incorporated, vivo

Title:	TP to TR 38.771 on MU

Document for:	Approval

# 1  Introduction

In this paper, we provide test proposal to TR 38.771 on MU assessments for STxMP testing.

# 2  Text proposal

## <<Start of Change>>

## 5.4	Test procedure

<Editor’s note: outcome of test procedure for STxMP UE RF testing can be captured in this clause>

### 5.4.1	Min peak EIRP

<<Unchanged part is omitted>>

After RAN4 discussion, the conclusion on AoA pair (AoA1 and AoA2) selection is as following which is a combination of Option 1 and Option 2:

-	  If UE declares the AoA1, then

a) The declaration of AoA1 direction can be any direction but need to be aligned with the measurement grid of beam peak search of single CC without STxMP.

b) AoA2 direction is to use AoA1 + or - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in    xz plane of test system

c) AoA1+ UE declared offset or AoA1 - UE declared offset needs to be declared by UE

d) If AoA1 and AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test.

-   If UE doesn’t declare AoA1, then

a) the default AoA1, i.e., the beam peak direction of single CC operation without STxMP is applied

b) AoA2 direction is to use AoA1 + and - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

c) AoA2 direction is AoA1 + the declared offset

d) AoA2 direction is AoA1 - the declared offset

e) If AoA1 and any of the two AoA2 directions pass EIRP PUMAX,f,c,k requirements, the DUT pass the test.

Note that the above AoA pair (AoA1 and AoA2) selection is based on the UE declared UE orientation.

## <<End of Change>>

## <<Start of Change>>

# Annex A:
Measurement uncertainty budget for STxMP UE RF testing methodology

<Editor’s note: the outcome of MU budget for STxMP UE RF testing methodology can be captured in this clause>

The uncertainty tables shall be presented with two stages:

-	Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

-	Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

MU for STxMP testing which is leveraged from legacy EIRP/TRP MU budget for PC1 and PC5 specified in Table B.3.2-6 of [38.903].

Compared to legacy uncertainty assessments for UE RF testing defined in [38.903], the value for the uncertainty source of systematic error related to beam peak search can be set as 0. The error due to finite testing directions can be handled in test tolerance. How to impact TT will be finalized in RAN5.

The preliminary MU budget for UE RF testing methodology is defined in Table A.1-1.

Table A.1-1: Uncertainty assessment for Minimum EIRP, EIRPmax and TRP measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC1 and PC5 UEs and normal temperature condition

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.02 | Normal | 2.00 | [0.01] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 1) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | Uncertainty of the RF power measurement equipment (NOTE 3) | 2.16 | Normal | 2.00 | [1.08] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.10 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Influence of TRP measurement grid (NOTE 4) | 0.25 | Actual | 1 | [0.25] |
| 14 | Influence of beam peak search grid (NOTE 5) | 0.00 | Actual | 1 | [0.00] |
| 15 | Multiple measurement antenna uncertainty (NOTE 9) | 0.15 | Actual | 1 | [0.15] |
| 16 | DUT repositioning | 0.00 (NOTE 4) 0.35 (NOTE 5) | Rectangular | 1.73 | [0.00] (NOTE 4) [0.20] (NOTE 5) |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 17 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 18 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 19 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 20 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | [0.75] |
| 21 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 22 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 23 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 24 | Quality of quiet zone for calibration process (NOTE 1) | 0.4 | Actual | 1.00 | [0.4] |
| 25 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 26 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 27 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Systematic uncertainties (NOTE 6) |  |  |  | Value |
| 28 | Systematic error due to TRP calculation/quadrature (NOTE 4) |  |  |  | [0.00] |
| 29 | Influence of noise (23.45GHz <= f <= 32.125GHz) |  |  |  | [0.13] |
| 29 | Influence of noise (32.125GHz < f <= 40.8GHz) |  |  |  | [0.27] (NOTE 4) [0.20] (NOTE 5) |
| 30 | Systematic error related to beam peak search (NOTE 5) |  |  |  | [0.00] |
| Total measurement uncertainty |  |  |  |  | Value |
| EIRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.63] |
| EIRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.70] |
| TRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.64] |
| TRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.78] |
| NOTE 1: The quality of quiet zone is the same for EIRP and TRP. Value based on procedure defined in clause D.2 of TR 38.810 for Quiet Zone size less or equal to 30 cm. NOTE 2: The analysis was done only for the case of operating at max output power, in-band, non-CA and is valid for SISO and MIMO. NOTE 3: The assessment assumes maximum DUT output power. NOTE 4: This contributor shall only be considered for TRP measurements. NOTE 5: This contributor shall only be considered for EIRP measurements. NOTE 6: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 7: Void. NOTE 8: Void NOTE 9: Applies to the system which has a structure of mechanical feed antenna positioning. declaration. |  |  |  |  |  |

The impact due to finite testing directions including limited choices for AoA1&AoA2 test directions can be handled in an additional Test Tolerance, TTFiniteRes. FFS on the value of TTFiniteRes.

## <<End of Change>>