3GPP TSG-RAN4 Meeting #114bis R4-2504728 # Wuhan, Hubei, China, 07th – 11th April, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.870; CR; Draft; rev; -; Current version:; 19.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; Draft CR to 38.870 on NTN OTA performance metric and test method. Source to WG:; Vivo, Samsung, Apple. Source to TSG:; R4. Work item code:; TRP_TRS_MIMO_OTA_Ph3-Core; Date:; 2025-03-28. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This is further updated draft CR (based on endorsed R4-2502301 last meeting) to provide additional content to accommodate NTN test method and performance metric.. Summary of change:; Add sub-clause for NR-NTN and IoT-NTN radiated power performance metric and test procedure.. Consequences if not approved:; The spec is not appicable for NTN UE testing.. Clauses affected:; 5.3, 7.4.5, 7.4.6. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

## <<< START OF CHANGES >>>

### 4.2.1	UE use scenarios for TRP TRS test

The following use scenarios are considered for TRP TRS test:

-	Talk mode using head & hand phantom for narrow phones between 56 mm and 72 mm and for wide phones with a width >72 mm and <92 mm.

-	Browsing mode using hand phantom for narrow and wide phones

-	Using forearm phantom for wrist-worn devices

-	Free Space is used for devices not used in above-mentioned scenarios, other phantoms are not precluded for wearable devices

For TN testing of smartphones, both browsing mode and talk mode shall be covered. Free space (FS) testing for smartphone is low priority.

For NTN testing of smartphones, the use scenarios are categorized as following which are based on UE declaration:

- Usage scenario 1: both head+hand mode and hand only mode

- Usage scenario 2: hand only mode targeted for specific UE orientation usage

- Usage scenario 3: hand only mode targeted for arbitrary UE orientation usage

Note: For each supported NR NTN band, declare only one supported scenario.

For wrist-worn Redcap devices, forearm phantom is the first priority. FFS other Redcap form factor devices.

For other device types, free space (FS) testing configuration is the first priority.

Note:	the UE positioning guideline described in Clause 6 and phantom definition described in Annex D are RATs agnostic.

## <<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

5.3	Definition of the Partial sphere Radiated Power (PsRP)

5.3.1	Definition of the Partial sphere Radiated Power (PsRP) for AC

5.3.1.1	Applicability of PsRP metrics

The PsRP metrics are defined in Clauses 5.3.1.2 through 5.3.1.3, with the applicability provided in Table 5.3.1.1-1 below.

Table 5.3.1.1-1: Applicability of PsRP metrics

| Clause | PsRP definition | Applicability |
| --- | --- | --- |
| 5.3.1.2 | UHRP= | FFS |
| 5.3.1.3 | PsRP= | FFS |

5.3.1.2	Upper Hemisphere Radiated Power

The UHRP metric is defined as total radiated power in three-dimensional upper hemisphere. FFS details

5.3.1.3	Partial sphere Radiated Power (PsRP)

The PsRP metric is defined as the total power radiated in the Theta and Phi range from x to y degrees. FFS details

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

7.4.5	TRP test procedure for NR NTN

7.4.5.1	UE configuration

NR-NTN TRP and TRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [31], section 6.2.1 (UE maximum output power) using the default settings specified in TS 38.521-5 [31] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-5 Clause 4.3.

7.4.5.2	Test procedure

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NR-NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause 5.1.1.2.

For NR-NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [31] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

7.4.6	TRP test procedure for IoT-NTN

7.4.6.1	UE configuration

IoT-NTN TRP and TRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.A.1 (UE maximum output power for category M1) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-7 Clause 4.3.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [32], section 6.2.B.1 (UE maximum output power for category NB1 and NB2) using the default settings specified in TS 36.521-4 [32] and TS 36.508 [33] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-9 Clause 4.3.

7.4.6.2	Test procedure for cat M1 NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For cat M1 NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause 5.1.1.2.

For cat M1 NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

7.4.6.3	Test procedure for cat NB-IoT NTN

<Editor’s note: The test procedure and performance metric for scenario 2 defined in Clause 4.2.1 will be further added. >

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NB-IoT NTN UE support usage scenario 1 or usage scenario 3 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

The performance metric is TRP, and TRP value is calculated using the TRP integration approaches outlined in Clause 5.1.1.2.

For NB-IoT NTN UE declaring support of usage scenario 2 defined in Clause 4.2.1, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [32] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point [on the surface declared by the manufacturer] and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the PsRP performance metric using the approaches outlined in Clause 5.3.1

## <<< END OF CHANGES >>>