3GPP TSG-RAN4 Meeting #114-bis	R4-2504729

# Wuhan, China, 7th - 11st April, 2025

Agenda item:		7.14.2.3

Source:				Keysight Technologies UK Ltd, Spirent Communications, ETS-Lindgren, CAICT

Title:					TP for TR 38.762: Clarifications of Channel Models and Applicability

Document for:		Approval

# Introduction

This contribution intends to provide clarifcations of channel models given the different environmental conditions and path loss model assumptions used in different SDOs [1], [2] as agreed in the last WF [3].

# Discussion

As pointed out in [4], the channel model validation targets/limits for dynamic channel models with link adaptation are different between different SDOs, i.e., 3GPP RAN4 and CTIA. As highlighted in [1], [2], these differences are due to differences in path loss models and environmental conditions: while 3GPP RAN4 selected the UE noise limited environmental condition with the full path loss model from [5], CTIA selected the SIR based environmental condition with an alternate path loss model as the full path loss model yields signal (S) and interference (I) power levels that are not feasible in commercial 3D MPAC MIMO OTA systems [1] [2]; the CTIA approach was just re-confirmed in March 2025 with the endorsedment of [2]. The key differences between the CTIA and 3GPP RAN4 dynamic MIMO OTA test plan assumptions are summarized in Table 1.

Table 1: Key differences between CTIA and 3GPP Dynamic MIMO OTA test plans

|  | CTIA | 3GPP |
| --- | --- | --- |
| Environmental Condition | SIR-Based | UE Noise-Limited |
| Path Loss Model | Adjusted Path Loss based on K-Factor | Full Path Loss Model per TR 38.901 |
| Channel Models | UMa | UMa & UMi |

In the last RAN4 meeting, it was agreed to potentially clarify and/or rename the channel models [3]

### Sub-topic 1-1 Dynamic channel model emulation and validation Issue 1-1-1: Dynamic channel model validation targets in different SDOs Agreement: Pending the alignment of path loss models with different SDOs, the channel models and their applicability might need to be further clarified and/or renamed by RAN4#116.

Given the lack of alignment of path loss models between SDOs, this TP intends to provide additional clarifications and applicability statements.

Proposal 1: Approve the TP with the understanding that additional changes and modifications can be made

# References

- R4-2502175, On Channel Models in CTIA and 3GPP for Dynamic FR1 MIMO OTA, Keysight Technologies, Spirent Communications, CAICT, 3GPP TSG-RAN4 Meeting #114, February 2025

- OTAMIMO_2025_003_003 V1, Keysight Technologies, ETS-Lindgren, Spirent Communications, AT&T, Verizon
This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

- R4-2502296, WF for [114][328] MIMO_OTA_Ph3, CAICT, 3GPP TSG-RAN WG4 Meeting #114, February 2025

- R4-2500481, On FR1 MIMO OTA Dynamic Channel Model, Apple, 3GPP RAN WG4 Meeting #114, February 2025

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# 5	Performance metrics

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e.,

TMTUMi=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMi

and

TMTUMa=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMa

Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF

CTMTUMi,10%=CDF10%TMTUMi

CTMTUMi,50%=CDF50%TMTUMi

CTMTUMi,90%=CDF90%(TMTUMi)

CTMTUMa,10%=CDF10%TMTUMa

CTMTUMa,50%=CDF50%TMTUMa

CTMTUMa,90%=CDF90%(TMTUMa)

Whether and how to combine the CTMT metrics for different routes/channel models is FFS.

<<< Skip Unchanged Sections >>>

## 6.2	Environmental conditions

The test cases in this document shall utilize the UE noise limit condition, i.e., UE throughput is characterized as a function of signal power incident to the DUT antennas.

Here, the signal power takes the full UMa/UMi path loss model into account, i.e., the path loss is modelled per Table 7.4.1-1 of [6]. The normalized signal power with gNB antenna effect would therefore be modelled as illustrated in Figure 6.2-1 for the UMa route/model and in Figure 6.2-2 for the UMi route/model.

Figure 6.2-1: Signal Power Profile with gNB antenna effect for UE Noise-Limited Environment using UMa Route with full UMa path loss model from Table 7.4.1-1 of [6]

Figure 6.2-2: Signal Power Profile with gNB antenna effect for UE Noise-Limited Environment using UMi Route with full UMi path loss model from Table 7.4.1-1 of [6]

<<< Skip Unchanged Sections >>>

# 7	Channel models

The dynamic channel models, UMidyn and UMadyn, in this document assume the UE noise limit condition, i.e., UE throughput is characterized as a function of signal power, and take the full UMa/UMi path loss model of [6] into account.

<<< Skip Unchanged Sections >>>

# 8	Channel model validation

## 8.1	General

<Editor’s note: Additional frequencies for validation measurements are not precluded.>

This clause describes the MIMO OTA validation measurements required to ensure that the test conditions are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the designated test zone.

The channel model validations defined in this Clause apply only to the channel models defined in this document, i.e., the channel models with full path loss model [6] and the UE noise limited environment.

The following validation measurements shall be performed at 2450 MHz:

-	Path Loss (PL)

-	Power Delay Profile (PDP)

-	Doppler/Temporal correlation (TCF)

-	Spatial Correlation (SCF)

-	Cross-polarization (XPO)

The target values of validation items defined in following clauses are obtained computationally from propagation parameters of Annex A using the specified antennas. The reference channel model is ideal and primarily does not consider multi-probe OTA limitations. The only exception is the LOS path, which is quantized to the closest probe of MPAC configuration as defined in Clause 7.1. This quantization affects the target statistical values of spatial correlation in Clause 8.2.5.

## 8.2	Validation of the dynamic channel models

### 8.2.1	Measurement Setup

The measurement setup includes the following equipment listed in Table 8.2.1-1.

Table 8.2.1-1: Required Measurement Equipment

| Item | Quantity | Item | PL | PDP | TCF | SCF | XPO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Channel Emulator (CE) | x | x | x | x | x |
| 2 | 1 | Signal Generator (SG) (Optional, Note 1-2) | x | (x) | x | (x) | x |
| 3 | 1 | Signal Analyser (SAN) | x | (x) | x | (x) | x |
| 4 | 1 | Vector Network Analyser (VNA) (Note 3) | - | (x) | - | (x) | - |
| 5 | 1 | Dipole | x | x | x | x | x |
| 6 | 1 | Loop | - | - | - | - | x |
| Note 1. VNA can typically generate CW. Note 2. Some gNB emulators can generate NR/AWGN signals in non-signaling mode. Note 3. Frequency domain method uses VNA and time domain method SG + SAN. |  |  |  |  |  |  |  |

#### 8.2.1.1	Network Analyser (VNA) Setup

Figure 8.2.1.1-1 shows a typical network analyser setup for channel model validation purposes.

Figure 8.2.1.1-1: Setup for Anechoic Chamber VNA Measurements

#### 8.2.1.2	Signal Analyser (SAN) Setup

Figure 8.2.1.2-1 shows a typical signal generator and signal analyser setup for channel model validation purposes. Depending on CE implementation, the trigger direction between SAN and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

Figure 8.2.1.2-1: Setup for Signal Analyser Measurements

### 8.2.2	Validation of dynamic path loss

The intention of this validation measurement is to capture the emulated dynamic path loss (PL) as it is realized in the centre of the test zone. Measurement antenna is a vertical dipole.

#### 8.2.2.1	PL Method of Measurement

The measurement antenna is placed in the centre of the test zone. Time domain technique depicted in Figure 8.2.1.2-1 is used. Table 8.2.2.1-1 outlines the settings for the channel emulator, signal generator, and signal analyser respectively.

The time domain technique (time sweep) is used for the validation. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator, i.e., two orthogonally polarized co-located gNB antennas. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal generator and signal analyser settings are listed in Tables 8.2.2.1-2 and 8.2.2.1-3, respectively. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model.

Table 8.2.2.1-1: MPAC Dynamic Channel Model Specification

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Channel emulator mode | N/A | Triggered start and stop |
| Channel model |  | As specified in Clause 7.1 |
| Mobile speed | km/h | Dynamic, as specified in Clause 7.1 |

Table 8.2.2.1-2: MPAC Path Loss Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.2.1-3: MPAC Path Loss Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The channel emulator should issue a trigger signal when the channel model is started. When the channel model is run to the end, another trigger signal is issued. A time domain trace with the SAN is collected. Data recording is synchronized with the channel emulator trigger.

#### 8.2.2.2	PL Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

#### 8.2.2.3	PL Measurement Results Analysis

The measured channel gains (inverse of path loss) are averaged over time segments and normalized such that the maximum value across all segments is 0 dB. The resulting normalized channel gains are the target values.

Lengths of time segment and time increment between segments in seconds are specified separately for both models. Example values for start and end time of UMa route segments are given in Table 8.2.2.3-1 and of the UMi route segments are given in Table 8.2.2.3-2.

Table 8.2.2.3-1: Start and end times of time segments for UMa route

| Segment number | Start time [s] | End time [s] |
| --- | --- | --- |
| 1 | 1 | 5 |
| 2 | 10 | 15 |
| 3 | 21 | 42 |
| 4 | 47 | 52 |
| 5 | 62 | 67 |
| 6 | 82 | 87 |
| 7 | 90 | 95 |
| 8 | 102 | 107 |
| 9 | 115 | 120 |

Table 8.2.2.3-2: Start and end times of time segments for UMi route

| Segment number | Start time [s] | End time [s] |
| --- | --- | --- |
| 1 | 1 | 5 |
| 2 | 10 | 20 |
| 3 | 30 | 40 |
| 4 | 44 | 54 |
| 5 | 60 | 70 |
| 6 | 77 | 82 |
| 7 | 90 | 95 |

#### 8.2.2.4	PL Target Values

The target values for the UMa route are specified in Table 8.2.2.4-1 and illustrated in Figure 8.2.2.4-1 while the target values for the UMi route are specified in Table 8.2.2.4-2 and illustrated in Figure 8.2.2.4-2.

Table 8.2.2.4-1: UMadyn Dynamic Path Gain (Path Loss) Target Values for the Measured Normalized Channel Gain for the UMa Route with full path loss model [6]

| Segment # | UMadyn PL Target [dB] |
| --- | --- |
| 1 | [-0.8] |
| 2 | [0] |
| 3 | [-29.5] |
| 4 | [-27.6] |
| 5 | [-21.5] |
| 6 | [-30.1] |
| 7 | [-0.1] |
| 8 | [-19.8] |
| 9 | [-0.7] |

Figure 8.2.2.4-1: UMadyn Dynamic Path Gain (Path Loss) Targets for the Measured Normalized Channel Gain for the UMa Route with full path loss model [6]

Table 8.2.2.4-2: UMidyn Dynamic Path Gain (Path Loss) Target Values for the Measured Normalized Channel Gain for the UMi Route with full path loss model [6]

| Segment # | UMidyn PL Target [dB] |
| --- | --- |
| 1 | [-7.5] |
| 2 | [0] |
| 3 | [-16.6] |
| 4 | [-12.9] |
| 5 | [-16.5] |
| 6 | [-0.9] |
| 7 | [-0.7] |

Figure 8.2.2.4-2: UMidyn Dynamic Path Gain (Path Loss) Targets for the Measured Normalized Channel Gain for the UMi Route with full path loss model [6]

### 8.2.3	Validation of Power Delay Profile (PDP)

The purpose of this item is to validate the slow variation of power delay profile as observed by the DUT due to the dynamic channel model. PDP can be measured either by using the frequency sweep method and a network analyser or alternatively the time domain method with wideband signal transmitted through the test system. In both options the test signal is received by a test antenna in the test zone. The variation of PDP in the reference channel model is caused by the change of path delays and powers along the route. There are two categories of channel segments (defined in Tables 8.2.2.3-1 and 8.2.2.3-2) in the UMadyn and UMidyn models. Firstly, those with same CDL model throughout the segment, meaning that only the initial delay changes during the segment but excess delays remain unchanged, i.e., quasi time invariant. Secondly, some segments have different CDL models on preceding and subsequent way-points, meaning that both the initial delay and excess delays are time variant during the segment. Channel segments with quasi time invariant and time variant excess delays are validated differently.

In the UMadyn model the channel segments 1, 2, 6, 7, and 9 are in the first category and segments 3, 4, 5, and 8 in the second. In the UMidyn model, the channel segments 2, 6, and 7 are in the first category and segments 1, 3, 4, and 5 in the second.

The validation is done by observing the measured channel impulse responses and averaging them to a PDP estimate. PDP can be evaluated either by using the frequency sweep method and a network analyser or alternatively the time domain method with wideband signal transmitted through the test system. In both options the test signal is received by a test antenna in the test zone.

#### 8.2.3.1	PDP Method of Measurement using Frequency-Domain Technique

A network analyser transmits frequency tones through the test system when the frequency sweep technique is used in the validation. See Figure 8.2.1.1-1 for the block diagram. The signal is received by a test antenna within the test area. During each frequency sweep the channel model is paused by the channel emulator. After the sweep, channel model is stepped to the next time instant and paused again for the next frequency sweep. The time increment between consecutive frequency sweeps measuring the frequency responses of channel model instances is defined in Table 8.2.3.1-1. It is at most the inverse of twice the maximum Doppler frequency, i.e., Δt≤1(2fmax).

Channel model segments are specified in Tables 8.2.2.3-1 and 8.2.2.3-2. At the first time instant of a segment the model is paused and the complex channel frequency response is measured and stored. This is repeated with the specified time increment until the whole time segment is covered. The sequence of measured channel responses of the segment is analysed as defined in clause 8.2.3-4. The subsequent channel segments are measured then correspondingly. Network analyser settings are defined in Table 8.2.3.1-1. The measured channel frequency response of channel model segment s at frequency f and channel model time instant t is

Hsf,t=Hsf0+nΔf,ts,0+m Δt

where f0 is the lowest frequency point, n=0,1,…,N-1 is the index of frequency point, Δf is the spacing between frequency points, ts,0 is the first time instant of segment s, m=0,1,…,Ms-1 is the index of time instants, and Δt is the increment between measured time instants.

Table 8.2.3.1-1: MPAC PDP Network Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 200 |
| Number of frequency points | - | ≥1101 |
| Number of traces | - | One per every CIR within the segment |

#### 8.2.3.2	PDP Method of Measurement using Time-Domain Technique

FFS

#### 8.2.3.3	PDP Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

#### 8.2.3.4	PDP Measurement Results Analysis using Frequency-Domain Technique

The first operation is to remove the quasi time-variant initial delay from measured responses Hsf,t. The initial delay τit at each time instant t is calculated by dividing the instantaneous link distance with the speed of light (in vacuum). It is removed from each frequency response by time shifting in frequency domain and a new delay shifted response is

H'sf,t=Hsf,texpj2πfτit.

These responses are then transformed to impulse responses

hsτ,t=hsuΔτ,t

By the inverse Fourier transformation, where Δτ= 5 ns is the width of a delay bin and u=0,1,2,… is the index of the delay bin. Note that due to different cable lengths, calibration etc. of test setups the resulting measured PDPs must be further shifted such that the first path component of PDPs is set on zero excess delay. This can be done either by visual observation or by correlating the resulting dynamic PDPs with theoretical reference PDPs and choosing the delay shift yielding best correlation.

Segments with quasi time invariant excess delays for UMa route: 1, 2, 6, 7, and 9 and for UMi route: 2, 6, and 7:

Power delay profile for each segment is determined by the time average

P'sτ=1Msm=0Ms-1hsτ,ts,0+m Δt2.

Finally, the normalized PDP is

Psτ=P'sτmaxP'sτ.

All values of Psτ below −40 dB are neglected in validation. Peaks, i.e., local maxima of PsτdB (in dB scale) are identified and chosen as the target value of the time segment.

Segments with time variant excess delays for UMa are 3, 4, 5, and 8 and for UMi are 1, 3, 4, and 5:

These validation segment must be treated differently since straightforward averaging of the power per delay bin over the time domain would provide erroneous result. Therefore, PDPs are evaluated per cluster (tap) index not per delay bin in the case of time variant excess delays. The procedure is as follows. First, the excess delay at the first τlts,0 and the last τlts,0+ΔtMs-1 measured time instant of the segment is determined from time interpolated model parameters for each cluster l. The excess delay of lth cluster is linearly interpolated for each time sample of the measurement and then quantized to the delay resolution Δτ supported by the measurement. The resulting quantized cluster excess delay at mth time sample is τ'l,m. Then dynamic powers of l=1,…, Ls clusters on segment s are calculated as the sum of powers over time samples in delay bins determined by τ'l,m. Since cluster powers are dispersed across neighbouring delay bins due to limited measurement bandwidth, also the previous and the successive delay bin are included in the summation. This is expressed by the formula

P's,l=m=0Ms-1hsτ'l,m-Δτ ,ts,0+m Δt2+hsτ'l,m ,ts,0+m Δt2+hsτ'l,m+Δτ,ts,0+m Δt2.

Finally, the measured power per cluster is normalized such that the maximum cluster power per segment becomes unity

Ps,l=P's,lmaxP's,1, …,P's,Ls .

#### 8.2.3.5	PDP Measurement Results Analysis using Time-Domain Technique

FFS

#### 8.2.3.6	PDP Target Values

The target values for the UMa route are specified in Table 8.2.3.6-1 and illustrated in Figure 8.2.3.6-1 which uses the time segments defined in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.3.6-2 and illustrated in Figure 8.2.3.6-2 which uses the time segments defined in Table 8.2.2.3-2. Note that red lines in the figures denote the power per excess delay and blue lines the power per cluster index. Entries of the table contain either discrete excess delay/power pairs or cluster index/power pairs, depending on the segment type. In the case of segments with time variant excess delays the initial and final excess delays of clusters must be as specified in the model parameter tables in Annex A and rounded to the delay resolution supported by the measurement bandwidth (1/200 MHz = 5 ns).

Figure 8.2.3.6-1 Target PDP Segments of the UMadyn Channel Model with full path loss model [6]

Table 8.2.3.6-1 UMadyn PDP Targets of the UMa Route with full path loss model [6]

| Segment # |  | UMadyn Target PDP |
| --- | --- | --- |
| 1 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P1τ [dB] | [0, -18.5, -20.2, -22.5, -36.0, -25.5, -31.9] |
| 2 | Delay τ [ns] | [0, 130, 170, 245, 380, 745, 885] |
|  | Power P2τ [dB] | [0, -19.2, -20.9, -23.2, -32.8, -23.9, -34.5] |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | [-9.8, 0.0, -1.9, -6.3, -8.2, -6.6, -3.3, -8.9, -8.3, -11.7, -8.5, -16.3, -10.0, -13.4, -12.1, -12.8, -29.3, -29.0, -18.4, -19.3, -17.5, -31.9, -29.8] |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | [-15.0, 0.0, -2.0, -9.7, -11.8, -14.2, -3.9, -13.3, -6.9, -20.8, -6.5, -16.0, -19.0, -17.5, -12.3, -11.3, -28.2, -29.1, -22.3, -18.2, -17.2, -30.3, -28.0] |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | [-8.2, -0.8, -3.7, -3.5, -5.2, 0.0, -1.6, -3.7, -10.4, -10.1, -13.4, -22.5, -7.9, -13.5, -16.2, -18.9, -23.1, -21.4, -23.4, -22.5, -20.3, -26.5, -33.1] |
| 6 | Delay τ [ns] | [0, 80, 235, 300, 340, 445, 475, 790, 985, 1550, 1675, 2000, 2040, 2295, 2415, 2565] |
|  | Power P6τ [dB] | [-13.0, -3.3, 0.0, -12.6, -33.3, -15.8, -19.7, -25.1, -18.0, -17.8, -16.4, -39.6, -20.4, -19.0, -20.7, -29.4] |
| 7 | Delay τ [ns] | [0, 50, 65, 175, 240, 335, 495, 1090, 1875] |
|  | Power P7τ [dB] | [0, -17.1, -21.2, -15.0, -21.0, -24.9, -19.9, -32.8, -32.8] |
| 8 | Cluster index l | 1, 2, ..., 23 |
|  | Power P8,l [dB] | [0.0, -5.7, -7.9, -11.5, -15.2, -15.4, -9.1, -14.1, -14.0, -19.6, -12.3, -19.9, -25.2, -20.9, -26.6, -27.9, -23.1, -25.6, -26.3, -33.4, -35.9, -27.5, -38.8] |
| 9 | Delay τ [ns] | [0, 60, 130, 170, 245, 380, 745, 885, 1180] |
|  | Power P9τ [dB] | [0, -36.1, -17.9, -19.5, -22.0, -38.8, -27.0, -29.5, -38.7] |

Figure 8.2.3.6-2 Target PDP Segments of the UMidyn Channel Model with full path loss model [6]

Table 8.2.3.6-2 UMIdyn PDP Targets of the UMi Route with full path loss model [6]

| Segment # |  | UMidyn Target PDP |
| --- | --- | --- |
| 1 | Cluster index l | 1, 2, ...11, 13, …, 18 |
|  | Power P1,l [dB] | [0.0, -21.9, -22.5, -24.3, -16.8, -16.7, -16.5, -18.3, -25.5, -24.5, -29.6, -24.0, -30.0, -34.7, -40.0, -39.9, -37.8] |
| 2 | Delay τ [ns] | [0, 35, 75, 100, 140, 430, 510, 680] |
|  | Power P2τ [dB] | [0.0, -35.0, -17.4, -18.8, -21.5, -28.8, -28.2, -37.5] |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | [-17.1, 0.0, -2.8, -2.8, -4.8, -1.6, -2.0, -4.4, -5.9, -9.0, -6.1, -26.4, -7.5, -21.5, -20.3, -10.7, -15.2, -16.2, -16.8, -16.8, -15.5, -19.2, -23.6] |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | [-11.3, 0.0, -2.5, -4.3, -6.1, -2.0, -2.0, -5.2, -7.8, -11.5, -8.4, -21.8, -9.3, -15.9, -16.6, -13.6, -21.0, -20.8, -19.5, -19.3, -18.0, -24.4, -27.7] |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | [-11.0, 0.0, -2.0, -5.6, -7.5, -5.9, -3.1, -8.2, -8.1, -11.2, -8.4, -17.6, -9.4, -14.4, -13.3, -12.5, -27.1, -26.7, -17.5, -18.8, -16.6, -30.1, -28.9] |
| 6 | Delay τ [ns] | [0, 30, 100, 140, 195, 285] |
|  | Power P6τ [dB] | [0.0, -20.5, -17.4, -25.9, -33.6, -20.0] |
| 7 | Delay τ [ns] | [0, 30, 100, 140, 195, 285] |
|  | Power P7τ [dB] | [0.0, -20.9, -16.5, -24.2, -30.6, -20.0] |

### 8.2.4	Validation of Doppler/Temporal Correlation

The purpose of this item is to validate the slow variation of Doppler effect due of the dynamic channel model. This is done indirectly by observing the temporal correlation function (TCF). TCF at a few time lags are evaluated from the measured narrowband signal transmitted through the test system.

#### 8.2.4.1	TCF Method of Measurement

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal analyser and signal generator settings are defined in Tables 8.2.4.1-1 and 8.2.4.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model.

Table 8.2.4.1-1: MPAC TCF Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.4.1-2: MPAC TCF Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The signal analyser records I/Q samples at the rate of 15 times the maximum Doppler frequency (72.24 Hz). The full model length is measured at once.

#### 8.2.4.2	TCF Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

#### 8.2.4.3	TCF Measurement Results Analysis

Segments of recorded I/Q samples are selected, autocorrelation of I/Q samples of a segment is calculated, and a few samples of the resulting temporal correlation function are picked for the validation. Time lags to be picked for TFC Δt are 4.6 ms and 20.2 ms.

#### 8.2.4.5	TCF Target Values

The target values for the UMa route are specified in Table 8.2.4.5-1 and illustrated in Figure 8.2.4.5-1 which uses the time segments defined in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.4.5-2 and illustrated in Figure 8.2.4.5-2 which uses the time segments defined in Table 8.2.2.3-2. Estimated temporal correlation values at time lag Δt=4.6 ms and Δt=20.2 ms are illustrated in the top and bottom figure, respectively. Target values are shown within time segment limits.

Figure 8.2.4.5-1: UMadyn Temporal Correlation Function of UMa Route for Two Different TFC Time Lags

Table 8.2.4.5-1: Dynamic UMadyn Temporal Correlation Targets of the UMa Route with full path loss model [6]

| Segment # | UMadyn Target TCF at Δt=4.6 ms | UMadyn Target TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | [0.970] | [0.876] |
| 2 | [0.961] | [0.879] |
| 3 | [0.994] | [0.886] |
| 4 | [0.436] | [0.355] |
| 5 | [0.648] | [0.345] |
| 6 | [0.523] | [0.100] |
| 7 | [0.801] | [0.228] |
| 8 | [0.837] | [0.399] |
| 9 | [0.940] | [0.616] |

Figure 8.2.4.5-2: UMidyn Temporal Correlation Function of UMi Route for Two Different TFC Time Lags

Table 8.2.4.5-2: Dynamic UMidyn Temporal Correlation Targets of the UMi Route with full path loss model [6]

| Segment # | Target UMidyn TCF at Δt=4.6 ms | Target UMidyn TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | [0.871] | [0.437] |
| 2 | [0.952] | [0.857] |
| 3 | [0.514] | [0.195] |
| 4 | [0.698] | [0.071] |
| 5 | [0.743] | [0.372] |
| 6 | [0.959] | [0.814] |
| 7 | [0.965] | [0.854] |

### 8.2.5	Validation of Spatial Correlation

The purpose of this item is to validate the slow variation of angular power distribution as observed by the DUT due of the dynamic channel model. The variation of PAS in the reference channel model is caused by the change of path angles, path powers, and UE orientation along the route. The validation is done indirectly by observing the spatial correlation function (SCF). SCF is evaluated from the measured narrowband signal transmitted through the test system and received by a test antenna in a few spatial positions within the test zone.

#### 8.2.5.1	SCF Method of Measurement

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator. The signal is received by a test antenna in a specific position within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal analyser and signal generator settings are defined in Tables 4.2.3.1-1 and 4.2.3.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model. The position of test antenna is changed and the measurement is repeated. All spatial positions are illustrated in Figure 8.2.5.1-1.

Table 8.2.5.1-1: MPAC SCF Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.5.1-2: MPAC SCF Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The full model length is measured at once.

Figure 8.2.5.1-1: Spatial sampling points within the test zone at 2450 MHz

Table 8.2.5.1-3: Spatial sample points, i.e., positions of test antenna for the SCF validation at 2450 MHz

| Point number | x [mm] | y [mm] | z [mm] |
| --- | --- | --- | --- |
| #1 (reference point) | 0 | -150 | 0 |
| #2 | -22.88 | -148.29 | 0 |
| #3 | -86.27 | -122.71 | 0 |
| #4 | -149.90 | -5.73 | 0 |

#### 8.2.5.2	SCF Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole

#### 8.2.5.3	SCF Measurement Results Analysis

Time segments of recorded I/Q samples are selected. For each time segment the cross correlation (with zero time lag) of I/Q samples measured in different spatial sample points is calculated. Spatial sample points picked for SFC have at maximum Euclidian distance Dd = 1.7 wavelength to the reference sample point. Absolute values of estimated complex spatial correlations per time segment are chosen as the target SCF values.

#### 8.2.5.4	Target Values

The target values for the UMa route are specified in Table 8.2.5.4-1 and illustrated in Figure 8.2.5.4-1, which uses the time segments along the dynamic UMadyn model proposed in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.5.4-2 and illustrated in Figure 8.2.5.4-2, which uses the time segments along the dynamic UMidyn model proposed in Table 8.2.2.3-2. Estimated spatial correlation values at spatial spacing Δd=22.9 mm, Δd=90.5 mm, and Δd=208.0 mm (referenced to reference point #1) are illustrated in Figures 8.2.5.4-1 and 8.2.5.4-2. Target values are shown within time segment limits.Figure 8.2.5.4-1: UMadyn Spatial Correlation Function of UMa Route with full path loss model [6] for Three Different SFC Test Antenna Positions

Table 8.2.5.4-1: UMadyn Spatial Correlation Targets of the UMa Route with full path loss model [6] for 2450 MHz

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | [0.986] | [0.908] | [0.932] |
| 2 | [0.963] | [0.611] | [0.326] |
| 3 | [0.800] | [0.160] | [0.111] |
| 4 | [0.699] | [0.721] | [0.492] |
| 5 | [0.831] | [0.414] | [0.156] |
| 6 | [0.794] | [0.393] | [0.454] |
| 7 | [0.928] | [0.481] | [0.367] |
| 8 | [0.834] | [0.283] | [0.605] |
| 9 | [0.959] | [0.574] | [0.220] |

Figure 8.2.5.4-2: UMidyn Spatial Correlation Function of UMi Route with full path loss model [6] for Three Different SFC Test Antenna Positions

Table 8.2.5.4-2: UMidyn Spatial Correlation Targets of the UMi Route with full path loss model [6] for 2450 MHz

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | [0.944] | [0.758] | [0.733] |
| 2 | [0.981] | [0.937] | [0.939] |
| 3 | [0.787] | [0.228] | [0.153] |
| 4 | [0.764] | [0.328] | [0.314] |
| 5 | [0.856] | [0.572] | [0.286] |
| 6 | [0.973] | [0.809] | [0.685] |
| 7 | [0.982] | [0.950] | [0.943] |

### 8.2.6	Validation of Cross-Polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values of cross polarization (XPO).

#### 8.2.6.1	XPO Method of Measurement

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator, i.e., that correspond to two co-located ±45° slanted linearly polarized elements. The signal is received by a vertically polarized test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal generator and signal analyser settings are listed in Tables 8.2.6.1-1 and 8.2.6.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model. The measurement is repeated with a horizontally polarized test antenna, placed in the same position. The result is a stored sequence of channel gains of segment s received with the vertically and horizontally polarized test antennas are denoted hV,st=hV,sts,0+m Δt and hH,st=hH,sts,0+m Δt, respectively, where m=0,…,Ms Note that the time increment Δt and correspondingly the number of stored time samples is different in this measurement as compared to the PDP measurement.

Table 8.2.6.1-1: MPAC path loss Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.6.1-2: MPAC path loss Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

#### 8.2.6.2	XPO Measurement Antenna

Measurement antennas shall be a vertically-oriented dipole and a loop.

#### 8.2.6.3	XPO Measurement Results Analysis

The measured channel gains (inverse of path loss) are averaged over each time segment s to obtain average powers

PV,s=1Msm=0Ms-1hV,sts,0+m Δt2PH,s=1Msm=0Ms-1hH,sts,0+m Δt2.

The resulting V/H power ratios

Xs=PV,sPH,s

of time segments s are the target values.

#### 8.2.6.4	XPO Target Values

The target values for the UMa route are specified in Table 8.2.6.4-1 and illustrated in Figure 8.2.6.4-1 which uses the time segments defined in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.6.4-2 and illustrated in Figure 8.2.6.4-2 which uses the time segments defined in Table 8.2.2.3-2.

Figure 8.2.6.4-1: Simulated Dynamic Narrowband UMadyn Polarization Power Ratios with Time Segment Limits for the UMa Route with full path loss model [6]

Table 8.2.6.4-1: UMadyn Polarization Power Ratio Target Values for the Measured Channel Gain with V and H Polarized Test Antenna for the UMa Route with full path loss model [6]

| Segment # | UMadyn XPO Target [dB] |
| --- | --- |
| 1 | [22.2] |
| 2 | [22.4] |
| 3 | [8.5] |
| 4 | [9.5] |
| 5 | [7.4] |
| 6 | [7.0] |
| 7 | [18.5] |
| 8 | [12.2] |
| 9 | [22.0] |

Figure 8.2.6.4-2: Simulated Dynamic Narrowband UMidyn Polarization Power Ratios with Time Segment Limits for the UMi Route with full path loss model [6]

Table 8.2.6.4-2: UMidyn Polarization Power Ratio Target Values for the Measured Channel Gain with V and H Polarized Test Antenna for the UMi Route with full path loss model [6]

| Segment # | UMidyn XPO Target [dB] |
| --- | --- |
| 1 | [18] |
| 2 | [21.7] |
| 3 | [8.6] |
| 4 | [8.1] |
| 5 | [8.5] |
| 6 | [21.1] |
| 7 | [20.7] |

<<< END OF CHANGES >>>