3GPP TSG-RAN WG4 Meeting #114bis	R4-2504735

Wuhan, China, April 7th – April 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 with updates for EEIRP requirement conformance testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is CR to TS 38.141-2 with updates to test requirement and test procedure part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates in test procedure section. Updates of test requirement section, updates for mechanical down-tilt part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be missleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2504435 |  |  |  |  |  |  |  |  |

<Start of changes>

  - 6.9	OTA spatial emission

### 6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.9.2	Minimum requirement

The minimum requirement for BS type 1-H and BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

### 6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

### 6.9.4	Method of test

#### 6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level with the corresponding BW.

In the test range, the EUT is mounted at the positioner with mechanical down-tilt set to zero.

#### 6.9.4.2	Procedure

When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

The test range shall be calibrated according to calibration method described in TR 37.941, clause 8.3.

1. Place the BS at the positioner, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS with the coordinate system used by the test system.

3. The measurement device characteristics shall be: Detection mode set to True RMS.

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2.

5. Orient the positioner (and the BS) to the angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex N.3.

6.  Configure test beam(s) and test equipment for measuring test beam k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

8. Repeat step 6 and 7 for all K test beams.

9. Repeat step 5, 6, 7 and 8 for all M and N positioner angles.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for bins i=1..7 as described in  Annex N.

In the test procedure the order of Step 8 and Step 9 may be exchangeable.

<End of changes>