3GPP TSG-RAN WG4 Meeting #114bis   											             R4-2504742

Wuhan, China, April 7 - 11, 2025

Agenda item:			7.26.5

Source:	Huawei

Title:	WF on NR LP-WUS BS RF

Document for:	Approval

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS

- WF

  - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth.

  - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB.

    - X can be different for different channel bandwidth.

    - FFS on the value of X and use 3 as starting point.

Issue 1-2: How to comprehend the de-boost on the NR transmission power

- Proposals

  - How to attend the required power for LP-WUS power boosting should be treated as implementation issue and thus there is no need to reflect it in the specification. (Nokia)

- Agreement in AH

  - There is no need to reflect NR power de-boosting in the specification.

Issue 1-3: Whether to define EVM requirements for LP-WUS

- Options

  - Option 1: No need to define EVM requirements for LP-WUS. (Huawei)

    - Because simulation results show that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirements do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. (Nokia)

    - Because frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR. (ZTE)

  - Option 2: EVM requirements for LP-WUS is needed. (CMCC)

    - Option 2-1 (CATT):

      - RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

      - RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

      - RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

      - RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

    - Option 2-2 (Ericsson):

      - Define the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS as 8%.

      - Wait further RAN1 progress on PAPR discussion.

- WF

  - Further check in next meeting whether EVM for LP-WUS needs to be specified.

Issue 1-4: Whether/how to verify EVM requirements for LP-WUS

- Options

  - Option 1: There is no need to do the EVM verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS. (Nokia)

  - Option 2: Without specific EVM requirement defined for LP-WUS, just use a higher-order EVM test for NR with embedded LP-WUS as the verification metric. (Huawei)

  - Option 3: The OOK4 signal quality test can be dependent on the 64QAM EVM test. (Ericsson)

  - Option 4: Others.

- WF

  - Consider together with EVM requirement for LP-WUS

    - If no EVM requirement is defined, no verification is needed

    - If EVM requirement is defined, FFS on whether verification is needed.

Issue 1-5: Submitted TP

- Proposals

  - Proposal 1: R4-2503637 TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2). (ZTE)

  - Proposal 2: R4-2504392 TP on TX signal quality (clause 7.2.4, Ericsson)

- WF

  - Postpone R4-2504392 to next meeting

Work split for LP-WUS TR/TS BS part

- Proposals

  - TPs for TR 38.774 BS part, and draft CRs for TS 38.104/38.141-1 are split among companies as below:

| TPs for TR 38.774 | Volunteer companies |
| --- | --- |
| 7.2.1 General | ZTE |
| 7.2.2 LP-WUS power boosting | ZTE |
| 7.2.3 Regulation relevant spectrum requirements | Already done |
| 7.2.4 Transmitted signal quality | CATT |
| 7.2.5 Other RF requirements |  |
| Draft CR for TS 38.104 | - Power boosting definition: Huawei - Transmitter signal quality (if needed): |
| Draft CR for TS 38.141-1 | - Manufacturer declaration: CATT - Transmitter signal quality (if needed): |