# 3GPP TSG-RAN WG4 Meeting #114bis                            R4-2504743

# Wuhan, China, 7th-11th Apr, 2025

# Source:        ZTE Corporation, Sanechips, Ericsson, Huawei, Nokia, CATT

# Title:           TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2)

# Agenda Item:   7.26.3

# Document for:  Approval

- Introduction

According to the discussion in previous RAN4 meetings, we provided a text proposal for TR 38.774 on Clause 7.2.2 for LP-WUS power boosting [1][2][3].

- Reference

- R4-2502305 Way Forward for [114][308] NR_LPWUS, RAN4#114, Athens, Greece, Feb. 2025.

- R4-2419802 WF for [113][308] NR_LPWUS, RAN4#113, Orlando, USA, Nov. 2024.

- R4-2416589 Way Forward for [112bis][308] NR_LPWUS, RAN4#112bis, Hefei, China, Oct. 2024.

- Text Proposal

<Start of Text Proposal>

### 7.2.2	LP-WUS power boosting

- Two approaches to define the LP-WUS power boosting are discussed, e.g. legacy dynamic range similar to NB-IoT power boosting and EPRE ratio. The definitions for these two approaches are:

For legacy dynamic range similar to NB-IoT power boosting, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

For EPRE ratio, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over NR REs (the NR carrier excluding the LP-WUS REs).

The above two approaches are mathematically related, and the specific analysis can be found in R4-2407547 and R4-2419482.

RAN4 decides to adopt legacy dynamic range similar to NB-IoT power boosting. The declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. The minimum power boosting level should be included in the BS manufacturer declaration table, but not defined as a minimum requirement.

<End of Text Proposal>