3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504746

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Way Forward for [114bis][306] NR_duplex_evo_General

Document for:	Approval

# Topic #1: General issues

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

    - FFS the value of X, e.g., X = 33dBm as candidate value, and study the value for different number of TX chains.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as candidate value, and study the value for different number of TX chains.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS only, as the starting point

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as candidate value, and study the value for different number of TX chains.

##### Issue 1-2-3: Restriction on guardband size for SBFD

Agreement:

- Restriction on guardband size for SBFD:

  - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband.

    - The declaration of guardband size should ensure the benefits from SBFD operation.

      - This sub-bullet shall be reflected in the specification, and detailed wording can be FFS.

##### Issue 1-2-4: Restriction on DL subband size

Agreement:

- If the SSB symbol is not overlapped with SBFD symbol,

  - There is no restriction on DL subband size.

- If the SSB symbol is overlapped with SBFD symbol,

  - FFS the impact from RAN4 perspective.

##### Issue 1-2-5: FR2-1 CBW support for SBFD DUD case

Agreement:

- If the SSB symbol is not overlapped with SBFD symbol,

  - 100MHz for DUD case is allowed for FR2-1.

- If the SSB symbol is overlapped with SBFD symbol,

  - In Rel-19, the BS channel bandwidth for DUD configuration shall be no less than 200MHz for FR2-1.

### Sub-topic 1-3: Requirement applicability for SBFD

##### Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

Agreement:

- The following RAN4 requirement set applicability for SBFD-capable BS is agreed:

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.9 | 12.5.9 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

### Sub-topic 1-4: BS RF specification structure for SBFD requirements

##### Issue 1-4-1: BS RF specification structure for SBFD requirements

Agreement:

- The following clause structure is adopted for Chapter 12 for requirements for SBFD-capable BS:

12	Requirements for SBFD-capable BS	2

12.1	General	2

12.2	Conducted transmitter characteristics for SBFD-capable BS	3

12.2.1	General	3

12.2.2	Base station output power for SBFD	3

12.2.3	Output power dynamics for SBFD	3

12.2.4	Transmit ON/OFF power for SBFD	4

12.2.5	Transmitted signal quality for SBFD	4

12.2.6	Unwanted emissions for SBFD	4

12.2.7	Transmitter intermodulation for SBFD	4

12.2.8	Transient period between SBFD and non-SBFD	5

12.2.9	In-channel adjacent subband leakage for SBFD	5

12.3	Conducted receiver characteristics for SBFD-capable BS	5

12.3.1	General	5

12.3.2	Reference sensitivity level for SBFD	5

12.3.3	Dynamic range for SBFD	5

12.3.4	In-band selectivity and blocking for SBFD	5

12.3.5	Out-of-band blocking for SBFD	5

12.3.6	Receiver spurious emissions for SBFD	5

12.3.7	Receiver intermodulation for SBFD	5

12.3.8	In-channel selectivity for SBFD	5

12.3.9	In-channel adjacent subband blocking for SBFD	5

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS	5

12.5.1	General	5

12.5.2	Radiated transmit power for SBFD	5

12.5.3	OTA base station output power for SBFD	5

12.5.4	OTA output power dynamics for SBFD	5

12.5.5	OTA transmit ON/OFF power for SBFD	5

12.5.5.1	General	5

12.5.5.2	OTA transmitter OFF power for SBFD	5

12.5.5.3	OTA transient period for SBFD	5

12.5.6	OTA transmitted signal quality for SBFD	5

12.5.7	OTA unwanted emissions for SBFD	6

12.5.8	OTA transmitter intermodulation for SBFD	6

12.5.9	OTA transient period between SBFD and non-SBFD	6

12.5.10	OTA in-channel adjacent subband leakage for SBFD	6

12.6	Radiated receiver characteristics for SBFD-capable BS	6

12.6.1	General	6

12.6.2	OTA sensitivity for SBFD	6

12.6.3	OTA reference sensitivity level for SBFD	6

12.6.4	OTA dynamic range for SBFD	6

12.6.5	OTA in-band selectivity and blocking for SBFD	6

12.6.6	OTA out-of-band blocking for SBFD	6

12.6.7	OTA receiver spurious emissions for SBFD	6

12.6.8	OTA receiver intermodulation for SBFD	6

12.6.9	OTA in-channel selectivity for SBFD	6

12.6.10	OTA in-channel adjacent subband blocking for SBFD	6

12.7		Radiated performance requirements for SBFD-capable BS

### Sub-topic 1-5: Draft CR and work splitting

##### Issue 1-5-1: Draft CR and work splitting

Way forward:

- The following work splitting will be followed in RAN4#115 and #116, aiming at WI completion by Sep. 2025.

  - In the below work splitting, the same requirements for conducted and OTA counterparts are assigned into the same package.

| Work splitting packages | Relevant impacted clauses in TS38.104 | Responsible company |
| --- | --- | --- |
| Package-1 | Chapter 2 References Chapter 3 Definitions, symbols and abbreviations Chapter 5 Operating bands and channel arrangement | TBA |
| Package-2 | 12.2 Conducted transmitter characteristics for SBFD-capable BS 12.2.1 General 12.2.2 Base station output power for SBFD 12.2.3 Output power dynamics for SBFD 12.5 Radiated transmitter characteristics for SBFD-capable BS 12.5.1 General 12.5.2 Radiated transmit power for SBFD 12.5.3 OTA base station output power for SBFD 12.5.4 OTA output power dynamics for SBFD | TBA |
| Package-3 | 12.2.4 Transmit ON/OFF power for SBFD 12.2.5 Transmitted signal quality for SBFD 12.5.5 OTA transmit ON/OFF power for SBFD 12.5.6 OTA transmitted signal quality for SBFD | TBA |
| Package-4 | 12.2.6 Unwanted emissions for SBFD 12.2.7 Transmitter intermodulation for SBFD 12.5.7 OTA unwanted emissions for SBFD 12.5.8 OTA transmitter intermodulation for SBFD | TBA |
| Package-5 | 12.2.8 Transient period between SBFD and non-SBFD 12.2.9 In-channel adjacent subband leakage for SBFD 12.5.9 OTA transient period between SBFD and non-SBFD 12.5.10 OTA in-channel adjacent subband leakage for SBFD | TBA |
| Package-6 | 12.3 Conducted receiver characteristics for SBFD-capable BS 12.3.1 General 12.3.2 Reference sensitivity level for SBFD 12.3.3 Dynamic range for SBFD 12.6 Radiated receiver characteristics for SBFD-capable BS 12.6.1 General 12.6.2 OTA sensitivity for SBFD 12.6.3 OTA reference sensitivity level for SBFD 12.6.4 OTA dynamic range for SBFD | TBA |
| Package-7 | 12.3.4 In-band selectivity and blocking for SBFD 12.3.5 Out-of-band blocking for SBFD 12.6.5 OTA in-band selectivity and blocking for SBFD 12.6.6 OTA out-of-band blocking for SBFD | TBA |
| Package-8 | 12.3.6 Receiver spurious emissions for SBFD 12.3.7 Receiver intermodulation for SBFD 12.6.7 OTA receiver spurious emissions for SBFD 12.6.8 OTA receiver intermodulation for SBFD | TBA |
| Package-9 | 12.3.8 In-channel selectivity for SBFD 12.3.9 In-channel adjacent subband blocking for SBFD 12.6.9 OTA in-channel selectivity for SBFD 12.6.10 OTA in-channel adjacent subband blocking for SBFD | TBA |

### Sub-topic 1-7: Update for HPUE duty cycle evaluation

##### Issue 1-7-1: Update for HPUE duty cycle evaluation

Agreement:

- Clarification of the term “UL symbols” used in TS 38.101-1 and TS38.101-2 for uplink duty cycle evaluation:

  - For non-SBFD-aware UE, the legacy UL symbols shall be used for uplink duty cycle evaluation i.e. the term “UL symbols” shall be reused;

  - For SBFD-aware UE, “UL symbols” shall include the uplink symbols in the non-SBFD symbol configuration and SBFD symbols for uplink duty cycle evaluation.

# Topic #2: PUSCH with resource muting for CLI handling

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way forward:

- RAN4 try to draft a reply LS to RAN1 based on existing results.

  - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

# Topic #3: SBFD adjacent channel coexistence

### Sub-topic 3-2: SBFD-TDD co-existence in adjacent channel

##### Issue 3-2-1: Fair coexistence between SBFD and TDD networks

Way forward:

- RAN4 can further wait RAN1 conclusion on fair co-existence between SBFD and TDD networks.

# Topic #4: New BS RF requirements for SBFD

### Sub-topic 4-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 4-1-1: Location of transient period for Case-B (from SBFD symbol to UL symbol)

Agreement:

- For Case-B, guard symbol(s) should be reserved before UL symbols

  - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation.

##### Issue 4-1-2: Location of transient period for Case-C (from DL symbol to SBFD symbol)

Agreement:

- For the location of transient periods Case-C,

  - Option 1: guard symbols reserved in non-SBFD DL symbol(s)

  - Option 2: blanked SBFD symbols in the beginning of SBFD UL transmission shall be allowed

  - Option 3: both Option 1 and Option 2 shall be allowed, up to gNB implementation.

##### Issue 4-1-3: RAN4 requirement impact

Agreement:

- For the location of transient period between SBFD and non-SBFD symbols,

  - No need to be specified in RAN4 core requirement;

  - For conformance testing for relevant requirements,

    - FFS test configurations.

##### Issue 4-1-4: Text proposal for transient period requirement applicability on SBFD subbands

Way forward:

- Text proposal for transient period requirement applicability on SBFD subbands:

  - Discuss TP for new SBFD chapter directly.

### Sub-topic 4-2: LS to RAN1 on RAN4 agreement of transient period

##### Issue 4-2-1: LS to RAN1 on RAN4 agreement of transient period

Way forward:

- RAN4 prepares a LS to RAN1 by capturing the existing agreement on transient period.

### Sub-topic 4-3: In-channel adjacent subband leakage

Agreement:

- The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”).

##### Issue 4-3-2: Assumption for in-channel adjacent subband leakage requirement

Way forward:

- FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage.

##### Issue 4-3-3: Absolute limit for in-channel adjacent subband leakage

Way forward:

- Absolute limit for in-channel adjacent subband leakage:

  - FFS by considering the following exemplary cases by measurement or simulation:

    - DU/UD: 80MHz (DL) – 20MHz (UL)

    - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL)

    - Other cases are not precluded

  - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

##### Issue 4-3-4: Necessity of in-channel adjacent subband leakage ratio

Way forward:

- No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114.

### Sub-topic 4-4: In-channel adjacent subband blocking

##### Issue 4-4-1: In-channel adjacent subband blocking requirement

Way forward:

- New requirement for in-channel adjacent subband blocking:

  - The requirement can be derived by considering co-site inter-sector case and/or inter-site case

    - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification.

  - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

##### Issue 4-4-2: Assumption of gNB-to-gNB CLI handling

Way forward:

- FFS the assumption of gNB-to-gNB CLI handling for two cases:

  - Inter-site case:

    - Note: for the blocking requirement, RX beam direction cannot be considered

  - Co-site inter-sector case:

    - Note: Inter-sector is near field interaction

- FFS sending LS to RAN1 for this.