3GPP TSG-RAN WG4 Meeting #114bis	R4-2504755

# Wuhan, China, 7th – 11th April, 2025

Title: 	WF on NR_ATG_enh demodulation requirement

Agenda Item:	7.12.7

Source: 	CMCC

Document for:	Approval

<Topic 1> DL CA

Issue 1-1-1: CA Capability

Agreement:

- CA_C

- CA_AX

Issue 1-1-2: CA Duplex mode

Agreement:

- TDD 30 kHz + TDD 30 kHz

- FDD 15 kHz + TDD 30 kHz

FFS:

- FDD 15 kHz + FDD 15 kHz

Issue 1-1-3: Test setups

Issue 1-1-3-1: TDD pattern

Agreement:

- Use legacy TDD pattern 7DS2U as the staring point

FFS:

- new TDD pattern 30D4S6U

Issue 1-1-3-2: Rank

- Proposal 1: Rank 2

- Proposal 2: Rank 1

Issue 1-1-3-3: MCS

Agreement:

- 16QAM MCS 13, 64QAM MCS 22, 256QAM MCS 24 for initial simulation

Issue 1-1-3-4: Propagation condition

Agreement:

- AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz

Issue 1-1-3-5: Antenna configuration

Agreement:

- 2T2R and 2T4R

Issue 1-1-3-6: Test metric

Agreement:

- 70% of max TP

Issue 1-1-3-7: Bandwidth

Agreement:

- all bandwidth configuration for FDD 15kHz and TDD 30kHz

- Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified.

- FFS:

- RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A.

Issue 1-1-3-8: Others

Agreement:

- For the other test parameters, the configuration in Table 5.2A-1/2/3 in TS 38.101-4 can be reused

- Issue 1-1-4: CA configuration selection rules

- Proposal 1:

  - Define the CA configuration selection rules as

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- Issue 1-1-5: Test applicability rules

- Proposal 1:

  - Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

<Topic 2> DL MIMO

Issue 2-1-1: Antenna configuration

Agreement:

- 2T2R and 2T4R

- FFS the antenna correlation:

- Proposal 1: XP-low

Issue 2-1-2: TDD pattern

Agreement:

- Use legacy TDD pattern 7DS2U as the staring point

- FFS whether the new TDD pattern 30D4S6U will be used

Issue 2-1-3: Rank

Agreement:

- Rank 2

Issue 2-1-4: MCS

Agreement:

- For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24

Issue 2-1-5: Propagation condition

Agreement:

- AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz

Issue 2-1-6: Other simulation assumptions

Agreement:

- For other simulation assumptions, the agreements from Rel-18 can be reused, including:

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Issue 2-2-1: PMI

Agreement:

- No need to introduce PMI requirement

Issue 2-2-2: RI

- Proposal 1:

  - Investigate the feasibility of introducing the RI requirement.

  - For antenna configuration and propagation channel, reuse the configuration from DL MIMO demodulation test. For other test parameters, reuse the configuration from legacy.

  - Decide the SNR and γby simulation results

<Topic 3> UL MIMO

Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO

Agreement:

- The legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 2Tx 2 layer will be applied for ATG

Issue 3-1-2: Whether to introduce new test cases for ATG UL MIMO

- Proposals

  - Proposal 1:

    - incremental PUSCH 2 layer test cases need to be introduced

  - Proposal 2:

    - Do not introduce new 2Tx PUSCH requirements for ATG scenario

Issue 3-1-3: Antenna configuration for incremental test

- Proposals

  - Proposal 1:

    - 2T2R, 2T4R and 2T8R with X-pol low correlation

  - Proposal 2:

    - 2T2R, 2T4R and 2T8R

  - Proposal 1:

    - 2T2R

Issue 3-1-4: Propagation condition for incremental test

- Proposals

  - Proposal 1:

    - For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference

Issue 3-1-5: Bandwidth & SCS for incremental test

- Proposals

  - Proposal 1:

    - 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS

Issue 3-1-6: Rank for incremental test

- Proposals

  - Proposal 1:

    - Rank 2

Issue 3-1-7: MCS for incremental test

- Proposals

  - Proposal 1:

    - 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation

  - Proposal 2:

    - consider low modulation order for high rank transmission

  - Proposal 3:

    - MCS 16

Issue 3-1-8: DMRS port for incremental test

- Proposals

  - Proposal 1:

    - DMRS port 2 {0,1}

Issue 3-1-9: TPMI for incremental test

- Proposals

  - Proposal 1:

    - TPMI index 0

Issue 3-1-10: Others

- Proposals

  - Proposal 1:

    - Type A

    - HARQ retransmission

    - 70% of maximum throughput

  - Proposal 2:

    - Others same as existing Rel-15 BS PUSCH demodulation requirement with 2Tx