3GPP TSG-RAN WG4 Meeting #114bis	R4-2504762

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Simulation assumptions for SCM

Source: 	Huawei, HiSilicon, MediaTek

Agenda item:	7.16.2

Document for:	Information

Background

This contribution summarizes the simulation assumptions for spatial channel model.

- PDSCH for alignment

Common parameters are captured in Table 2-1

Table 2-1: Common simulation assumptions for PDSCH

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | For CDL channel: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination For TDL channel: Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Cluster model 1

Simulation assumptions for cluster model 1 are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for comparison of cluster model 1

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Number of clusters | 4 | 4 |
| Cluster powers p1, p2, p3, p4 | [0dB, 0dB, -9dB, -9dB] | [0dB, 0dB, -9dB, -9dB] |
| Per cluster configuration | XPL-High, TDLC300-10 | XPL-High, TDLC300-10 |
| Cluster TX steering in 1st array dim θtx1(1),θtx1(2),θtx1(3), θtx1(4) | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim θrx1(1),θrx1(2),θrx1(3), θtx1(4) | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Cluster model 2

Simulation assumptions for cluster model 2 are captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for comparison of cluster model 2

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10 | XPL-High, TDLC300-10 |
| Cluster-tap mapping | Table 2.2-2 | Table 2.2-2 |

Table 2.2-2: Cluster model 2 with 4 TX-RX clusters

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θtx1(2)=-π/2 θrx1(2)=-π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θtx1(3)=-π θrx1(3)=-π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θtx1(4)=-3π/2 θrx1(4)=-3π/2 |
| 12 | 2595 | -16.0 |  |

Cluster model 3

Simulation assumptions for cluster model 3 are captured in Table 2.3-1:

Table 2.3-1: Simulation assumptions for comparison of cluster model 3

| Parameters | Values |
| --- | --- |
| Rank | 8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,4,1) |
| Precoder for PDSCH and DMRS | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Channel powers p1, p2, | p1, p2=[0dB, -9dB] |
| Cluster-tap mapping | Table 2.3-2 |

Table 2.3-2: Cluster model 3 with 4 clusters

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam for TDL channel 1 p1 | Tx-Rx Beam for TDL channel 2 p2 |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 | Cluster 3 θtx1(3)=-π θrx1(3)=-π |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Cluster 2 θtx1(2)=-π/2 θrx1(2)=-π/2 | Cluster 4 θtx1(4)=-3π/2 θrx1(4)=-3π/2 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

CDL channel (For alignment)

Simulation assumptions for CDL channel specifically are captured in Table 2.4-1:

Table 2.4-1: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

- PMI for alignment

Common parameters are captured in Table 3-1:

Table 3-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8TX |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II [FFS Subband or Wideband for Type I] |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Test metric |  |  |  | - SNR@90% of maximum throughput with follow Type I PMI - Throughput ratio of follow Type 1 PMI to random PMI with SNR@90% of maximum throughput with follow Type 1 PMI - SNR@90% of maximum throughput with follow eType II PMI - Throughput ratio of follow eType II PMI to random PMI with SNR@90% of maximum throughput with follow eType II PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

Cluster model 1

Other parameters for cluster model 1 please refer to Table 3.1-1:

Table 3.1-1: PMI simulation assumptions for cluster model 1

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Number of clusters | 4 |
| Cluster powers p1, p2 | [0dB,0dB,-9dB,-9dB] |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Cluster TX steering in 1st array dim θtx1(1),θtx1(2),θtx1(3),θtx1(4) | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim θrx1(1),θrx1(2),θrx1(3),θrx1(4) | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Cluster model 2

Other parameters for cluster model 2 please refer to Table 3.2-1:

Table 3.2-1: PMI simulation assumptions for cluster model 2

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,2,1) |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Cluster-tap mapping | Table 2.2-2 |

CDL channel (For alignment)

Other parameters for CDL channel please refer to Table 2.4-1