3GPP TSG-RAN WG4 Meeting #114bis										          R4-2504892

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.1.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_ENDC_RF_Ph4

Document for:	Approval

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

Sub-topic 1: RRM core requirements for 6Rx capable UE

Issue 1-1: Interruption requirements at SRS antenna port switching

- Proposals

  - Option 1: Existing interruption requirements at SRS antenna port switching were reused for 6Rx UEs.

- Recommended WF

In RAN4#112bis meeting, the follow agreement was made [R4-2416848].

<Agreement> Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE Agreement: - The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed. - This can be revisited if the existing requirement cannot be applied to any SRS antenna switching pattern agreed in RAN1.

Option 1 further confirmed the agreement. No further discussion is needed.

Sub-topic 2: RRM performance requirements for 6Rx capable UE

Issue 2-1: Antenna connection configuration for 6 Rx capable UE

Agreement:

New antenna connection configurations are to be defined for 6 Rx capable UE, taking the antenna connection configuration of existing 8RX capable UE as reference.

Issue 2-2: Whether a new clause of antenna connection for 6 Rx capable UEs is to be introduced

Agreement:

A new clause of antenna connection for 6 Rx capable UEs is to be introduced

Issue 2-3: SNR level for RLM and BFD/CBD testing for 6Rx capable UE

Agreement:

RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE.