# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504897

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.11.7

Source:	Apple

Title:	WF on RRM requirements for NR_NTN_Ku_bands

Document for:	Information

# Way forward

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement:

- Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band

Issue 1-2: applicability update (section 3.6.12).

Proposals:

- Option 1: The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies. (LGE)

Recommended WF

- Continue discussion

Issue 1-3: Measurements of inter-frequency NR cells with NTN carrier (section 4.2.2.12).

Proposals:

- Option 1: The differentiation of requirements in clause 4.2.2.12 should not be based on the Rel-18 frequency range definition but on UE/Antenna type and capabilities. (Eutelsat Group)

Recommended WF

- Continue discussion

Issue 1-4: inter-satellite cell reselection requirements (section 4.2C).

Proposals:

- Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs in Rel-19. (Nokia)

Recommended WF

- Continue discussion

Issue 1-5: UE measurement capability (section 4.2C.2.1).

Proposals:

- Option 1: For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:

  - Intra-frequency carrier [in the case of VSAT UE with single receive beam, non-simultaneous monitoring intra-frequency carriers from more than one SAN], and

  - Depending on UE capability, FR1-NTN UE 7 NR inter-frequency carriers and for all VSAT UE up to [8] inter frequency carriers. (Eutelsat Group)

Recommended WF

- Continue discussion

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3)

Agreement:

- Reuse the same value of T_sat_beam as in Ka band for Ku band

- For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

Issue 1-7: timing accuracy requirements for case 1 and 2 (section 7.1C)

Agreement:

- Both 15kHz and 30kHz SCS for timing error requirements should be considered for Ku-band

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  | 30 | 30 | 22*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Issue 1-8: timing accuracy requirements for case 3 (section 7.1C)

Agreement:

- 7.5*64*Tc with the same side condition as agreed for Rel-18 Ka band.

Issue 1-9: timing accuracy requirements for case 4 (section 7.1C)

Agreement:

- The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4

  - Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case.

  - Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

Issue 1-10: timing accuracy requirements applicability (section 7.1C)

Agreement:

- The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. FFS how to capture it in the specification.

Issue 1-11: NTAoffset (section 7.1C.2)

Agreement:

- If UE is not provided with the information n-TimingAdvanceOffset, the default value of N_"TA offset" is set as 0 for FR1-NTN numerology and FR2-NTN numerology.

Issue 1-12: RLM (section 8.1C)

Proposals:

- Option 1: Confirm the assumption that LMax is equal to 8 for FR1-NTN (Nokia)

Recommended WF

- Continue discussion

Issue 1-13: terminology update

Proposals:

- Option 1: change ‘FR2-NTN’ to ‘[single receive beam] VSAT UE’ (Eutelsat Group)

- Option 2: others

Recommended WF

- Continue discussion on how to align terminology.

Issue 1-14: CR split

| Section Number | Section/ sub-section title | Leading company |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations |  |
| 4 | SA: RRC_IDLE state mobility | ZTE |
| 6 | RRC_CONNECTED state mobility | Huawei |
| 7 | Timing | MTK |
| 8 | Signalling characteristics |  |
| 9 | Measurement Procedure | Nokia |