3GPP TSG-RAN WG4 Meeting #114bis	R4-2504898

# Wuhan, China, 7th – 11th April, 2025

Title: 	WF on RRM requirements for NR_ATG_enh

Agenda Item:	7.12.7

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM core requirement

Issue 1-1-2: UE Beam Type capability signaling

Issue 1-1-2-1: The description of capability definition

Agreement:

- Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC.

  - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination.

Issue 1-1-2-2: The Report granularity of UE capability

FFS:

- Per BC

Issue 1-1-2-3: The Report feature

Agreement:

- Conditional mandatory

  - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band [of the supported Band combination]

Issue 1-1-2-4: The Report functionalities between FDD and TDD

Agreement:

- No functionality difference between FDD and TDD

- FFS how to reflect it on the spec.

Issue 1-1-2-5: The Report functionalities between FR1 and FR2

Agreement:

- FR1 only

Issue 1-1-5: Specification structure

Agreement:

- Create a new clause for ATG interruption requirement.

Issue 1-2-1: Whether Interruptions at NR SRS carrier based switching requirements need to be introduced

Agreement:

- Don’t introduce NR SRS carrier based switching interruption requirements.

Issue 1-2-2: Interruption requirements

- Proposals:

  - Proposal 1:

    - For the interruptions caused by SCell addition/release and any types of SCell activation/deactivation, distinguish the interruption requirements for Category 1 and 2.

      - Category 1: antenna array assumption in intra-band CA and Case 4 in inter-band CA

      - Category 2: other cases

    - For other interruptions, define unified interruption requirements for all UE antenna types.

Issue 1-2-3: SCell activation delay enhancement

- Proposals

  - Proposal 1: If the target SCell is known by the UE and frequency ranges of both bands are less than or equivalent to X MHz with common beam management the Tactivation_time can be 3ms

  - Proposal 2: if the target SCell is unknown, frequency ranges of both bands are less than or equal to X MHz Tactivation_time is 3ms+TFirstSSB+ 2ms.

Issue 1-3-1: Deactivated SCell measurement

Agreement:

- For intra-band CA and Case 4 of inter-band CA, no need to introduce L1/L3 measurement sharing factor.

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement

- Proposal 1:

  - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

    - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Issue 1-3-3: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For ATG UE antenna type Case 4

Agreement:

- For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement, following availability also applied for Case#4 UE.

  - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

  - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured

Issue 1-3-5: Measurement enhancement for ATG UE with antenna array

Candidate options:

- UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control.

  - Option 1: this can already be achieved by the existing configuration.

  - Option 2: Consider enhancement for Rel-19 ATG CA

<Topic 2> RRM perf requirement

Issue 2-1-1: Measurement accuracy

Agreement: 

- For ATG L1/L3 measurement of SCell/SCC, reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network.

Issue 2-2-1: Test configuration

Agreement:

- Following three test configurations are supported for both PCell and SCell:

  - Configuration 1:15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode

  - Configuration 2: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode

  - Configuration 3: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode

    - If legacy TN test cases don’t have this test configuration, then no need to introduce it into the corresponding ATG test cases.

Issue 2-2-2: Test applicability rule

- Proposal 1:

  - UE is only required to be tested in one of the supported test configurations

  - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

Issue 2-2-3: Test setups

Agreement:

- Following test setups agreed in R18 can be reused:

- TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell

- Channel model: AWGN with residual doppler channel model

- Number of neighbour cells: one neighbour cell in the test if needed

- GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

- BS location: The specific gNB reference location is emulated by test system

- RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases.

- OCNG:

  - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

  - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

  - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

Issue 2-2-4: Test method and scaling factor for UE with antenna array

FFS:

- The scaling factor related with antenna array will not be applied in test requirements

- The test parameters, test requirements for UE with antenna array on both PCC and SCC

Issue 2-2-5: Test scope

Merged proposal:

| Test category | Test cases | Company comments |
| --- | --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | CMCC: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | CMCC: Support HW: Support Ericsson: Support |
|  | interruption at NR SRS antenna port switching | HW: FFS |
|  | Interruptions at NR SRS carrier based switching | CATT: FFS |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support Ericsson: Support HW: Support |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX | CMCC: Support CATT: Support HW: Support Ericsson: Support |
|  | Direct SCell activation at SCell addition of known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell activation at handover with known SCell in FR1 | CMCC: Support CATT: Support HW: Support |
|  | Direct SCell Activation at RRC Resume | CMCC: FFS CATT: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | CMCC: Support CATT: Support HW: Support |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CMCC: Support CATT: Support HW: Support |
|  | Inter-band SSB-less SCell Activation based on A-TRS | CMCC: Support CATT: Support HW: FFS |
| Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | Ericsson: Support |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support Ericsson: Support |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs | CMCC: Support CATT: Support |
|  | non-simultaneous Timer based BWP switch delay on multiple CCs | CMCC: FFS |
| SCell dormancy switch | SCell dormancy switch | CMCC: Support |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX | CMCC: Support |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement | CMCC: FFS |
| Others | Intra-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | Inter-frequency measurement(with scheduling restriction verified as well) | ZTE: Support |
|  | L1-RSRP measurement | ZTE: Support |
|  | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | Ericsson: Support |
|  | The interruption of SCell addition/release | ZTE: Support |
|  | The interruption of SCell activation/deactivation | ZTE: Support |