# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504899

# Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.17.6

Source:	Moderator (Apple)

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Approval

# Issue 0: Work plan of test case (performance part) for NR_RRM_Ph5

Issue 0: Work plan of test case (performance part) for NR_RRM_Ph5

Agreement in online session: Approve the work plan in R4-2503530.

# FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

- Details of whether/how the CSSF factor is modified can be left to CR discussion.

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618.

Issue 1-2-2: Expand solution 1 to FR1 case

[Way forward]:

- FFS:

  - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band.

    - Option 1a (Nokia):

      - Expand the solution 1 to at least FR1 intra-band contiguous CA.

      - Details:

        - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

        - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

  - Option 2 (Huawei): Not to expand solution 1 to FR1 band.

  - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Issue 1-3-1: whether solution 3 includes NR-DC scenario or not?

Agreement in online session:

- Solution 3 includes NR-DC of FR1 and FR1 + FR2

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

[Way forward]:

- FFS:

  - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

    - Note: 3-searchers will be used by UE if UE report this capability.

- FFS: Other restriction (triggered by Huawei):

  - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple)

  - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple)

  - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT)

    - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

Issue 1-3-3: detailed CSSF table design for solution 3

Agreement in online session:

- This requirement applies regardless of whether measurement gap is configured or not.

Issue 1-4-1: UE capability for CSSF enhancement

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

CR splitting

- FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

CR; Company. CR for solution 1 (measure one carrier per-band); Apple. CR for solution 3 (3 searchers); Huawei.

# Topic #2: RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode (7.17.4)

Issue 2-1-1: TC list for Rx BSF optimization

Agreement in online session: Further discuss based on the following table:

| Index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CMCC |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | CATT |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Huawei |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | Samsung |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) | OPPO |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) | Intel |

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1:

Agreement in online session:

- Define the test cases for solution 1 of CSSF enhancement following the table below:

| TC index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | ZTE |

- TC for solution 3:

[Way forward]:

- FFS:

  - Define the test cases for solution 3 of CSSF enhancement following the table below:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |