3GPP TSG-RAN WG4 Meeting #114bis	R4-2504907

Wuhan, China, 07 – 11 April 2025

Agenda item:			7.25.3

Source:	Huawei

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part2

Document for:	Approval

- Topic #1: Adaptation of common signal/channel transmission

Sub-topic 1-1 SSB adaptation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

    - SSB adaptation in SCell for NCD-SSB.

    - Time domain positions of SSBs within a burst is same before and after SSB adaptation

  - Note: Above could be further updated based on further RAN1 progress if any.

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: L1 requirements at transition

Agreement

  - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline.

  - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation

Agreement:

- RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

    - For neighbour cell L3 measurement, legacy requirements apply.

Open issue for further discuss:

- For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects:

  - Whether to define L3 measurement requirement at transition.

  - Any other RRM requirements impacts due to additional SMTC configurations.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Agreement:

    - Normal SCell activation

    - Direct SCell activation

    - PUCCH SCell activation

    - FFS Rel-18 SCell activation with L3 reporting

    - FFS Multiple SCells activation

- Topic #2: OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

    - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing

Agreement

- Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1:

  - reference cell is the NES cell

  - NTA for UL-WUS is defined as 0

  - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

  - Note: The IE can be updated based on other WG’s progress.

- Open issues

Issues captured in summary R4-2503626

- Reference

R4-2503626	Topic summary for [114bis][226] Netw_Energy_NR_enh_Part2