3GPP TSG-RAN WG4 Meeting # 114bis			                               R4-2504908

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.26.5

Source:	Moderator (vivo)

Title:	WF on RRM requirements for NR_LPWUS

Document for:	Approval

Topic #1: Agreement on RRM core requirements for LP-WUS/WUR

- Sub-topic 1-1 General aspects

Issue 1-1-10: Considerations on higher priority frequency layer

Agreement:

- Support relaxed higher priority frequency layer search at case 1

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

Agreement:

  - For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE

Spec impact and CR split consideration

Agreement:

|  | Clause | Impact Comment |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations | Yes |
| 4 | SA: RRC_IDLE state mobility | Yes |
| 5 | SA: RRC_INACTIVE state mobility | Yes |
| 6 | RRC_CONNECTED state mobility | No |
| 7 | Timing | No |
| 8 | Signalling characteristics | FFS |
| 9 | Measurement Procedure | FFS |
| 10 | Measurement Performance requirements | No |
| Annex B | Conditions for RRM requirements applicability for operating bands | Yes |

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: General on LR accuracy requirement

Agreement:

No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Agreement:

For FR1:

    - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

    - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

    - Note: [2 or 2.5] dB RF impairment margin is assumed

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

Agreement:

Regardless of  whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Issue 1-2-4-1: On LR measurement filtering requirement

Agreement:

  - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR.

Issue 1-2-5-2: On MR evaluation requirements

Agreement:

- Reuse legacy Nserv

- Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

Issue 1-2-6-1: On the number of SSBs

Agreement: Aim to conclude the issue by Tuesday in RAN4 #115.

Issue 1-2-6-2: On SSBs number for different SSB periodicities

Agreement

Same SSB number for different SSB periodicity

Agreement

When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay.

Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

Agreement:

  - No need to define the LR-WUR based cell selection criterion evaluation in case 1.

- Sub-topic 1-3 MR RRM relaxation

- Sub-topic 1-4 LP-WUR CONNECTED mode

- Sub-topic 1-5 Others

Topic #2: Agreement on simulation assumptions and results

Sub-topic 2-1 On simulation assumptions and parameters

Sub-topic 2-2 Simulation results alignment and summary

Topic #3: Open issues and Recommendations

Details on all open issues can be found in [1] and [2]. All other open topics in [1] are contribution driven.

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Recommendations:

Postpone the discussion until more RAN2 progress

Issue 1-1-2: Threshold for switch between different cases

Recommendations:

Up to RAN2 decision

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

Recommendations:

For both issue 1-1-5, 1-1-6 and 1-1-7, criteria are up to RAN2’s consideration and close these issues.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Recommendations:

Further discuss whether LR evaluation requirements apply when LR is “ON” at legacy state.

Issue 1-1-10: Considerations on higher priority frequency layer

Recommendations:

Further discussion on detail on requirements and related tentative agreement.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Recommendations:

Continue discussion

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

Recommendations:

Continue discussion

Issue 1-2-4-0: On LR measurement requirement

Recommendations:

Direct discussion in the CR

Issue 1-2-4-1: On LR measurement filtering requirement

Recommendations:

Continue discuss on whether to define and how to define LP-SS based LR filtering requirements

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Recommendations:

Continue discuss on detail at next meeting

Issue 1-2-6-1: On the number of SSBs

Recommendations:

For the number of samples and side conditions could consider:

1) determine the SSB number based on minimum number for AGC, timing etc., without link to any particular side conditions

2) determine the SSB number based on minimum number for AGC, timing etc., considering LR side condition.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Recommendations:

Direct discussion in the CR

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

Recommendations:

Direct discussion in the CR

Issue 1-4-1: LP-WUR at CONNECTED mode

Recommendations:

Companies which is interested can continuously discuss.

Note:

Suggestion from the proposed company:

Discuss the interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG.

Check whether the interruption requirements specified in Clause 8.2 of TS 38.133 also apply to WUS monitoring by WUR in the Connected mode

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Recommendations:

Based on previous agreement, the suggestion that ideal RSRP is derived from the serving cell signal after the fading channel in P2 is already used as a base.

Issue 2-1-7: On OOK interference

Recommendations:

For the P1 in issue 2-1-7, encourage company to check whether there is any issue.

Issue 2-1-11: Others on simulation assumptions and campaign

Recommendations:

For the P2 in the issue 2-1-11:

- For company who use both I and Q branch, please indicate 2 branches are used in simulation results.

- For company use one branch, please indicate which branch is used in simulation results.

Reference

[1] R4-2503627, Topic summary for [114bis][217] NR_LPWUS, vivo, RAN4 114bis

[2] R4-2504921, Ad hoc minutes for [114bis][217] NR_LPWUS, vivo, RAN4 114bis