3GPP TSG-RAN WG4 Meeting # 114bis											R4-2504911

Wuhan, China, 07 – 11 April 2025

Title: 	WF on RRM requirements for XR_Ph3

Agenda Item:	7.28.3

Source: 	Nokia

Document for:	Approval

# Topic #1: Scenarios for measurement skipping

## Issue 1-1: Deployment scenarios

< Agreement>:

- Skipping requirements applies for NR-SA

## Issue 1-5: L1/L2 triggered mobility measurements and procedures

<Agreement>

- Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping.

# Topic #2: Measurements with gaps

## Issue 2-1: Types of measurement gaps

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

<Agreement>:

- Gaps which shall not be cancelled

  - MUSIM

  - Positioning

- For other gap types:

  - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19

  - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows.

# Topic #3: XR skipping requirements impact

## Issue 3-2: Formulas for measurement delay extension

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

- Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms, ( 8 + Nskip)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

    - FFS: CSSFinter

## Issue 3-3: Formulas for measurement delay extension with DRX

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

## Issue 3-4: Conditions for measurement skipping

<Way forward>:

- Option 1: No conditions are defined

- Option 2: Define the maximum number of gaps skipped during the measurement period.

- Option 3: Define a maximum separation between two non-skipped gaps for measurement

# Topic #4: Time offset for measurement skipping processing

## Issue 4-1: Time offset requirement for measurement gap skipping

<Agreement>:

- Not define additional condition for the optional capability of 3 ms.

## Issue 4-2: Time offset requirement for cancelling skip indication

<Conclusion>:

The following is the common understanding in RAN4 RRM session:

- The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

  - X=5ms as baseline and X=3ms as optional UE capability.

  - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

# Topic #5: Need/feasibility of UE assistance information

## Issue 5-1: General on UAI

<Agreement>:

Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting.

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

  - R ∈ {0, 20, 40, 60}%

# Topic #6: Additional signalling aspects

## Issue 6-1: Semi-static schemes

<Way forward>:

- Further discussion based on recommended options

  - Option 1: Do not consider semi-static solutions

  - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

    - Option 2a: RRC pattern-based

    - Option 2b: MAC CE bitmap for skipping indication

    - Option 2c: RRC configuration patterns with MAC CE activation

  - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

## Issue 6-2: Skipping indication on frequency domain

<Way forward>:

- Proposals

  - Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2:

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

  - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

# Topic 8 List of impacted clauses and worksplit

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| 9.15.5 | Inter-frequency L1-RSRP with MG | vivo |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |