# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504916

# Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.17.6

Source:	Moderator (Apple)

Title:	AdHoc minutes for [114bis][208][209] NR_RRM_Ph5

Document for:	Approval

# FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

[Discussion]:

Can we remove the bracket for [EN-DC]?

Agreement:

- the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios.

  - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC

  - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only

- Details of whether/how the CSSF factor is modified can be left to CR discussion.

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

- Proposal 1 (OPPO): For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- [Discussion]:

  - [offline received comment from RAN2]:Clarify “Network indication of a specific SCC per-band for measurement” is dynamic indication based on MAC CE or semi-static indication based on RRC signaling? Can we send an updated LS to RAN2?

[Nokia]: Suggest to leave to RAN2

[HW]: the signaling shall be discussed in RAN4
[Ericsson]: the baseline is multiple MOs are configured, which MO is chosen to measure can be discussed in RAN4. Don’t need give further information to RAN2.

[ZTE]: this indication can be dynamic

[Intel]: so far no measurement configuration is from MAC CE.

“Network indication of a specific SCC per-band for measurement” is

Option 1 (Apple, HW, ZTE, CATT): dynamic indication based on MAC CE

Option 2 (QC, OPPO): semi-static indication based on RRC signaling

Option 3 (ZTE, Nokia, Ericsson, MTK): leave to RAN2 to decide.

Issue 1-3-1: whether solution 3 includes NR-DC scenario or not?

- Option 1(QC, CATT, Nokia, Apple, Huawei, CTC, Intel): Yes, solution 3 includes NR-DC of FR1 and FR1 + FR2

  - Option 1a(Apple): NR-DC can be included in solution 3 if the UE capability is defined separately for {FR1 only CA}, {FR1 NR-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}.

- Option 2(MTK): Remove NR-DC from (FR1 + FR2 scenario) while it can be kept for (FR1 scenario).

Agreement in online session:

- Solution 3 includes NR-DC of FR1 and FR1 + FR2

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

- Option 1(QC, Apple): Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

- Option 2(Nokia): Network may indicate to use the 3rd searcher for a specific SCC if UE supports the capability.

  - Option 2a (Ericsson, ZTE):

    - NW is able to indicate the single intended serving cell (FR1) to be exclusively served by the third searcher.

      - In this case, the first and the second searcher manage other serving cells in the conventional manner.

      - If no NW indication, the third searcher resource sharing is same as the second searcher sharing.

      - The above may be only applied in FR1 only configuration.

- Option 3(vivo): impact on UE implementation needs be considered to determine whether  network indication method should be considered or not.

- Option 4 (HW): Due to the design of Solution 1 and the analysis of FR1 and FR2 measurement characteristics. The 3-searcher solution can be defined as follows:

| One of 3 searchers | Measurement Objectives |  |  |
| --- | --- | --- | --- |
| Searcher 1 | FR1 PCC |  |  |
| Searcher 2 | FR1 SCC(s) | FR1 inter-frequency MO(s) with no measurement gap | inter-RAT MO(s) with no measurement gap |
| Searcher 3 | FR2 SCC where neighbour cell measurement is required and | FR2 SCC(s) where neighbour cell measurement is not required. | FR2 inter-frequency MO(s) with no measurement gap |

Note: For the searcher used to measure the FR2 MOs, the FR2 SCC that requires neighbour cell measurement shares 50% of the searcher, while the remaining 50% is shared between the FR2 SCCs that do not require neighbour cell measurement and the FR2 inter-frequency MOs with no measurement gap.

- Option 5 (Intel):

  - The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

  - The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice

- [Discussion]:

  - Check whether tentative agreement can be formally agreed?

[Ericsson]: we can keep it open, it will not impact the 3 searcher solutions.

[Way forward]:

FFS:

- Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF.

  - Note: 3-searchers will be used by UE if UE report this capability.

  - Other restriction (triggered by Huawei):

    - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple)

    - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple)

    - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT)

      - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

[Discussion]:

[Nokia]: if UE wants to avoid this overlapping case, UE can indicate to not support this 3 searcher

[OPPO]: agree with Nokia when UE is not supporting 3 searching. We need applicability rule for colliding case.

[Ericsson]: UE can use capability to indicate not supporting the overlapping case

[ZTE]: support option 3

[HW]: option 3 and 3a only has difference on the wording. CSSF outside overlapped with per-FR MG will result into 3+1 searchers in this release. UE can support this capabilities simultaneously.

Issue 1-3-3: detailed CSSF table design for solution 3

- Proposal 1 (CMCC): as for how to reflect 3 searchers on CSSF, it is proposed as proposed table in R4-2503764.

- Proposal 2 (Intel): The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

- Proposal 3 (Intel): This requirement applies only when there is no measurement gap configured in Rel-19.

Agreement in online session:

- This requirement applies regardless of whether measurement gap is configured or not.

Issue 1-4-1: UE capability for CSSF enhancement

- Proposal 1 (Apple):

  - to have a new optional per-UE capability for CSSF enhancement solution 1 in Rel-19 and it shall be a separated UE capability from 3 searcher solution (i.e., solution 3).

  - For solution 3, the UE capability is defined separately for {FR1 only CA}, {FR1 NR-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC} (Apple, CMCC)

- Proposal 2 (CMCC):

  - for UE capability on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it is proposed as following:

    - For solution 3 (3 searchers for CSSF), define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC), as agreed in last meeting. (CMCC, Apple)

    - For solution 1 (measure one serving CC per ban), no UE capability is needed since UE measurement is simplified compared with legacy way

- Proposal 3 (HW):

  - Per-FR gap and new 3-searcher capabilities should be restricted so that only one can be configured at once.

- Proposal 4 (Ericsson):

  - A UE may indicate support for both Solution 1 and Solution 3 through a single capability declaration.

- [Discussion]:

  - Check whether UE capability for solution 1 is per-UE or per-FR?

Agreement:

- UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR.

- UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Issue 1-2-2: Expand solution 1 to FR1 case

- Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band.

  - Option 1a (Nokia):

    - Expand the solution 1 to at least FR1 intra-band contiguous CA.

    - Details:

      - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

      - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA.

- Option 2 (Huawei): Not to expand solution 1 to FR1 band.

- [Discussion]:

  - [Moderator]: check if following option can be a compromise.

    - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

[HW]: in FR1, use one CC measurement to represent other CCs is risky

[QC]: agree with HW

[Nokia]: want to apply the same principle for FR1 and FR2

[Ericsson]: the propagation condition can be different between FR1 and FR2. But enable/disable it can be controlled by network.

[ZTE]: support option 3.

[Intel]: support option 3.

CR splitting:

Draft CRs in this meeting

| R4-2503504 | Nokia, Nokia Shanghai Bell | For solution 1 and 3 |
| --- | --- | --- |
| R4-2503529 | Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung | For solution 1 only |

| CR | company |
| --- | --- |
| CR for solution 1 | ? |
| CR for solution 3 | ? |

# FR2-1 L3 measurement delay by optimizing BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

- Issue a: Whether to limit the serving cell to FR2 serving cell:

  - Agreement in online session:

    - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell.

- Issue b: Whether to add the hysteresis and TimeToTrigger parameters:

  - Can companies agree on the combined Option 3?

    - Combined Option 3: RAN4 encourage RAN2 in the LS to consider the benefits of hysteresis and TimeToTrigger parameters, e.g., avoiding ping-pong effect between activation and deactivation of FBS, when designing triggering condition signalling, but whether to add them is up to RAN2.

  - Agreement in online session: Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable.

- [Discussion]:

  - CATT triggered LS discussion on RAN4 reflector

Sub-topic 1-2 Requirements of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- [Discussion]:

Agreement:

- Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability.

Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

- [Discussion]:

Discussion by using tables from NTT DCM:

Agreement:

  - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3 |
| 2 | Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3 |
| 3 | Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3 |
| 4 | Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS]. |
| 7 | RRC Re-establishment/redirection Can be discussed directly in the draft CR. |

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

- [Discussion]:

- Discuss the option(s) to finalize quitting conditions and re-evaluation mechanism.

  - Note: Option 1c/1d/3a/3b includes re-evaluation mechanism, which may be coupled with the quit conditions or not, they are collected as follow:

    - Option 1c: After quitting fast measurement, UE shall re-activate fast measurement no longer than T2 from the quitting time.

    - Option 1d: (after quitting), the UE makes another exam on the criteria that are configured for the serving cell quality. If the criteria are met, the UE continues to use reduced beam sweeping factor while the UE exits the state when the quality criteria are no longer met.

    - Option 3a: Introduce another timer, and UE re-evaluates the serving cell quality threshold condition after it expires. This timer starts to run when UE deactivates Rx BSF reduction feature.

    - Option 3b: After deactivating FBS, the UE may re-evaluate the condition of enabling FBS, at least after a minimal time interval ‘time to active’ after deactivated or disabled.

# Topic #2: RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode (7.17.4)

Issue 2-1-1: TC list for Rx BSF optimization

Agreement in online session: Further discuss based on the following table:

| Index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | TBA |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | TBA |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | TBA |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) | TBA |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) | TBA |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) | TBA |

- [Discussion]:

  - Check the volunteer company and check if the bracket in the above table can be removed?

  - Check the TC number in 3rd column.

Issue 2-2-1: TC list for CSSF optimization

- TC for solution 1:

Agreement in online session:

- Define the test cases for solution 1 of CSSF enhancement following the table below:

| TC index | Scenario | Need or not | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA | TBA |

- [Discussion]:

  - Check the volunteer company and check if the bracket in the above table can be removed?

- TC for solution 3:

  - Define the test cases for solution 3 of CSSF enhancement following the table below:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA | TBA |

- [Discussion]:

  - Check the volunteer company and discuss the TC number for CA/DC mode?

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Scenarios of test cases

- [Discussion]:

  - RAN4 to define test cases for eEMR-based fast SCell activation based on the following aspects:

    - Scenarios:

      - Normal SCell activation

      - Direct SCell activation at SCell addition

      - PUCCH SCell activation

    - Frequency range:

      - FR1 and FR2

    - UE capabilities and/or network configurations:

      - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

      - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

      - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

  - Discuss whether to choose a subset of combinations of the above aspects to define test cases.

  - Discuss the applicability rule whether UE is required to pass all of test cases.