# 3GPP TSG-RAN WG4 Meeting #114bis	                 		                                R4-2504922

# Wuhan, China, 7 – 11 April, 2025

Agenda item:			7.27.3

Source:	Apple

Title:	Ad-hoc minutes for NR_Mob_Ph4

Document for:	Information

# Topic #1: Event triggered L1 measurement reporting

#### Issue 2-5: whether and how to coordinate with R19 MIMO

Candidate options:

- Option 1: RAN4 shall develop the two features independently given that UE is allowed to only support one of the two features. (CATT, Apple, CMCC, HW, CTC, vivo)

Recommended WF:

- Agree on option 1.

Discussion:

MTK: does it mean UE is not allowed to support both R19 LTM and R19 MIMO?

QC: this means RAN4 develop requirement separately.

Vivo: this is tranditional way. We defined requirement for different features independently.

MTK: can we say requirements do not apply if UE supports both features?

E///: this is only about capability.

MTK: this is about number of events that UE needs to support.

Agreement:

- RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

#### Issue 2-7: Impact on other LTM requirements

Tentative agreement in RAN4#114:

- UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable.

  - Further discuss how to capture this in RRM spec.

Candidate options in RAN4#114bis:

- Option 1: tentative agreement may not work, since UE may stop measuring the beam that triggers L1 report if NW activates some other TCI state(s) after event-triggered L1 report, which may result the reported TCI state unknown. (MTK)

- Option 2: UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met, then the TCI can be regarded as known. (HW)

- Option 3: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured. (E///)

- Option 4: (Nokia)

  - Capture the tentative agreement about cell switch and TCI state activation delay applicability in RAN4#114 WF as a separate applicability condition i.e. do not edit the existing known TCI state condition.

  - If UE sends report on leave, this leads to an associated TCI state being known for 1280 ms after the report similar as any other L1 report i.e. requirements do not stop applying immediately when report on leave is sent.

Recommended WF:

- Discuss the options.

Agreement:

- For FR1:

- UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

  - Case 1: leaving condition of the event is configured and corresponding condition is not met,

    - the related cell switch delay and TCI state activation requirements are applicable

  - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

#### Issue 2-8: which result to report if UE has chance to update measurement after report is triggered and before report is actually sent

Candidate options:

- Option 1: UE ignores new L1 result after L1 report is triggered. (Xiaomi)

- Option 2: UE consider new L1 result after L1 report is triggered

- Option 3: up to UE implementation (Apple)

Recommended WF:

- Discuss the options.

QC: RAN1 or RAN2 may be a better place to discuss this.

E///: this is a corner case. We see no problem to leave it to UE implementation.

Agreement:

- up to UE implementation.

  - This can be revisited pending progress in other working groups.

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo)

- Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

Recommended WF:

- Discuss the options.

#### Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options:

- Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC)

- Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///)

- Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

Recommended WF:

- Discuss the options.

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2 (Apple)

  - Seems no need to capture assumptions listed under option-1 into core requirements.

Recommended WF:

- Discuss the options.

# Topic #2: CSI-RS based L1 measurement

#### Issue 4-1: Known cell definition in CSI-RS based L1 measurement

Issue 4-1-1: Whether to use detectable or measurable?

- Candidate options:

  - Option 1: confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable”. (Apple, CATT, Huawei, Ericsson)

- Recommended WF:

  - Option 1

Agreement:

- Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable”

Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell

- Candidate options:

  - Option 1: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Xiaomi, OPPO, MediaTek, ZTE, Huawei, Ericsson, China Telecom, vivo, Nokia)

- Recommended WF:

  - Option 1

Agreement:

- Use legacy side condition of SINR=-3dB.

#### Issue 4-11: Whether to consider CSI-RS based L1-SINR measurement

- Candidate options:

  - Option 1: (CATT, Apple, China Telecom, vivo)

RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- Recommended WF:

  - Option 1

Agreement:

- RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

#### Issue 4-7: CSI-RS resource type

- Candidate options:

  - Option 1: (Apple, CMCC, ZTE, Ericsson, CATT, Nokia, vivo)

Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

- Recommended WF:

  - Option 1

Agreement:

- Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS.

#### Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2

Agreement:

Whether step 2 can be skipped for negiboring cell or not based on new UE capability reporting.

Further discuss whether step 2 can be skipped for serving cell or not.

#### Issue 4-2-2: How to interpretate the UE capability (for FR2-1) for the case of neighboring cell

Agreement:

For a UE not supporting the capability, Step 2 is necessary.

    - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

    - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

    - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

    - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

    - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

#### Issue 4-2-3: Requirement for keeping step 2 and skipping step 2?

- Candidate options:

  - Option 1: Huawei

Define requirement for both the case with keeping step 2 and the case with skipping step 2.

- Recommended WF:

  - Merged into issue 4-2-2.

#### Issue 4-3: Definition of “inside/outside active BWP”

- Candidate options:

  - Option 1: Xiaomi

The CSI-RS of candidate cell should be considered inside the active BWP if the BW of the CSI-RS is within active BWP of the UE.

  - Option 2: (CATT)

The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP.

All cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’.

  - Option 3: (Apple)

A CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

  - Option 4: (CMCC, Apple)

It is proposed that the CSI-RS of candidate cell is considered inside the active BWP if

    - all the CSI-RS resources are confined within the BWP (for fully overlap case), or

    - the RBs of the CSI-RS within the active BWP is no less than 48 RBs (for partially overlap case)

  - Option 5: (Nokia)

Fully within active BWP means that all the resources are confined within the active BWP.

Partially within active BWP means that some of the resources may be outside of the active BWP but at least 48 PRBs are within the active BWP

Outside of the active BWP means that less than 48 PRBs are within the active BWP.

  - Option 6: (vivo)

RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM:

    - Option A: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP.

    - Option B: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE.

    - Option C: (preferred): a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

- Recommended WF:

Tentative:

  - A CSI-RS is considered to be within active BWP if the RBs of the CSI-RS within the active BWP is no less than 48 RBs.

  - A CSI-RS resource is regarded as outside active BWP if <48 RBs of the CSI-RS resource is within any active BWP.

Agreement:

RRM requirements (delay and accuracy) apply under the following condition:

- at least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

#### Issue 4-5: Measurement on CSI-RS with repetition ON/OFF

- Candidate options:

  - Option 1: CATT, China Telecom

In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

In FR2, RAN4 will define the requirements for the following case:

    - CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

  - Option 2: Apple, ZTE, Nokia

RAN4 prioritize L1 measurement with repetition OFF for RAN4 requirement discussion at this stage and wait for RAN1 agreement on L1 measurement with repetition ON.

  - Option 3: Huawei

Proposal 4: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

  - Option 4: vivo

Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1.

- Recommended WF:

  - RAN4 prioritize L1 measurement with repetition OFF for RAN4 requirement discussion.

  - Do not consider CSI-RS with repetition on or wait until RAN1 has conclusion.

MTK: RAN1 already ruled out repetition ON.

Agreement for FR2:

- Only consider CSI-RS with repetition OFF in RAN4 requirements.

#### Issue 4-12: Early CSI acquisition on a candidate cell

- Candidate options:

  - Option 1: Qualcomm

For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.

For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded:

    - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc.

  - Option 2: Nokia

Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

- Recommendation

  - For discussion

#### Issue 4-8: Restriction on CSI-RS resource configuration

Agreement in RAN4#114:

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

- Candidate options:

  - Option 1: Ericsson

In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

- Recommended WF:

  - For discussion

#### Issue 4-10: Definition of frequency layer

- Candidate options:

  - Option 1: Apple, Nokia

Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded.

  - Option 2: vivo

If RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement.

    - If the CSI-RS resources among candidate cells are overlapping with at least 48RBs, and have the same SCS and CP type, it is considered as one frequency layer.

Vivo: since we already agreed to only consider the case where the CSI-RS is within active BWP, we don’t need to have definition of frequency layer.

Agreements:

    - No need to introduce definition of frequency layer.

#### Issue 4-14: others

- Candidate options:

  - Option 1: Nokia

After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

Discuss how many measurements CSI-RS measurements UE is capable of performing during each measurement period.

- Recommended WF:

  - For discussion

MTK: there is no requirement when two sets of CSI-RS, which cannot be measured simultaneously, collide with each other.

# Topic #3: conditional LTM

Issue 1-2-7: Tmeasure

- Candidate solutions:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. (CTC, vivo, CATT, CMCC, Apple)

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. (Huawei, Xiaomi, vivo, CATT)

  - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. (Huawei, Xiaomi, vivo, CATT)

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. (Huawei, vivo, CATT)

  - FFS on CSI-RS based L3-RSRP measurement. (Huawei, CATT)

    - Proposal 1b: (Nokia)

      - When the execution condition is L1-based, Tmeasure in CLTM to be defined using the existing L1 measurement period for serving cell and candidate cells, with Treport = 0.

      - When the execution condition is L3-based, Tmeasure in CLTM to follow the definition of Tmeasure in CHO.

- Proposal 2: Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled. (MTK)

    - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

    - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

    - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1.

    - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15.

- Recommended WF

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

Agreement:

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19.

Issue 1-2-15: Others

- Proposal 1: RAN4 to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell. (Nokia)

- Recommended WF

  - Discussion is needed.

Vivo: RAN2 is discussing this issue, i.e. whether UE shall continue L1 measurement. Maybe we can wait for their progress.

QC: we don’t want to change UE measurement behaviour.

MTK: we prefer to follow rule introduced in R18.

Agreement:

    - L1 measurement behaviour, which was introduced in R18, is not changed. CLTM requirements do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2.

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

- Candidate solutions:

  - Option 1: Define requirements for L1 measurement on deactivated Scell. (CMCC, E///, ZTE, MTK)

  - Proposal 1a: Only in R19 CLTM. (CATT)

  - Proposal 1b: (Huawei)

- Whether interruptions exist on activated serving cells

- Whether to extend L1 measurement period for deactivated SCC.

- Option 2: Define requirements for UE based TA measurement. (CTC, vivo, CMCC, ZTE, QC)

  - Proposal 2a: Focus on how to define the feasible requirements. (CATT)

  - Proposal 2b: (QC)

  - Tx timing for the target cell = current Tx timing + 2 x (downlink frame boundary of reference cell – downlink frame boundary of the target cell), where the definition of ‘timing reference cell’ is the reference cell defined in the clause of ‘7.1 UE transmit timing’ in TS38.133. (QC)

  - FFS on the side conditions in terms of SNR, timeline for the downlink frame boundary estimation, etc.

  - Proposal 2c: The issues listed below need to be taken into account. (Nokia)

•	Requirements to cover not only Rel-19 CLTM but ideally also Rel-18 LTM.

•	Which reference signals are expected to be used when performing UE-based TA measurements?

•	How long does the acquisition of the UE-based TA take after candidate cell configuration?

•	UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

•	Existing transmit timing accuracy requirements shall apply.

•	How to ensure the accuracy of the UE-estimated TA?

- Option 3: Not define requirement for any of them. (Apple, MTK, Nokia)

- Recommended WF

  - Discussion is needed.

  - Considering we only have three meetings left, if we could not achieve any agreement on option 1 and option 2. Maybe we could consider option 3.

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

- Candidate solutions:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. (CTC, vivo, Apple, Huawei, ZTE)

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. (CATT, ZTE)

    - Proposal 1b: TEvent_DU needs to be updated. (ZTE)

    - Proposal 1c: Apart from the starting point of the delay. (vivo, HW)

- Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell. (Nokia)

- Recommended WF

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-3-3: The components of subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple)

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU. (E///)

- Proposal 3: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, where the TCLTM_interrupt = TCLTM-processing + TCLTM-IU. (Nokia)

- Recommended WF

Waiting for the conclusions of CLTM delay

### CR skeleton

| Expected impact | Notes | New or impacted section | Companies |
| --- | --- | --- | --- |
| Definitions and abbreviations |  | 3.1 3.3 |  |
| CLTM delay requirements | Depending on RAN2 conclusion | (new) 6.x or 6.3 | China Telecom |
| UL transmits timing requirements | TBD: UE based TA measurement | 7.1 7.3 |  |
| PDCCH-order RACH on neighbor cell | The related R18 RAN4 requirements could be as baseline | 6.2.2C |  |
| TCI state activation | The related R18 RAN4 requirements could be as baseline | 8.25 |  |
| L1-RSRP measurement | The related R18 RAN4 requirements could be as baseline | 9.14 9.15 |  |
| L3 measurement based CLTM | The CHO requirements could be as baseline |  |  |