3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504937

# Wuhan, China, 07th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-19 TC on SCell Activation and Deactivation Delay Requirement, and PSCell addition and release delay with less than 5 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To introduce new Test Cases for SCell Activation and Deactivation Delay requirements with less than 5 MHz. - To introduce new Test Cases for PSCell addition, change delay with less than 5 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following test cases: - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW - Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Without validation, SCell Activation and deactivation of known SCells, UEs may fail to meet activation/deactivation timing, causing latency issues, lower peak data rates, and inefficient CA performance in scenarios with less than 5 MHz bandwidth. - Without Validation, the Addition and Release Delay of an unknown NR FR1 PSCell may cause less than 5 MHz DC UE to be delayed, or incorrect PSCell handling can degrade service quality, resulting in increased latency, missed scheduling opportunities, or uplink interruptions during NR dual connectivity operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3.X, A6.5.11.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R4-2504460 merged |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from 2504459 |  |  |  |  |  |  |  |  |

### <<Start of Change0>>

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.X	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW

##### A.6.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation, and the SCell operate with a bandwidth of less than 5 MHz.

Supported test configurations are specified in Table A.6.5.3.X.1-1 below. General test parameters for known Scell activation case is specified in Table A.6.5.3.1.1-2. Supported Cell specific test parameters for NR PCell specified in Table A.6.5.3.1.1-3. Supported Cell specific test parameters for NR SCell specified in Table A.6.5.3.1.1-4 apply except those specified in Table A.6.5.3.X.1-2.

The test procedure specified in A.6.5.3.1 applies to this test.

Table A.6.5.3.X.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.5.3.X.1-2: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| BWchannel | ConfigSCell 1 | MHz | 3 MHz |  |  |
| BWoccupied | ConfigSCell 1 | RB | 12 |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.2 FDD |  |  |
| SSB Configuration | ConfigSCell 1 |  | SSB.13 FR1 |  |  |
| Io Note3 | ConfigSCell 1 | dBm/ 2.7 MHz | -60.31 | -58.58 | -58.58 |
| NOTE : On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |

##### A.6.5.3.X.2	Test Requirements

Test requirements specified in Clause A.6.5.3.1.2 apply to this test except Tactivation_time will be replaced with TFirstSSB+ T∆ + 5ms.

### <End of Change0>>

### <<Start of Change1>>

### A.6.5.11	PSCell addition and release delay

#### A.6.5.11.X	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz

##### A.6.5.11.X.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delay under NR-DC with a bandwidth of less than 5 MHz meets the requirements defined in Clauses 8.9.2 and 8.9.3, where the PSCell is unknown to the UE at the time of addition.

Supported test configurations are shown in Table A.6.5.11.X.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 and radio channel 2 in FR1, respectively. Test parameters are given in Tables A.6.5.11.X.1-2 below. The cell-specific parameters as specified in Table A.6.5.11.1-2 with config 1 apply except those specified in Table A.6.5.11.X.1-2 .

The test procedure specified in A.6.5.11.1 applies to this test.

Table A.6.5.11.X.1-1: Supported test configurations for FR1 PSCell Addition and Release with less than 5 MHz

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 3 MHz (12 PRB), FDD |

Table A.6.5.11.X.1-2: Cell Specific Parameters for FR1 PSCell Addition and Release with less than 5 MHz

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 | T4 |
| BWchannel | MHz | 1 |  | 3: NPRB,c = 12 |  |  |  |  |
| Data PRBs allocated |  | 1 |  | 12 |  |  |  |  |
| RMSI CORESET parameters |  | 1 |  | CR.1.2 FDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 |  | CCR.1.6 FDD |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 |  | 15 |  |  |  |  |
| SSB configuration |  | 1 |  | SSB.13 FR1 |  |  |  |  |
| IoNote | dBm/2.7MHz | 1 |  | N/A |  | –63.4 dBm |  |  |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.5.11.X.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell no later than 212ms  with 12 PRB SSB bandwidth. Otherwise 172 msNote1 from the start of T2.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T3.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T3.

The UE shall stop sending CSI reports for PSCell no later than 20 ms from the start of T4.

All the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90 %.

Note1:	The PSCell addition delay can be expressed as follows as specified in clause 8.9.2:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + 3* T∆ + TPSCell_ DU + 2 ms with 12 PRB SSB bandwidth.

Otherwise, Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

Where:

TRRC_delay = 50 ms

Tprocessing = 20 ms

Tsearch 	= 60 ms

T∆ = 20 ms

TPSCell_ DU = 1*10+10 = 20 ms

### <<End of Change1>>