3GPP TSG-RAN WG4 Meeting # 114bis											R4-2504950

Wuhan, China, 7th - 11th April 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Ad-hoc #2 minutes for NR_duplex_evo

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [114bis][213] NR_duplex_evo for the following issues:

Issue 2-1-1a

Issue 2-1-1b

Issue 2-1-1c

Issue 2-1-4

Issue 2-1-6

Issue 2-1-7

Topic #2: SBFD operation impacts

- Issue 2-1-1a: CSI-RS L1 measurement – collision with UL

- Proposals

  - Proposal 1 (vivo, CATT, Nokia, Samsung, CMCC, HW, ZTE, MTK, QC):

    - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement.

  - Proposal 2 (vivo, Nokia, HW, QC):

    - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission.

      - Follow the R16 NR-U approach (HW, QC)

  - Proposal 3 (MTK):

    - no need to define new requirements in RAN4 RRM to capture the UE behaviour for the collision between DL CSI-RS and dynamically scheduled UL transmission.

- Recommended WF in draft WF

  - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement.

  - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission

    - Option 1: following the Rel-16 NR-U approach

    - Option 2: others

Adhoc Discussion:

  - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission

    - Option 1: following the Rel-16 NR-U approach

    - Option 2: others

HW: we prefer to option 1. This is quite similar as R16 NRU, so reuse the principle as R16 NR-U.

Nokia: we also support option 1.

QC: we also support option 1.

MTK: not concern on OP1. Ask for the clarification. The TDD collision happened. The RAN1 didn’t add new patteren. We just add the requirements.

Agreement:

  - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement.

  - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach.

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Proposals for measurement period for DUD case

  - Proposal 1 (vivo, CATT, Nokia, Samsung, CMCC, HW, ZTE, QC):

    - Existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

      - Provided that the number of consecutive CSI-RS PRBs in at least one of the DL subband is no less than the existing thresholds (QC)

  - Proposal 2 (OPPO, E///):

    - Doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

  - Proposal 3 (CTC, Apple):

    - Define UE capability on whether UE can measures both DL subbands on each CSI-RS resource occasion

      - If UE supports the capability, existing measurement period for CSI-RS based L1 measurement apply

      - If UE does not support the capability, existing measurement period for CSI-RS based L1 measurement is doubled

  - Proposal 4 (MTK):

    - Introduce new requirements where the new requirements include a minimum threshold of 8ms on top of existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

- Recommended WF for measurement period for DUD case

  - Further discuss the following options.

    - Option 1: Existing measurement period for CSI-RS based L1 measurement apply

      - Option 1-1: for Case 1, 2 and 3

      - Option 1-2: for Case 1, 2 only

    - Option 2: Define UE capability on whether UE can measure RSSI in both DL subbands simultaneously.

      - If UE does not support the capability, measurement period is

        - Option 2-1: doubled compared to existing measurement period

        - Option 2-2: based on a lower bound of 8ms

      - If UE supports the capability, existing measurement period apply

- Proposals for measurement accuracy for Case 1 and 2

  - Proposal 1 (vivo, CATT, Nokia, Samsung, CMCC, HW, CTC, ZTE, MTK, QC, E///):

    - Existing measurement accuracy requirements apply.

  - Proposal 2 (E///):

    - 2 sets of requirements can be defined based on 48PRBs and 24PRBs for CSI-RS based L1-RSRP and L1-SINR.

- Recommended WF for measurement accuracy for Case 1 and 2

  - It is suggested to agree on P1 based on majority view.

- Proposals for measurement accuracy for Case 3

  - Proposal 1 (vivo):

    - Only consider case 3 when both X1 and X2 > y, y is a lower bound up to discussion.

  - Proposal 2 (CATT):

    - Use existing accuracy requirements as starting point and introduce a margin to mitigate the impact of discontinuous CSI-RS distribution in frequency domain.

  - Proposal 3 (Nokia, E///):

    - Existing accuracy requirements apply if X1>X4 and X2>X4.

    - In case the new specified conditions are not fulfilled, the existing accuracy requirements can be relaxed to account the negative impact of the measurement on SBFD symbols.

    - 2 sets of requirements can be defined based on 48PRBs (legacy existing requirements)and 24PRBs (relaxed requirements) for CSI-RS based L1-RSRP and L1-SINR.

  - Proposal 4 (Samsung, E///):

    - The accuracy can be specified with the condition X4 = X3/2 for existing L1-RSRP or RLM/BFD/CBD requirements. The accuracy can be changed depends on simulation results difference.

  - Proposal 5 (E///):

    - 2 sets of requirements can be defined based on 48PRBs (X3) and 24PRBs (X4=X3/2) for CSI-RS based L1-RSRP and L1-SINR.

  - Proposal 6 (CMCC):

    - Existing measurement accuracy requirements apply.

  - Proposal 7 (HW):

    - Existing accuracy requirements apply for L1-RSRP/L1-SINR, provided that number CSI-RS RBs in each DL subband is no less than 24,

    - Relaxed accuracy requirements or propagation condition apply for RLM/CBD/BFD, provided that number CSI-RS RBs in each DL subband is no less than 12.

  - Proposal 8 (CTC):

    - existing accuracy requirements could not apply, whether to define relaxed accuracy requirements can be further discussed.

  - Proposal 9 (ZTE):

    - FFS whether existing accuracy requirements could apply

  - Proposal 10 (MTK, QC):

    - RAN4 to deprioritize case 3, because existing accuracy requirements can’t be reused.

- Recommended WF for measurement accuracy for Case 3

  - Views are very diverse, suggest to consider the following options

    - Option 1: existing accuracy requirements apply

      - Option 1-1: without any condition

      - Option 1-2: with the condition that X1>X4 and X2>X4

    - Option 2: relaxed accuracy requirements or relaxed side condition apply

      - Option 2-1: without any condition

      - Option 2-2: with the condition that X1>X4 and X2>X4

    - Option 3: no accuracy requirements apply

    - Different options can be adopted for L1-RSRP/SINR and RLM/BFD/CBD

Recommended in draft WF:

  - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD

  - Existing delay and accuracy requirements apply for Case 1 and Case 2

  - If UE indicates that CSI processing time is scaled by 2,

    - Option1: Add a lower bound of 8ms on top of existing delay requirements.

    - Option2: further discuss how to define delay requirements.

  - Further discuss the following options for Case 3

    - Option 1: No delay or accuracy requirements apply

    - Option 2: Existing delay and accuracy requirements apply provided that X1 > X4 and X2 > X4

    - Option 3: Existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 and X2 > X4

    - Option 4: Delay requirements are doubled and existing accuracy requirements apply provided that X1 > X4 and X2 > X4

    - Option 5: Define a UE capability for simultaneous CSI-RS reception on the two DL subbands for DUD

      - If UE supports the capability, go with option 2

      - If UE does not support the capability, go with option 1, 3 or 4.

Adhoc Discussion:

  - If UE indicates that CSI processing time is scaled by 2,

    - Option1: Add a lower bound of 8ms on top of existing delay requirements.

    - Option2: further discuss how to define delay requirements.

QC: we would like to further check this one. So prefer option 2 to keep it open in this meeting.

MTK: just clarify the option1 from our side. The 2x is coming from the RAN1. The CSI-RS resource time is 4ms. The CSI processing time is added to 2x for UE capability. We are also ok for QC’s suggest for further check by all companies.

Apple: we also prefer to keep it open. Come back in next meeting.

  - Further discuss the following options for Case 3

    - Option 1: No delay or accuracy requirements apply

    - Option 2: Existing delay and accuracy requirements apply provided that X1 > X4 and X2 > X4

    - Option 3: Existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 and X2 > X4

    - Option 4: Delay requirements are doubled and existing accuracy requirements apply provided that X1 > X4 and X2 > X4

    - Option 5: Define a UE capability for simultaneous CSI-RS reception on the two DL subbands for DUD

      - If UE supports the capability, go with option 2

      - If UE does not support the capability, go with option 1, 3 or 4.

Nokia: clarify for the measurement period. Whether it is related to the CSI-RS processing time.

QC: we can support option 1. And if we want to define requirements, can also support op4. Not accept option 2 and 3.

Samsung: we can support opotion3 by defining the req. with the relaxation of accuracy .

HW: we also support option3. If preclude this case, it is really, If strong concern from UE side, we can also support Option 5 depends on the UE capability.

ZTE: for the delay requirements, by our understanding, it is also related to the CSI-RS processing time. We don’t understand why cannot define the unique requirements for Case 1&2&3. We are open to discuss for OP3 for accuracy req.

CATT: we support OP3. From NW perspective, it is not expected to extended from UE side by double the measurement period. Prefer to the existing measurement period, but can relax some accuracy.

QC: question to OP3: what is the relaxation for accuracy. There are just two meetings left for core completion. 
Nokia: prefer OP2. We can compromise to OP3. But for OP4, we cannot accept by doubling the measurement period.

MTK: for the OP4, there is the misunderstanding. The D1 and D2 will receive some interfere of each of them of D1 and D2. That is why we prefer to not define such corner case with the BW is smaller. Option 3 means further simulation is needed. Similar view as QC for how many will be relaxed.

E///: for OP4, there two issues as mentioned from MTK. The CSI-RS resource measurement will be interfered in the middle. The number of RBs in total, will it still be the same as legacy. Question to MTK: If 24PRB for D1, and 24PRB for D2. The more samples cannot solve the issue with interfere even double the measurement period.

ZTE: in legacy wide band,  the CSI-RS can be configured by non-continuous subbands. So in DUD case, it is quite similar as the non-continuous sbubbands (RBs) for the CSI-RS mapping. So that’s why we don’t understand x2 measurement period is needed.

QC: to ZTE: it is different from legacy. The UL band is the middle of D1 and D2. Even PDSCH/PDCCH can be reception simultaneously. But for the measurement, it is not the same as the data reception. For the option 3, we can accept by relax the accuracy by the single subband.

Samsung: different UE implementation can handle the measurement and data processing by different ways.

QC: we prefer not to change UE implementation from ours.

Apple: RAN4 don’t define requirements for all cases. Why we discuss the core requirements to cover case 3.

MTK: case 2 is beyond the 48 PRBs. It can cover many cases. It is not so important for case 3.

E///: to QC: as HW mentioned, from the NW perspective, NW would like to receive the reporting from both of them.  10MHZ is typical. It depends from NW how many RBs of UL will be used. So the narrowed DL subbands are quite useful for the deployment and reasonable case to check the CSI-RS quality. Prefer to define the core requirements for this case.

Agreement:

- Background:

  - Different cases for DUD

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

    - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

- Existing delay [number of samples] requirements and accuracy requirements apply for Case 1 and Case 2

For Case 3:

    - Option 1: No delay or accuracy requirements apply

    - Option 2: Existing delay and accuracy requirements apply provided that X1 > X4 and X2 > X4

    - Option 3a: Existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, the accuracy requirements depends on the X4

    - Option 4: Delay requirements are doubled and existing accuracy requirements apply provided that X1 > X4 and X2 > X4

X1+X2>X3(48 for L1-RSRP and 24 for RLM), X4 is (24 PRBs for single DL subband)

For RLM, for Case 3, no requirement?

- Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Proposals for L1-RSRP and L1-SINR

  - Proposal 1 (vivo, CATT, Nokia, Samsung, CMCC, HW, ZTE, MTK)

    - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network

  - Proposal 2 (Nokia)

    - For measurement period, consider the overlapping between the configured CSI-RS resources and the configured symbol type

    - For measurement accuracy, discuss whether different side conditions should be considered based on the indicated symbol type

  - Proposal 3 (HW)

    - Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach

  - Proposal 4 (MTK)

    - No need to change RRM requirements for L1-RSRP/L1-SINR.

- Proposals for RLM/BFD/CBD

  - Proposal 1 (vivo, CATT, Samsung, HW, ZTE, MTK)

    - For CSI-RS based RLM/BFD/CBD measurements, UE is expected to measure CSI-RS resources on both symbol types.

      - CSI-RS on SBFD symbols is larger than X RBs for DU configuration or Y RBs for DUD configuration for CSI-RS based RLM/BFD/CBD measurement (vivo)

      - The power difference between SBFD symbols and non-SBFD symbols is needed (Samsung)

      - Existing measurement period for CSI-RS based RLM/BFD/CBD apply (HW, MTK)

  - Proposal 2 (Nokia, CMCC)

    - RAN4 to assume the CSI-RS configuration for RLM/BFD is limited to one symbol type, or the configuration is not limited to one symbol type but the network indicates the valid symbol type for CSI-RS based RLM/BFD.

- Recommended WF for L1-RSRP and L1-SINR in draft WF

    - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network

    - For L1-RSRP and L1-SINR measurement period

      - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

      - Option 2: further discuss which approach to use

- Recommended WF for RLM/BFD/CBD

    - Discuss following options for the UE measurement behaviour

      - Option 1: measurement of a single CSI-RS resource is limited to one symbol type

        - Option 1-1: as a side condition of RAN4 requirements

        - Option 1-2: based on NW configuration

      - Option 2: measurement of a single CSI-RS resource can be in both symbol types

    - Discuss whether and how to update RLM/BFD/CBD measurement period

      - Option 1: extend the measurement period to account for the overlapping between the CSI-RS resource occasions and the valid symbol type, e.g. by following NR-U approach

      - Option 2: existing measurement period apply

        - Option 2-1: provided that CSI-RS BW is larger than a lower bound

        - Option 2-2: provided that UE is informed the power difference between SBFD and non-SBFD symbols

Adhoc Discussion:

Agreement:

- Issue 2-1-4: Requirements for RACH requirements

  - Discuss whether and how RACH requirements are to be updated for SBFD operation based on RAN2 agreement.

Adhoc Discussion:

Agreement:

- Issue 2-1-6: Requirements for MG and BWP switch

  - Include transmission in SBFD slots in addition to the UL slots, wherever applicable in the specification, e.g. in MG and BWP switch requirements.

Adhoc Discussion:

Agreement:

- Issue 2-1-7: Requirements for UL resource muting

  - RAN4 to confirm there is no RRM impact due to UL resource muting

Adhoc Discussion:

Agreement: