3GPP TSG-RAN WG4 Meeting #114bis			    					           R4-2505093

# Wuhan, China, April 7-11, 2025

Title: 	WF on simultaneous Rx-Tx issues for UL CA_n5-n8 and others

Agenda Item:	6.1

Source: 	Huawei

Document for:	Approval

Background:

In RAN4#114, we had agreement in [1] for FDD-FDD band combination that

- For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this due to UL CA_n5-n8 is TBD - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling. - Deviations to this above due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

In RAN4#113, we sent LS [2] to ask RAN2 to define UE capability related to non-simultaneous on (n8UL, n5DL) in the FDD-FDD band combination CA_n5-n8. And RAN2 replied the LS with further question on the details about the requested capability. RAN4 need to provide sufficient information to RAN2 for reference, including the conclusions and progress in NR_700800900_combo_enh, LTE_NR_R19_Simult_RxTx and NR_LBCA_Sw, to design the capability.

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

- Option 1: Only support 1UL/2DL CA. Single UL in n5

- Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

2) Dedicated RF filters implementation with partial frequency range

- Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In particular, Option 1 has already been specified in Rel-18, while RAN4 has decided to specify Option 2 in Rel-19. A UE supporting Option 3 enables simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions. However, this option may be specified in Rel-20 or a later release.

Moreover, RAN4 has already specified Option 1 in Rel-18, and intends to standardize Option 2 in Rel-19, which requires a future-proof signaling design to support that the capabilities for the simultaneous operation of nA-UL with nB-DL and nA-DL with nB-UL can be reported independently, as requested in the original LS R4-2420410. If only Option 2 is assumed, scheduling restriction at network can work however it is not a future-proof solution.

In addition, CA_n5-n8 may not be the only FDD-FDD combo with non-simultaneous Rx-Tx issue. For band combination without sufficient separation between DL carrier in one band and UL carrier in the other band, simultaneous DL-UL operation is difficult. In [3], two operators in North America identify two sub-1GHz to be aggregated, CA_n26-n106 with 2MHz isolation between n26 DL and n106 UL.

Figure 1 Frequency range illustration for CA_n26-n106

For Rel-19 low band carrier aggregation via switching [4], there is no simultaneous Rx-Tx between FDD UL and SDL. From the duplexer design perspective, the frequency separation between the bands does not pose a feasibility challenge in general. Things are quite similar to that for FDD-FDD without sufficient isolation between nA UL and nB DL in CA_n12-n29.

Figure 3 Frequency range illustration for CA_n12-n29

<Way forward>

For FDD-FDD and FDD-SDL inter-band CA band combinations with partial overlapping frequency or without sufficient separation between DL in one band and UL in the other band, such as, CA_n5-n8 (n8 UL and n5 DL) and CA via low band switching(n12UL and n29DL, n26DL and n106 UL), etc., that have problems in supporting simultaneous Rx-Tx in UL of band nA and DL of band nB, a signalling-related solution should be considered.

Reference

[1] R4-2502840, WF on simultaneous Rx-Tx, Huawei, RAN#114, Athens, February, 2025

[2] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, RAN4#113, Orlando, November, 2024

[3] R4-2411176, Motivation for a new work item on LB+LB CA based on switching in Rel-19, Apple, , Skyworks Solutions Inc., BT plc, TELUS, Bell Mobility, Anterix, Southern Linc, RAN4#112.

[4] RP-243317, New WID on low band carrier aggregation via switching, RAN#106